| TSG SA WG3 #6, Sophia Antipolis, 29th September - 1st October S3-99332 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | TS 33.102 |  |  |  |  |  | CR |  |  |  | 15 |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | V3.1.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  | SA#6 |  |  | for approval |  |  |  |  |  | x |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  | Be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  | x | ME |  |  |  |  |  |  |  | x | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | S3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 1-10-99 |  |  |  |  |
| Subject: |  |  | Secure UMTS-GSM Interoperation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | Security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | The proposed mechanism allows secure UMTS-GSM interoperability by using standardised conversion functions for generating the authentication vectors needed in each network. The proposed conversion functions are designed with the aim to define a simple, efficient and secure system to facilitate secure interoperation for all envisaged GSM/UMTS roaming and handover scenarios, based on the following principles: - GSM security parameters are generated from the UMTS security parameters, when needed. - GSM security parameters depend on the UMTS security algorithms, under the control of the HPLMN. - UMTS security parameters are generated from the GSM security parameters, when needed. - Traffic load (sometimes international signalling) is optimised, just the needed security parameters (GSM or UMTS) are transmitted. - Conversion functions are very simple and efficient. No additional either computational load or memory is introduced. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | Sections 6.3.7, 6.4.8.1 , 6.6.9.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.3.7	Interoperability with 2G networks

Note:	The description below is mainly based on CS domain procedures. It will be extended to also completely describe PS procedures.

#### 6.3.7.1	Interoperability for UMTS users

A general principle in designing the security interoperation between 3G and 2G networks has been that a UMTS user (i.e. a user with a USIM issued by a R99 HLR/AuC) shall get UMTS level of security whenever possible.

The mechanism described here achieves intersystem operability between UMTS and GSM networks allowing secure interoperation between both networks for UMTS users (USIM). The following figure illustrates the different scenarios of interoperability for UMTS users:

UMTS

HLR/AuC

UMTS

MSC/VLR

GSM

MSC/VLR

Non-UMTS capable

UMTS Access Network

UMTS AV

(RAND, XRES, CK, IK, AUTN)

GSM Data

(RAND,  Kc)

UMTS Data

(RAND, CK, IK, AUTN)

GSM Data

(RAND,  Kc)

GSM AV

(RAND, SRES, Kc)

GSM

MSC/VLR

UMTS capable

GSM Data

(RAND,  Kc)

UMTS AV

(RAND, XRES, CK, IK, AUTN)

GSM

Access Network

GSM

Access Network

GSM

Access Network

Or UMTS Data

(RAND, CK, IK, AUTN)

Or UMTS Data

(RAND, CK, IK, AUTN)

Figure 13: Interoperability for UMTS Users

The UMTS authentication parameters are generated by the UMTS HLR/AuC and USIM by use of the home operator specified algorithms for this purpose.

Upon receipt of an authentication data request from a UMTS SN/VLR or a UMTS capable GSM SN/VLR , the HLR/AuC sends an ordered array of n UMTS authentication vectors (quintuples) to the SN/VLR.

If the UMTS MSC/VLR is able to handle GSM radio access network, the MSC/VLR shall be able to derive a GSM authentication vector from the received UMTS vector, by means of the standardised conversion functions defined below, in order to provide the GSM security parameters to the GSM radio access network. Whether GSM Data or UMTS Data is used depends on the terminal capabilities.

Upon receipt of an authentication data request from a non-UMTS capable GSM SN/VLR, the HLR/AuC shall derive the GSM authentication vectors from the UMTS vectors, by means of the standardised conversion functions defined below. Then, the HLR/AuC sends an authentication response back to the SN/VLR that contains an ordered array of n GSM authentication vectors (triples). The HLR/AuC may have pre-computed GSM authentication vectors or may derive them on demand from the UMTS authentication vectors.

On the mobile side, the USIM shall derive the GSM authentication parameters from the UMTS authentication parameters by means of the standardised conversion functions, when the MS is located in the GSM radio access network.

The previous procedures are also applicable to the corresponding PS network and so as to the corresponding SGSN entity.

Subsequently the following entities shall implement the standardised conversion functions for generating GSM authentication vectors (triplets) from UMTS authentication vectors (quintuplets):

- UMTS HLR/AuC

- UMTS MSC/VLR

- UMTS SGSN

- UMTS capable GSM MSC/VLR

- UMTS capable GSM SGSN

- USIM

Interoperability with non-UMTS capable GSM entities is achieved by use of the standardised conversion functions implemented in the HLR/AuC. The handover case is described in sections 6.4.8.1 and  6.6.9.4.

The following conversion functions shall be computed for generating the GSM authentication parameters:

- Generation of GSM RAND

f: (RANDU) -> RANDG;  		RANDG = RANDU

- Generation of GSM SRES

f: (XRES) -> SRES;  			SRES =XRES1  XRES2  XRES3  XRES4

whereby XRES = XRES1 || XRES2 || XRES3 || XRES4; and with XRESn 32 bits each. If any of XRESn is not used, it is assumed zeros.

- Generation of GSM Kc

f: (CK, IK) -> Kc;  				Kc = CK1  CK2  IK1  IK2

whereby CK1 (resp. IK1) is the first half and CK2 (resp. IK2) is the second half of CK (resp. IK)

The GSM authentication vector is generated using the UMTS authentication parameters. Consequently, the generated triplet depends on the UMTS authentication algorithms and inputs parameters for these algorithms, all this information under the control of the HE, being the algorithms operator specific.

The GSM authentication and key generating algorithms are specified as follows:

A3: RANDG -> SRES;			SRES = f(f2K(RANDU))

A8: RANDG -> Kc;				Kc = f(f3K(RANDU), f4K(RANDU))

#### 6.3.7.2	Interoperability for GSM users

The mechanism described here achieves intersystem operability between UMTS and GSM networks allowing secure interoperation between both networks for GSM users (SIM). The following figure illustrates the different scenarios of interoperability for 2G users:

GSM

HLR/AuC

UMTS

MSC/VLR

GSM

MSC/VLR

Non-UMTS capable

UMTS Access Network

GSM AV

(RAND, SRES, Kc)

GSM Data

(RAND,  Kc)

UMTS Data

(RAND, CK, IK)

GSM Data

(RAND,  Kc)

GSM AV

(RAND, SRES, Kc)

GSM

MSC/VLR

UMTS capable

GSM Data

(RAND,  Kc)

GSM AV

(RAND, SRES, Kc)

GSM

Access Network

GSM

Access Network

GSM

Access Network

Figure 14: Interoperability for GSM Users

The GSM authentication parameters are generated by the GSM HLR/AuC and the SIM by use of the home operator specified algorithms for this purpose.

Upon receipt of an authentication data request from any SN/VLR (UMTS or GSM), the HLR/AuC sends an ordered array of n GSM authentication vectors (triplets) to the SN/VLR.

If the UMTS MSC/VLR is able to handle UMTS radio access network, the MSC/VLR shall be able to derive a UMTS authentication vector from the received GSM authentication vector, by means of the standardised conversion functions defined below, in order to provide the UMTS security parameters to the UMTS radio access network.

On the mobile side, the UE shall derive the UMTS authentication parameters from the GSM authentication parameters generated by the SIM by means of the standardised conversion functions, when the MS is located in the UMTS radio access network.

The previous procedures are also applicable to the corresponding PS network and so as to the corresponding SGSN entity.

Subsequently the following entities shall implement the standardised conversion functions for generating UMTS authentication parameters from GSM authentication vectors (triplets):

- UMTS MSC/VLR

- UMTS SGSN

- UE

The following conversion functions shall be computed for generating the UMTS authentication parameters:

- Generation of UMTS RAND

f: (RANDG) -> RANDU;  				RANDU = RANDG

- Generation of UMTS XRES

f: (SRES) -> XRES;  					XRES = SRES

- Generation of UMTS CK

f: (Kc) -> CK;  							CK = 0..0 || Kc

- Generation of UMTS IK

f: (Kc) -> IK;  							IK = Kc || Kc

- Generation of UMTS AUTN

The authentication token AUTN is not used for GSM users.

The UMTS authentication vector is generated using the GSM authentication parameters. Consequently, the generated quintuplet depends on the GSM authentication algorithms and inputs parameters for these algorithms, all this information under the control of the HE, being the algorithms operator specific.

A GSM user should receive the security level provided by the home network. The effective encryption key length used in UMTS radio access network is the GSM Kc length, provided by the HE; Integrity protection is provided by using the GSM encryption key; and the AUTN parameter is not used.

The UMTS authentication and key generating algorithms are specified as follows:

f2: RANDu -> XRES;						XRES =A3(RANDG)

f3: RANDu -> CK;							CK = 0..0 || A8(RANDG)

f4: RANDu -> IK;								IK = A8(RANDG) || A8(RANDG)

f: (RANDu, SEQ, AK, MAC) -> AUTN;	Parameter not used

6.4.8	Integrity protection procedures

Integrity protection is performed by appending a message authentication code (MAC-I) to the message that is to be integrity protected. The MS can append the MAC-I to signalling messages as soon as it has received a connection specific FRESH value from the RNC.

If the value of HFNMS is larger or equal to the maximum value stored in the USIM, the MS indicates to the network in the RRC connection set-up that it is required to initialize a new authentication and key agreement.

Note:	The precise set-up of data integrity is for further study.

#### 6.4.8.1	Handover

Note:	It is expected that in case of inter-operator handover a handover agreement exists that covers all charging aspects. Agreements on cipher key transportation/re-use shall also be included.

1)	Intra-system:

When a handover occurs, the IK is transmitted within the system infrastructure from the old RNC to the new one to enable the communication to proceed, and the synchronisation procedure is resumed. The key IK remains unchanged at handover.

2)	Inter-system:

UMTS  GSM

The GSM network entered by the user handing over from UMTS does not provide integrity protection and the integrity key (IK) is not needed in the GSM network.

GSM  UMTS

The UMTS network entered by the user handing over from GSM will enable integrity protection. This will involve setting the integrity protection key (IK). There are two options:

- Handover from a UMTS capable GSM network

When handover occurs, the IK is transmitted from the old MSC/VLR to the new one to enable the communication to proceed.

- b)   Handover from a non-UMTS capable network. The method below is not recommended but presened if it will be a service requirement. There are doubts whether there is a requirement for this kind of handover and whether it is possible on, for instance, the MAP/E interface.

The UMTS integrity key shall be derived by the UMTS network an UE by using the conversion functions specified in section 6.3.7.2 for this purpose.

When handover occurs, the Kc is transmitted form the old MSC/VLR to the new one to enable the communication to proceed.

### 6.6.9	Ciphering procedures

#### 6.6.9.1	Starting of the ciphering and deciphering processes

The MS and the RNC must co-ordinate the instants at which the enciphering and deciphering processes start.

This procedure takes place under the control of the network some time after the completion of the authentication procedure (if any), or after the key CK has been made available at the RNC.

No information elements for which protection is needed must be sent before the ciphering and deciphering processes are operating.

The transition from clear text mode to ciphered mode proceeds as follows: deciphering starts in the RNC, which sends in clear text to the MS a specific message, here called "Start cipher". Both the enciphering and deciphering start on the MS side after the message "Start cipher" has been correctly received by the MS. Finally, enciphering on the RNC side starts as soon as a frame or a message from the MS has been correctly deciphered at the RNC.

[diagram to be added]

#### 6.6.9.2	Synchronisation

The enciphering stream at one end and the deciphering stream at the other end must be synchronised, for the enciphering bit stream and the deciphering bit streams to coincide.

Synchronisation is guarantied by driving UEA by an explicit time variable, COUNT, derived from an appropriate frame number available at the MS and at the RNC.

The diagram below summarises the implementation indications listed above, with only one enciphering/deciphering procedure represented (the second one for deciphering/enciphering is symmetrical).

[diagram to be added]

#### 6.6.9.3	Layer for ciphering

The layer on which ciphering takes place depends on the Layer 2 mode of the data. Data transmitted on logical channels using a non-transparent RLC mode (either Acknowledged Mode or Unacknowledged Mode) is ciphered in the RLC sub-layer of Layer 2. Data transmitted on a logical channel using the transparent RLC mode is ciphered at the MAC sub-layer of Layer 2.

#### 6.6.9.4	Handover

Note:	It is expected that in case of inter-operator handover a handover agreement exists that covers all charging aspects. Agreements on cipher key transportation/re-use shall also be included.

1)	Intra-system

When a handover occurs, the CK is transmitted within the system infrastructure from the old RNC to the new one to enable the communication to proceed, and the synchronisation procedure is resumed. The key CK remains unchanged at handover.

2)	Inter-system

UMTS  GSM

The GSM network entered by the user handing over will enable ciphering protection. This involves setting the GSM cipher key (Kc).

The GSM cipher key shall be derived by the UMTS network by using the conversion functions specified in section 6.3.7.1 for this purpose.

When handover occurs, the generated Kc is transmitted from the old MSC/VLR to the new one to enable the communication to proceed.

GSM  UMTS

The UMTS network entered by the user handing over from GSM will enable confidentiality protection. This will involve setting the cipher  key (CK). There are two options:

- Handover from a UMTS capable GSM network

When handover occurs, the CK is transmitted from the old MSC/VLR to the new one to enable the communication to proceed.

- Handover  from a non-UMTS capable network. The method below is not recommended but presented if it will be a service requirement. There are doubts whether there is a requirement for this kind of handover and whether it is possible on, for instance, the MAP/E interface

The UMTS cipher key shall be derived by the UMTS network an UE by using the conversion functions specified in section 6.3.7.2 for this purpose.

When handover occurs, the derived Kc is transmitted form the old MSC/VLR to the new one to enable the communication to proceed.