Tdoc S3-99454

TSG-SA WG3 (Security) meeting #8

Sophia Antipolis, 16-19 November, 1999

Source:	ETSI SMG10 / 3GPP TSG-SA WG3

To:	TIA TR-45 / AHAG

Title:	Principles concerning the development of a common authentication mechanism to support global roaming

To aid global roaming across all 3G systems, S3 would like to propose that the following principles are adopted by TR-45 / AHAG:

- All 3G family member systems support the 3GPP AKA mechanism as a minimum.

- If other features not provided by the 3GPP AKA mechanism are required by particular 3G family systems then additional mechanisms must be developed for, and supported by, that particular system providing that the ability to support global roaming is not impaired.

These principles would allow features required by particular family member systems to be made available to users of that system. Users of other family member systems may not be able to support such features when roaming to the affected system. However, based on the likely roaming patterns between different 3G family member systems, it is argued that the inability to support these features for some roaming users will have negligible impact on the overall system performance.

One example feature which would fall into this category would be support for authentication based on a global challenge in 3G ANSI-41 systems. In this case it is proposed that such a feature is not provided when users from other 3G systems roam to 3G ANSI-41 networks.