3GPP TSG SA WG3 Meeting #9 					     S3-99510

Helsinki, Finland, 7th-9th December

Source:	  BOSCH Telecom

Title:  “3GPP terminal identity security: levels, requirements and mechanisms"

Agenda Item:       Terminal security 8.3

Document for:	Discussion /decision

___________________________________________________________________________

## This is a living document to discuss the presents and future security levels of terminal identity (IMEI). The requirements and services are defined for each security level in consideration. Possible mechanisms are given to be proposed to evaluate the impact of secure identity on  3GPP Security Architecture.

## The first version of this document is prepared based on Doc s3 99-401 based on [1] to [10] which includes a digest to the documents presented so far in s3 group. Doc s3 99-439 is also involved, it includes a proposal for essential requirements for secured terminal identity for 3GPP terminals.

Two security levels are identified, protected IMEI without provability and protected IMEI with provability.

- Protected IMEI without provability (current GSM status)

  - Requirements:

The requirements are defined after being improved by change request [4] (CR to GSM 02.09 V4.4.0,  Doc AP99-093). The wording is as follows:

- It shall not be possible to change the IMEI after the ME’s final production process.

It   shall resist tampering by any means (e.g. physical, electrical or software)

- The security policy for the Software Version Number (SVN) is such that it cannot be

readily changed by the user, but can be updated with changes to the software.

The security of the SVN shall be separate from that of the IMEI.

Fig.1 shows a basic scenario to realise these requirements.

Fig. 1  Current Terminal Identity concept in GSM

1.2  Security level

IMEI is sent in clear (open identity). The following characterise the security of that terminal identity

-     Relatively simple to implement

- No proof of origin or type approval is possible

- Cloning is basically possible. As sending an arbitrary IMEI is always possible from other terminal or other software source.

- A certain terminal can not be securely identified if the software is not provable.

- If the software is secured then IMEI is secure (software dependability). No secured software execution is known without cryptographic means.

- Network failed in GSM to control illegal terminals whatever the reason is for such control.

It is noted that the software security can not be separated from IMEI security and vice versa. The reasons are:

- The software is the last source of  IMEI and is the entity which defines IMEI. The stored IMEI is not necessary the same as the sent one

- If the software can be changed, then any  forged IMEI could be sent. Many attack scenarios are possible

- If the software is made provable, then IMEI could be seen as somehow secure. However a substitution attack is always possible (one scenario is to substitute the software with other copied piece of software which satisfy the software security rules).

- A basic security rule is violated in this system. “ unidentified  entity is a part of the system”, this eases substitution attacks. The same holds for the base station.

The above requirement  No. 2 appear to be contradictory !!

1.3  Usage/relevant service:

- Deter using stolen terminals

- Blacklisting type non-approved terminals

- Identify emergency call terminal

- SIM lock

1.4  Impact on security architecture:

No impact. The methodology to protect IMEI by manufacturers is up to the manufacturer, as no common proof mechanism is necessary.

1.5 Advantages:

- Relatively simple to realize

- No standardization is necessary

- Protected and provable IMEI

A  cryptographically secured  IMEI is proposed for 3GPP to cope with future 3GPP requirements and applications

2.1 Requirements proposal (source Bosch Telecom):

- An open IMEI and its corresponding secret part SIMEI is to be stored in a non-volatile memory in a tamperproof physical entity which is hard to remove from the terminal

- A common proof algorithm is to be defined to cryptographically prove that the SIMEI corresponding to IMEI resides in the terminal.

- IMEI should not be easy to modify, but if modified the proof algorithm should fail (see also 7)

- The manufacturer should electronically sign IMEI before leaving the factory. No body other than the manufacturer should be able to generate equivalent signature.

- Any operator and other legitimate party  should be able to offer a time varying challenge to the terminal to prove the manufacturer signature. The manufacturer publishes some open or secret means to enable the proof of its signature (prove terminal origin and thus the originally manufactured terminal quality and capability).

- The terminal should include multiple provable mobile identities  MI’s as IMEI which could be separately defined by other identity owners as operator, user, regulator, authority and others (64 MI’s  with 64 Bits and corresponding SMI’s with 128 Bits each appear to be adequate for future needs). The same scenario 1 to 5 should be possible.

- The signature SIG should also be possible to be generated  from many predefined SMI’s and many  predefined challenges CH’s.

- It should be possible to modify MI and SMI by the owner of SMI by presenting SMI again to the tamperproof device. This is to enable the reuse of identities by legal identity owners.

2.2  Security level

IMEI is sent in clear together with its (time variant challenged) proof of signature . The following characterise the security of that terminal identity

- Proof of origin or type approval is possible

- Cloning is not possible. As imitating the right signature needs the secret SIMEI.

- A certain terminal can be securely identified either if the software is not secure. No dependency between software and identity.

- Network can identify and control illegal terminals to carry out any relevant security action without destroying other services.

2.3  Usage/relevant service:

- Deter using stolen terminals

- Blacklisting type non-approved terminals

- Identify emergency call terminal

- Restricting some service to some class of terminals with special quality

- Effective implementation of the MExE  and Mobile IP security requirements

2.4  Impact on security architecture:

As standardised architectures are necessary, the following possible impact on security architecture is identified.

Architecture/possible mechanisms (source Bosch Telecom):

A common methodology to proof IMEI via common proof-mechanism is necessary.

If a provable identity is to be integrated and standardized in the 3GPP terminals, the following two technical requirements appear to be essential:

Hardware Requirements

To be able to cryptologically identify a unique terminal, the following two physical entities are essential: (Fig. 2 shows the overall concept)

- A tamperproof write-only secret nonvolatile identity to store SMI (1..64) with 128 Bits each  (whatever tamperproof  means in the current state of the art technology). The corresponding MI (1..64) with 64 bits each is also stored not necessarily protected in a write once /read non-volatile memory

- Define a commonly known Signature Function SF. This could be achieved by using:

Either  (SF1): a one way function ( say f8 or f9 or as in [6]) which maps this  secret  identity SMI or many identities SMI1, SMI2....and a challenge CH or many challenges CH1, CH2....into a signature response SIG to prove the existence of the secret identities in the terminal

SIG =   SF1 ( /SMI1,SMI2../ , /CH1, CH2.../)        (symmetric function)

Or (SF2) : some public-key one way function SF2 (say squaring function as Rabin-Lock) [7] which proves the existence of the write-only secret identities in the terminal.

SIG = SF2 (/SMI1, SMI2..../,  /CH1, CH2 .../)		(asymmetric function)

There should be no hardware possibility (trap door) to read  any SMI.

The tamperproof device should physically tied into some terminal core function.

Fig. 2   Terminal identity security concept for 3GPP

System  Requirements

These requirements are highly application and protocol dependent (tbd).

- One or many system protocols which proves the identity for relevant applications

- A central data base for some applications. This should be avoided as  far as possible

2.5  Advantages:

- Relatively simple to realize !

- Cloning is not possible

- Secured proof of origin and terminal type are possible

- Identification security does not depend on software security

- Illegal terminals could be identified, counteractions do not disturb other system functions

- New application horizons in 3GPP

References:

# [1]	Charles Brookson, DTI,UK, Terminal Security: Requirements for a Secure Identity

# A draft discussion document, included in Doc. s3 99-401.

[2]	Security mechanisms for the IMEI, Ericsson, s3-99 296

[3]	Provable Terminal Identity , Status and future applications, Bosch . Doc s3-99250.

[4]	CR to GSM 02.09 V4.4.0,  Doc AP99-093

[5]	Change Request to GSM 02.09, GSM 02.16, GSM 03.03 and GSM 11.10 to ensure IMEI security . Doc s3-99214.

[6]	Secret-Key Mechanisms for Secured Terminal Identification , Bosch, Doc s3-99127.

[7]	Provable Terminal Identity with a Public-Key Mechanism, Bosch, Doc s3 -99168

[8]	The use of Zero-Knowledge Identification mechanisms for 3G Terminal

Identification  , Vodafone, Doc s3-99303.

[9]	Repor 3GPP TSG-SA WG3 (Security) t to SA Meeting # 4,Miami, 21-23 June 1999 Michael Walker Chairman 3GPP TSG-SA WG3 Doc s3-99293

[10]     Proposal for IMEI Security, Mannesmann, T-Mobil, Doc s3-99106.

[11]	Mobile Station Application Excution Environment (MExE) 3G TS 23.057 V1.3.0