TSG SA WG3 #9, Helsinki, 7-9 Dec, 1999-12-09							S3-99536

| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TS 33.102 |  |  |  |  |  | CR |  |  |  | 40 |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | V3.2.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  | SA#6 |  |  | for approval |  |  |  |  |  | x |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  | ME |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | S3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 09-12-99 |  |  |  |  |
| Subject: |  |  | An authentication failure report mechanism from SN to HE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | Security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | This CR proposes to add a mechanism for reporting authentications failures from the SN to the HE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | Section 6.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

## 6.3	Authentication and key agreement

### 6.3.1	General

The mechanism described here achieves mutual authentication by the user and the network showing knowledge of a secret key K which is shared between and available only to the USIM and the AuC in the user’s HE. In addition the USIM and the HE keep track of counters SQNMS and SQNHE respectively to support network authentication.

The method was chosen in such a way as to achieve maximum compatibility with the current GSM security architecture and facilitate migration from GSM to UMTS. The method is composed of a challenge/response protocol identical to the GSM subscriber authentication and key establishment protocol combined with a sequence number-based one-pass protocol for network authentication derived from the ISO standard ISO/IEC 9798-4 (section 5.1.1).

An overview of the mechanism is shown in figure 4.

Figure 4: Authentication and key agreement

Upon receipt of a request from the SN/VLR, the HE/AuC sends an ordered array of n authentication vectors (the equivalent of a GSM "triplet") to the SN/VLR. Each authentication vector consists of the following components: a random number RAND, an expected response XRES, a cipher key CK, an integrity key IK and an authentication token AUTN. Each authentication vector is good for one authentication and key agreement between the SN/VLR and the USIM.

When the SN/VLR initiates an authentication and key agreement, it selects the next authentication vector from the array and sends the parameters RAND and AUTN to the user. The USIM checks whether AUTN can be accepted and, if so, produces a response RES which is sent back to the SN/VLR. The USIM also computes CK and IK. The SN/VLR compares the received RES with XRES. If they match the SN/VLR considers the authentication and key agreement exchange to be successfully completed. The established keys CK and IK will then be transferred by the USIM and the SN/VLR to the entities which perform ciphering and integrity functions.

SN/VLRs can offer secure service even when HE/AuC links are unavailable by allowing them to use previously derived cipher and integrity keys for a user so that a secure connection can still be set up without the need for an authentication and key agreement. Authentication is in that case based on a shared integrity key, by means of data integrity protection of signalling messages (see 6.4).

The authenticating parties shall be the AuC of the user’s HE (HE/AuC) and the USIM in the user’s mobile station. The mechanism consists of the following procedures:

A procedure to distribute authentication information from the HE/AuC to the SN/VLR. This procedure is described in 6.3.2. The SN/VLR is assumed to be trusted by the user’s HE to handle authentication information securely. It is also assumed that the intra-system links between the SN/VLR to the HE/AuC are adequately secure. Mechanisms to secure these links are described in clause 7. It is further assumed that the user trusts the HE.

A procedure to mutually authenticate and establish new cipher and integrity keys between the SN/VLR and the MS. This procedure is described in 6.3.3.

A procedure to distribute authentication data from a previously visited VLR to the newly visited VLR. This procedure is described in 6.3.4. It is also assumed that the links between SN/VLRs are adequately secure. Mechanisms to secure these links are described in clause 7.

### 6.3.2	Distribution of authentication data from HE to SN

The purpose of this procedure is to provide the SN/VLR with an array of fresh authentication vectors from the user’s HE to perform a number of user authentications.

Figure 5: Distribution of authentication data from HE to SN/VLR

The SN/VLR invokes the procedures by requesting authentication vectors to the HE/AuC.

The authentication data request shall include a user identity. If the user is known in the SN/VLR by means of the IMUI, the authentication data request shall include the IMUI. However, if the user is identified by means of an encrypted permanent identity (see 6.2), the HLR-message from which the HE can derive the IMUI is included instead. In that case, this procedure and the procedure user identity request to the HLR are integrated.

Upon the receipt of the authentication data request from the SN/VLR, the HE may have pre-computed the required number of authentication vectors and retrieve them from the HLR database or may compute them on demand. The HE/AuC sends an authentication response back to the SN/VLR that contains an ordered array of n authentication vectors AV(1..n).

Figure 6 shows the generation of an authentication vector AV by the HE/AuC.

Figure 6: Generation of an authentication vector

The HE/AuC starts with generating a fresh sequence number SQN and an unpredictable challenge RAND.

For each user the HE/AuC keeps track of a counter: SQNHE

To generate a fresh sequence number, the counter is incremented and subsequently the SQN is set to the new counter value.

Note 1:	The HE has some flexibility in the management of sequence numbers. Annex C and Annex F.3 contain alternative methods for the generation and verification of sequence numbers.

An authentication and key management field AMF is included in the authentication token of each authentication vector. Example uses of this field are included in Annex F.

Subsequently the following values are computed:

-	a message authentication code MAC = f1K(SQN || RAND || AMF) where f1 is a message authentication function;

-	an expected response XRES = f2K (RAND) where f2 is a (possibly truncated) message authentication function;

-	a cipher key CK = f3K (RAND) where f3 is a key generating function;

-	an integrity key IK = f4K (RAND) where f4 is a key generating function;

-	an anonymity key AK = f5K (RAND) where f5 is a key generating function.

Finally the authentication token AUTN = SQN Å AK || AMF || MAC is constructed.

Here, AK is an anonymity key used to conceal the sequence number as the latter may expose the identity and location of the user. The concealment of the sequence number is to protect against passive attacks only.

Note 1:	The need for f5 to use a long-term key different from K is ffs.

Note 2:	The requirements on f3, f4 and f5 are ffs.

Note 3:	It is also ffs in how far the functions f1, ..., f5 need to differ and how they may be suitably combined.

### 6.3.3	Authentication and key agreement

The purpose of this procedure is to authenticate the user and establish a new pair of cipher and integrity keys between the SN/VLR and the MS. During the authentication, the user verifies the freshness of the authentication vector that is used.

Figure 7: Authentication and key establishment

The SN/VLR invokes the procedure by selecting the next unused authentication vector from the ordered array of authentication vectors in the VLR database. The SN/VLR sends to the user the random challenge RAND and an authentication token for network authentication AUTN from the selected authentication vector.

Upon receipt the user proceeds as shown in Figure 8.

Figure 8: User authentication function in the USIM

Upon receipt of RAND and AUTN the user first computes the anonymity key AK = f5K (RAND) and retrieves the sequence number SQN = (SQN Å AK) Å AK.

Next the user computes XMAC = f1K (SQN || RAND || AMF) and compares this with MAC which is included in AUTN. If they are different, the user sends user authentication reject back to the SN/VLR with an indication of the cause and the user abandons the procedure.

Next the user verifies that the received sequence number SQN is in the correct range.

The USIM keeps track of a counter: SQNMS.

To verify that the sequence number SQN is in the correct range, the USIM compares SQN with SQNMS. If SQN > SQNMS the MS considers the sequence number to be in the correct range and subsequently sets SQNMS to SQN.

Note:	The MS and the HE have some flexibility in the management of sequence numbers. Annex C and Annex F.3 contain alternative methods for the generation and verification of sequence numbers.

If the user considers the sequence number to be not in the correct range, he sends synchronisation failure back to the SN/VLR including an appropriate parameter, and abandons the procedure.

The synchronisation failure message contains the parameter RANDMS || AUTS. 
Here RANDMS is the random value stored on the MS which was received in user authentication request causing the last update of SQNMS . 
It is AUTS = Conc(SQNMS ) || MACS.
Conc(SQNMS) = SQNMS  f5K(RANDMS) is the concealed value of the counter SQNMS in the MS, and. 
MACS = f1*K(SQNMS || RAND || AMF) where RAND is the random value received in the current user authentication request. 
f1* is a message authentication code (MAC) function with the property that no valuable information can be inferred from the function values of f1* about those of f1, ... , f5 and vice versa.

The AMF used to calculate MACS assumes a dummy value of all zeros so that it does not need to be transmitted in the clear in the re-synch message.

The construction of the parameter AUTS in shown in the following Figure 9:

Figure 9: Construction of the parameter AUTS

If the sequence number is considered to be in the correct range however, the user computes RES = f2K (RAND) and includes this parameter in a user authentication response back to the SN/VLR. Finally the user computes the cipher key CK = f3K (RAND) and the integrity key IK = f4K (RAND). Note that if this is more efficient, RES, CK and IK could also be computed earlier at any time after receiving RAND. The MS stores RAND for re-synchronisation purposes.

Upon receipt of user authentication response the SN/VLR compares RES with the expected response XRES from the selected authentication vector. If XRES equals RES then the authentication of the user has passed. The SN/VLR also selects the appropriate cipher key CK and integrity key IK from the selected authentication vector.

Conditions on the use of authentication information by the SN/VLR: Using the procedures described in subsections 6.3.1, 6.3.2 and 6.3.4, authentication vectors will have to be used in the specific order in which they were generated, otherwise the user will reject the authentication attempt. The SN/VLR shall use an authentication vector only once and, hence, shall send out each user authentication request RAND || AUTN only once no matter whether the authentication attempt was successful or not. A consequence is that authentication vectors cannot be reused. When a user changes from one VLR to another one and the new VLR requests remaining authentication vectors from the old VLR (cf. subsection 6.3.4) then the old VLR shall not retain any copies of these authentication vectors. When a VLR receives a “cancel location” request for a certain user it shall delete all authentication vectors relating to that user. When a VLR receives a location update request from a user and the VLR notices that authentication vectors relating to that user are still stored in the VLR it will delete this information and request fresh authentication vectors from the HE/AuC.

Different rules may apply when one of the alternative schemes for sequence number handling described in Annex C or Annex F.3 are applied. This is true in particular when the schemes based on windows or lists described in Annexes C.3 and C.4 are applied.

## 6.3.3.1	Cipher key selection

Because of the separate mobility management for CS and PS services, the USIM establishes cipher keys with both the CS and the PS core network service domains. The conditions on the use of these cipher keys in the user and control planes are given below.

##### 6.3.3.1.1	User plane

The CS user data connections are ciphered with the cipher key CKCS established between the user and the 3G CS core network service domain and identified in the security mode setting procedure. The PS user data connections are ciphered with the cipher key CKPS established between the user and the 3G PS core network service domain and identified in the security mode setting procedure.

##### 6.3.3.1.2	Control plane

When a security mode setting procedure is performed, the cipher/integrity key set by this procedure is applied to the signalling plane, what ever core network service domain is specified in the procedure. This may require that the cipher/integrity key of an (already ciphered/integrity protected) ongoing signalling connection is changed. This change should be completed within five seconds.

### 6.3.4	Distribution of authentication vectors between VLRs

The purpose of this procedure is to provide a newly visited VLR with unused authentication vectors from a previously visited VLR.

The procedure is shown in Figure 10.

Figure 10: Distribution of authentication data between SN/VLR

The procedure is invoked by the newly visited SN/VLRn after a location update request sent by the user. Typically the user identifies himself using a temporary user identity TMUIo and the location area identity LAIo of a location area under the jurisdiction of SN/VLRo. In that case this procedure is integrated with the procedure described in 6.1.4.

Upon receipt of the request the VLRo verifies whether it has any unused authentication vectors of the appropriate mode in its database and if so, sends the unused authentication vectors to VLRn. The previously visited VLRo shall then delete these authentication vectors from its database.

Upon receipt the VLRn stores the received authentication vectors.

If VLRo indicates that it has no authentication vectors or the VLRo cannot be contacted, VLRn should request new authentication vectors from the user’s HE using the procedure described in 6.3.2.

### Re-synchronisation procedure

An SN/VLR may send two types of authentication data requests to the HE/AuC, the (regular) one described in subsection 6.3.2 and one used in case of synchronisation failures, described in this subsection.

Upon receiving a synchronisation failure message from the user, the SN/VLR sends an authentication data request with a “synchronisation failure indication” to the HE/AuC, together with the parameters

-	RAND sent to the MS in the preceding user authentication request and

-	RANDMS || AUTS received by the SN/VLR in the response to that request, as described in subsection 6.3.3.

An SN/VLR will not react to unsolicited “ synchronisation failure indication” messages from the MS.

The SN/VLR does not send new user authentication requests to the user before having received the response to its authentication data request from the HE/AuC (or before it is timed out).

When the HE/AuC receives an authentication data request with a “synchronisation failure indication” it acts as follows: The HE/AuC verifies AUTS by computing f5K(RANDMS), retrieving SQNMS from Conc(SQNMS) and verifying MACS (cf. subsection 6.3.3.). If the verification is successful, but SQNMS is such that SQNHE is not in the correct range then the HE/AuC resets the value of the counter SQNHE to SQNMS.
Otherwise, the HE/AuC leaves SQNHE unchanged.

In all cases the HE/AuC sends an authentication data response with a new batch of authentication vectors to the SN/VLR. If the counter SQNHE was not reset then these authentication vectors can be taken from storage, otherwise they are newly generated after resetting SQNHE. In order to reduce the real-time computation burden on the HE/AuC, the HE/AuC may also send only a single authentication vector in the latter case.

Whenever the SN/VLR receives a new batch of authentication vectors from the HE/AuC in an authentication data response it deletes the old ones for that user in the VLR.

The user may now be authenticated based on a new authentication vector from the HE/AuC.

Optionally, in order to minimise extra effort by the HE/AuC, in an authentication data request with synchronisation failure indication the SN/VLR may also send the concealed sequence number Conc(SQNSN) corresponding to the last authentication vector received which the SN/VLR has in storage, i.e. it may send Conc(SQNSN) = RANDSN || SQNSNf5K(RANDMS).

On receipt the HE/AuC retrieves SQNSN from Conc(SQNSN). If the counter in the HE/AuC did not have to be reset and if SQNSN = SQNHE the HE/AuC informs the SN/VLR accordingly and does not send fresh authentication vectors. (In this way, a synchronisation failure does not cause the HE/AuC to produce extra authentication vectors when they are not needed.)

Figure 11 shows how re-synchronisation is achieved by combining a user authentication request answered by a synchronisation failure message (as described in subclause 6.3.3) with an authentication data request with synchronisation failure indication answered by an authentication data response (as described in this subclause).

MS

SN

HE

User auth request (RAND || AUTN)

synch failure (RANDMS || AUTS)

Auth data request (id), 
 synch failure ind (RAND || RANDMS || AUTS)

Auth data response (auth vectors)

Figure 11: Re-synchronisation procedure

### 6.3.6	Reporting authentication failures from the SGSN/VLR to the HLR

The purpose of this procedure is to provide a mechanism for reporting authentication failures from the serving environment back to the home environment.

The procedure is shown in Figure 12.

Figure 12: Reporting authentication failure from VLR/SGSN to HLR

The procedure is invoked by the serving network VLR/SGSN when the authentication procedure fails.  The authentication failure report shall contain the subscriber identity and a failure cause code. The possible failure causes are either that the network signature was wrong or that the user response was wrong.

When the home environment receives the authentication failure report it shall respond by an acknowledge back to the serving network. The HE may decide to cancel the location of the user after receiving an authentication failure report.

### 6.3.7	Length of sequence numbers

Sequence numbers shall be sufficiently long so that they cannot wrap around during the lifetime of the system. Consequently, in normal operations neither SQNMS nor SQNHE can wrap around during the lifetime of a USIM.

Note 1:	If the counters would derive sequence numbers from time (see Annex C), then a 32-bit counter that is derived from the number of seconds that have elapsed since January 1, 2000 would only wrap around in the year 2136. So a length of 32-bits for the sequence numbers and counters should be sufficient. For individual incremental counters, a smaller range of sequence numbers should be sufficient, as authentication and key agreement is expected to occur far less frequently than once every second. Shorter lengths would however exclude the use of time-derived sequence numbers.

Note 2:	Sequence numbers for CS and PS operation are expected to have the same length.

### 6.3.8	Support for window and list mechanisms

In Annex C.3 and Annex C.4 respectively, the window and list mechanisms for sequence number management in the USIM are described. If one of these mechanisms is employed in the USIM and if there is no need to conceal sequence numbers then the MS shall send information on the current value of the lowest entry SQNLO in the window or list to the SN/VLR at every location update. Sequence numbers which do not need to be concealed may be generated according to Annex C.2 or Annex C.6.

When the SN/VLR authenticates a user for the first time after receiving a new value SQNLO from the MS then the SN/VLR checks whether the sequence number of the authentication vector it wants to use is greater than SQNLO. The SN/VLR uses the AV only if this is the case. Otherwise, the AV is discarded. If all AVs have to be discarded the SN/VLR requests new ones from the HE/AuC.