3GPP TSG-SA3 Meeting #101-e 	S3- 202983

- e-meeting, November 9 - 20 2020

- Source:	Philips International B.V, CableLabs

- Title:	UE&Network-assisted UE avoidance and Network detection of FBS

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes a solution for KI#3 in TR 33.809.

# 2	Detailed proposal

We ask SA3 to kindly consider including the additional solution in TR 33.809 that addresses KI#3.

**** START OF CHANGE 1 ****

## 6.X	Solution #X: UE&Network-assisted UE avoidance and Network detection of FBS

### 6.X.1	Introduction

This solution addresses the security requirements in KI #3. According to the informative Annex E of TS 33.501 [7] for UE-assisted network-based detection of false base station, measurement reports sent by the UE can be used to detect a false base station. KI#3 also states that if UEs are using information from genuine base stations belonging to an operator, such guidance information from genuine base stations belonging to the operator network can be trusted to avoid/prevent UEs from connecting to false base stations both in CONNECTED mode and IDLE mode. In particular, the 5G system should be able to:

- Detect false base stations

- Employ methods to prevent UEs from connecting to false base stations

This solution describes how to extend Annex E of TS 33.501 to achieve this functionality in KI #3 (in bold) so that UEs avoid FBS even when they are in IDLE mode.

The basic idea behind this solution is based on the cell search procedure where a UE has to adquire the synchronization signal block (SSB). SSBs are transmitted in a burst, where different SSB in the burst have a different identifier (SB#) and each SSB is transmitted in a beam in a different direction. The maximum number of SSBs in a SS burst beams depends on the frequency. There are up to 4 SSBs if the frequency is under 3 GHz, up to 8 SSBs if the frequency is between 3 and 6 GHz, and up to 64 SSBs for frequencies higher than 6 GHz. Depending on the relative position of the UE with regard to a base station, the UE will measure a different power level for different beams. The UE will use the beam with the highest received power.

Figure 1 - Example

For instance, in Figure 1, UE2 scans the trustworthy base station (TBS) and receives the highest power from the green beam, the lowest power from the blue beam, and a power level in between the previous two from yellow and red beams.

Thus, if a UE knows (1) the position and expected radiation pattern (direction of the SB# sent by different beams) of a base station and (2) its own UE location, then the UE can estimate the expected SB# power distribution. Next, the UE can receive the SS burst of the base station and the measured SB# power distribution for the different beams. The UE can correlate both signals, e.g., regarding number of beams or relative powers, to obtain an indication whether the base station is a good one or not.

For instance, if UE2 in Figure 1 scans the fake base station (FBS), it would receive the highest power levels from the blue and yellow beams and a lower power level from the green and red beams. Then UE2 would have a strong indication that the BS is a fake one since this is not the expected radiation pattern from the trustworthy base station.

A way to see this proposal is to compare it with the operation of an email spam filter: (a) If an attacker writes a targeted phishing email towards a user, the spam filter usually cannot detect it; (b) a spam filter sometimes does not manage to filter out 100% of the spam. However, spam filters are still very useful since they manage to get rid of most of the spam reducing the impact of attacks. This proposal proposes a solution with similar features.

- An attacker might try to focus on a single victim, e.g, UE1 in Figure 1. In this case, UE1 receives the same power pattern from the FBS as it would receive from the trustworthy BS since the attacker is focusing on UE1 in his attack and the attacker has redirected the beams of the FBS so that they have the same radiation pattern as TBS from UE1’s location. Even if this targeted attack is feasible, all other UEs in the area will receive the wrong power patter. Thus, this approach reduces the impact of FBS attacks and it can do this already during the cell search procedure, even before the UEs connect to a BS and they are still in RRC_Idle mode. Even if an attacker can launch such a targeted attack towards UE1, UE1 will also realize inconsistencies since there are two base stations with exactly the same radiation pattern. The UE will share this information with the CN following Annex E of TS 33.501.

- Real-world experiments, e.g., [26], indicate that the measured SB# power distribution correlates with the expected beam distribution. Thus, this solution will allow the majority of the UEs to “filter out” most FBS very early in the cell search process, i.e., still in IDLE mode, saving resources and improving performance.

It is true that due to the wireless propagation features, a UE might measure a slightly different power distribution at some locations, e.g., due to reflections. UEs in those locations might classify a base station as less trustworthy.

This approach complements the methods in Annex E of TS 33.501.

### 6.X.2	Solution details

UE Initialization

- This solution requires that each UE has some information (INFO_gnb) about surrounding base stations. INFO_gnb includes:

  - amount of SSB beams used by each base station, identified by its Physical Cell Identifier (PCI).

  - (optional) the position of trustworthy base stations as well as the beam orientation (radiation pattern) when transmitting SSBs.

  - (optional) the position of known fake base stations as well as their known radiation pattern, assuming that the network has detected any FBS before.

INFO_gnb can be pre-configured in advance. Alternatively, the first time a UE enters a tracking area, the UE receives base station information (INFO_gnb) for that tracking area from the network.

The usage of the optional INFO_gnb requires that a UE can obtain its own location.

The distribution of INFO_gnb must be done in a secure way. This can be done by using AS security.

During this UE initialization phase, a UE does not have INFO_gnb, thus, the UE cannot discern trustworthy BS from FBS in IDLE mode. As soon as UE goes into connected mode, INFO_gnb is received in a secure way.

UE Operation (IDLE mode)

- BASIC CHECK: When a UE in IDLE mode needs to connect to the network, the UE performs cell search, the UE obtains the SSBs of a base station and checks whether the number of SSB beams observed from a base station with a given PCI matches the expected number of beams in INFO_gnb.

If this check does not succeed, the UE returns a low similarity score and goes to Step 4.

If the check succeeds, it returns a high similarity score and goes to Step 3.

- EXTENDED CHECK: If the UE has been provisioned with the optional INFO_gnb data and has access to its own location (e.g., through GPS data), the UE monitors the power level for different beams (PL_measured). Given its own position, and the known position of the base station it is scanning, the UE estimates the expected power level (PL_estimated) that it should measure. The UE can compute the similarity between both signals, PL_measured and PL_estimated, updating the similarity score (SS). For instance, the UE can compute the cross-correlation of the signals to obtain the SS.

- For each scanned base station, the UE computes such a similarity score, giving preference to base stations with a high SS when taking the final decision on joining a specific base station. An example of the expected operation is as follows:

Example 1: a UE observes two base stations and both of them are supposed to be trustworthy base stations. However, the SS obtained from the base stations are quite different. For BS#1, the SS is positive meaning that the radiation pattern is as expected, but for BS#2 the radiation pattern does not match. Thus, the UE takes the decision to join BS#1. The UE sends measurements to the core network.

UE Operation (CONNECTED mode)

- When the UE is in RRC-Connected state, the UE keeps scanning and reading SSBs and measuring the SS. If a UE is connected to a base station, and its SS is low, e.g., when it changes its position, while the SS of other cells are higher, then the UE can take the decision to switch cells.

- Once the UE has joined a base station, the UE provides the network with measurements regarding the measured radiation patterns at certain positions when obtaining the SSB and the computed SS. This information can be used by the network to (i) verify the UE decisions, (ii) centrally identify FBS updating INFO_gnb, or (iii) improve the algorithm to compute SS score.

Network operation:

- Network initialization: The network is in charge of tracking of the positions of base stations and their radiation patterns.

- (UE in IDLE mode) When a UE enters in a given area, the network is in charge of distributing INFO_gnb to the UE.

- (UE in CONNECTED mode) The network collects from the UE information in a secure way about the radiation pattern that a UE observes from different base stations at different locations. This information can be used together with other parameters, by the network to:

  - Verify the computations by the UE,

  - Estimate the position of FBS,

  - Improve the SS computation algorithm on UE.

Editor’s note: Feasibility and effectiveness in a real environment are for FFS.

Editor’s note: RAN needs to evaluate the potential impact.

Editor’s Note: Beamforming is not a compulsory feature. Whether this solution works when a gNB does not use beamforming is FFS.

Editor’s Note: The complexity of the storage and management of gNB information on the UE, while the UE is mobile, is FFS.

### 6.X.3	Evaluation

This solution fulfils Key Issue #3 when the UE is in the RRC-Idle and RRC-connected state, in particular:

- It allows a UE to detect/avoid FBS

- It allows the network to verify the computations of the UE and detect/avoid FBS

This solution fits in the framework for false BS detection in TS 33.501 Annex E, with extensions according to KI#3 to use information from genuine base stations belonging to an operator to prevent UEs from connecting to false base stations.

This solution enhances the measurements in Annex E of TS 33.501 to get a more accurate estimation of the existence and positions of FBSs.

This solution allows the UEs to assess whether a base station the UE is about joining or it has joined is trustworthy, or not. This assessment does not need to prevent the UE from joining any BS. However, this assessment gives a higher preference to those base stations with a good similarity score, and in this way, more UEs will join honest BS instead of FBS, making the system more robust. This also means that the attacker’s reward when placing FBSs is lower.

This solution cannot be used if the network operator does not disclose information regarding the number of SSBs associated to a base station.

The basic check in which a UE only checks the number of SSBs associated to a base station is a robust check that is less prone to propagation issues and does not require the knowledge of the locations of base station or UE or the orientation of the base station beams when broadcasting the SSBs.

This solution might lead to a situation in which a UE skips a trustworthy base station or chooses a different trustworthy BS if the radiation pattern that it measures at its location from that BS is different than the one that the UE had expected. This might be caused due to obstables that hinder the signal propagation, e.g., being inside of a building.

This solution makes use of information about the trustworthy base stations to prevent the UEs from joining FBSs. This is a robust method, likely more robust than using information about the known FBS with the same goal. The reason is that an attacker can change the features (position, name, frequency,…) of a FBS very fast, while the features of trustworthy base stations can either remain static or can be published in advance so that the UEs are always aware of them.

This solution has minimum impact on the existing signalling procedure. Impacts on the specific elements are as follows:

-	Impacts on the UE

- Modification of the cell search procedure to take into account the measured power levels during SBB adquisition and the similarity score when taking the decision of joining a base station.

- Modifications to be configured with INFO_gnb.

- Modifications to send SSB measurements to the network (extending Annex E of TS 33.501).

-	Impacts on the core network

- Requires providing UEs with INFO_gnb.

- Requires receiving measurements from the UEs related to the measured SSB signal and computed similarity scores.

**** END OF CHANGE 1 ****

**** START OF CHANGE 2 ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TR 33.969: "Study on security aspects of Public Warning System (PWS)".

[4]	Void

[5]	Altaf Shaik, Ravishankar Borgaonkar, Shinjo Park, and Jean-Pierre Seifert. 2018. “On the Impact of Rogue Base Stations in 4G/LTE Self Organizing Networks”. In Proceedings of the 11th ACM Conference on Security & Privacy in Wireless and Mobile Networks (WiSec '18). ACM, New York, NY, USA, 75-86. DOI: https://doi.org/10.1145/3212480.3212497

[6]	3GPP TR 28.861: "Telecommunication management; Study on the Self-Organizing Networks (SON) for 5G networks"

[7]        	3GPP TS 33.501: “Security architecture and procedures for 5G system”

[8]	IEEE1363.3-2013 - IEEE Standard for Identity-Based Cryptographic Techniques using Pairings

[9]	RFC6507, “Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI)”

[10]	SM9, “Identity-based cryptographic algorithms SM9”

[11]	ISO 29192-4, SC27 Lightweight crypto useing I2R-IBS

[12]	Dan Boneh; Ben Lynn & Hovav Shacham (2004). "Short Signatures from the Weil Pairing". Journal of Cryptology. 17 (4): 297–319.

[13]	3GPP TS 23.502: “Procedures for the 5G System (Release 15)”.

[14]	3GPP TR 33.861 v1.0.0: “Study on evolution of Cellular IoT security for the 5G System (Release 16)”

[15]	3GPP TS 36.331: " E-UTRA; Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.300: “NR; NR and NG-RAN Overall Description”

[17]	3GPP TS 38.413, “NG-RAN; NG Application Protocol (NGAP)”.

[18]	3GPP TR 36.355 "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[19] 	Rupprecht, David & Kohls, Katharina & Holz, Thorsten & Poepper, Christina. (2020). “IMP4GT: IMPersonation Attacks in 4G NeTworks”. 10.14722/ndss.2020.24283. https://imp4gt-attacks.net/media/imp4gt_camera_ready.pdf.

[20] 	Rupprecht, David & Kohls, Katharina & Holz, Thorsten & Popper, Christina. (2019). “Breaking LTE on Layer Two”. 1121-1136. 10.1109/SP.2019.00006. Available online at https://alter-attack.net/media/breaking_lte_on_layer_two.pdf.

[21]	Chlosta, Merlin & Rupprecht, David & Holz, Thorsten & Pöpper, Christina. (2019). “LTE security disabled: misconfiguration in commercial networks”. 261-266. 10.1145/3317549.3324927. Available online http://poepper.net/papers/WiSec19-LTEmisconfig.pdf.

[22]	Hussain, Syed & Echeverria, Mitziu & Karim, Imtiaz & Chowdhury, Omar & Bertino, Elisa. (2019). “5GReasoner: A Property-Directed Security and Privacy Analysis Framework for 5G Cellular Network Protocol”. 669-684. 10.1145/3319535.3354263.

[23]	Hojoon Yang, Sangwook Bae, Mincheol Son, Hongil Kim, Song Min Kim, and Yongdae Kim. 2019. “Hiding in plain signal: physical signal overshadowing attack on LTE. In Proceedings of the 28th USENIX Conference on Security Symposium (SEC’19)”. USENIX Association, USA, 55–72.

[24]	IETF RFC 4082. “Timed Efficient Stream Loss-Tolerant Authentication (TESLA): Multicast Source Authentication Transform Introduction".

[25]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[26]	A. Sibila, 5G NETWORK DEPLOYMENT SEMINAR, March 2019

**** END OF CHANGE 2 ****