3GPP TSG-SA3 Meeting #101-e 	S3-20xxxx

- e-meeting, 9th – 20th November 2020												   Revision of S3-203114

- Source:	Nokia, Nokia Shanghai Bell

- Title:	SCAS IMS: New Sub-Test Case against Bidding-down on Security Association

- Document for:	Approval

Agenda Item:	4.15

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed sub test case to be added in the current test case of “Bidding down on security association set-up” in TS 33.226.

# 2	References

[1]			3GPP TS 33.226	v0.2.0		Security assurance for IP Multimedia Subsystem (IMS)

[2]			S3-203113					Threat of bidding down attack on security association

# 3	Rationale

Based on the additional threat analysed in S3-203113, this pCR proposes to add the corresponding requirement and a sub-test case in clause 4.2.2.3.2 “Bidding down on security association set-up” in TS 33.226.

# 4	Detailed proposal

*************** Start of the Change ****************

##### 4.2.2.3.2	Bidding down on security association set-up

Requirement Name: Bidding down on security association set-up

Requirement Reference: TS 33.203 [3], clause 7.2

Requirement Description:

"After receiving SM7 from the UE, the P-CSCF shall check whether the integrity and encryption algorithms list, SPI_P and Port_P received in SM7 is identical with the corresponding parameters sent in SM6. It further checks whether SPI_U and Port_U received in SM7 are identical with those received in SM1. If these checks are not successful the registration procedure is aborted. The P-CSCF shall include in SM8 information to the S-CSCF that the received message from the UE was integrity protected as indicated in clause 6.1.5. The P-CSCF shall add this information to all subsequent REGISTER messages received from the UE that have successfully passed the integrity check in the P-CSCF. "

"If the P-CSCF is configured to apply confidentiality whenever the UE supports it then the P-CSCF always includes the encryption algorithms in SM6, which it supports, even if the UE did not include encryption algorithms in SM1".

as specified in TS 33.203 [3], clause 7.2.

Threat References: TBD

Test case:

Test Name: TC_BIDDING_DOWN_ON_SECURITY_ASSOCIATION_SET UP

Purpose:

Verify the P-CSCF checks whether the integrity and encryption algorithms list, SPI_P and Port_P received in SM7 is identical with the corresponding parameters sent in SM6.

Verify the P-CSCF checks whether SPI_U and Port_U received in SM7 are identical with those received in SM1.

Verify whether the P-CSCF abort the registration procedure, if the above checks are not successful.

Verify that the P-CSCF configured to apply confidentiality includes the encryption algorithms it supports in SM6, when receiving no encryption algorithms supported by the UE in SM1.

Procedure and execution steps:

Pre-Conditions:

-	The P-CSCF under test is connected in simulated/real network environment.

-	The list of ordered integrity and encryption algorithms are configured on the P-CSCF under test.

-	The P-CSCF under test is configured to apply confidentiatlity whenever the UE supports it.

-	The UE and the S-CSCF are simulated.

-	The UE supports a list of ordered integrity and encryption algorithms. The list contains at least one encryption algorithm other than NULL algorithm.

-	The tester has access to the Gm interface between the UE and P-CSCF.

-	The tester has access to the Mw interface between the P-CSCF and S-CSCF.

Execution Steps

This test is performed in the registration procedure, the UE sends a Register message towards the S-CSCF through the P-CSCF to register the location of the UE and to set-up the security mode.

Test case 1:

1)	The UE includes SPI_U, Port_U and UE integrity and encryption algorithms list in SM1, but the tester changes the UE’s algorithm list by removing the encryption algorithms before the UE sends SM1 to the P-CSCF under test.

2)	The P-CSCF under test receives SM1 and sends SM2 to the S-CSCF.

3)	The P-CSCF under test receives SM4 from the S-CSCF and sends SM6 to the UE.

Test cases 2~5 are performed as follows:

1) The UE sends SM1 with the Security Parameter Index values (SPI_U) and the protected ports selected by the UE (Port_U) to the P-CSCF under test.

2) The P-CSCF under test receives the SM1 with the Security Parameter Index values (SPI_U) and the protected ports selected by the UE (Port_U). The P-CSCF under test store the SPI_U and the Port_U received in the SM1.

3) The P-CSCF under test contains the SPI_P, the ports assigned by the P CSCF (Port_P) and a list of integrity and encryption algorithms supported by the P-CSCF under test. The P-CSCF under test sends SM6 to the UE.

4) The UE receives the SM6 from the P-CSCF under test.

Test case 2:

5)	The UE contains the incorrect SPI_U and Port_U, which are different from SPI_U and Port_U sent in SM1, and SPI_P and Port_P received in SM6, and a list of integrity and encryption algorithms received in SM6 supported by the P-CSCF under test in the SM7. The UE sends SM7 to the P-CSCF under test.

Test case 3:

5)	The UE contains the incorrect SPI_U and Port_U, which are different from SPI_U and Port_U sent in SM1, and incorrect SPI_P and Port_P, which are different from SPI_U and Port_U received in SM6, and a list of integrity and encryption algorithms received in SM6 supported by the P-CSCF under test in the SM7. The UE sends SM7 to the P-CSCF under test.

Test case 4:

5)	The UE contains the SPI_U and Port_U sent in SM1, and incorrect SPI_P and Port_P, which are different from SPI_U and Port_U received in SM6, and a list of integrity and encryption algorithms supported by the P-CSCF under test in the SM7. The UE sends SM7 to the P-CSCF under test.

Test case 5:

5)	The UE contains the SPI_U and Port_U sent in SM1, and SPI_P and Port_P received in SM6, and a list of integrity and encryption algorithms in the SM7 which are different from those sent by the P-CSCF under test in the SM6. The UE sends SM7 to the P-CSCF under test.

Expected Results:

For test case 1, the UE selects the first integrity and encryption algorithm combination on the list from the P-CSCF under test received in SM 6 which is also supported by the UE, and enforces integrity and confidentiality protection on SM7 sent to the P-CSCF under test. When receiving SM7, the P-CSCF under test fails to decrypt the message.

For test cases 2-5, the P-CSCF under test aborts the registration procedure.

Editor’s note: How to test the procedure abortion by the P-CSCF is FFS.

Expected format of evidence:

Provide evidence of the check of the product documentation in plain text. Save the logs and the communication flow in a .pcap file.

*************** End of the Change ****************