3GPP TSG-SA3 Meeting #101-e 	S3-20xxxx

- e-meeting, 9th – 20th November 2020												Revision of S3-203164

- Source:	Nokia, Nokia Shanghai Bell

- Title:	SCAS VNP: Threats on VNF-VNFM Interface

- Document for:	Approval

Agenda Item:	5.2

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed changes in TR 33.818 v0.8.0.

# 2	References

[1]	3GPP TR 33.818 v0.8.0	Security Assurance Methodology (SECAM); and Security Assurance Specification (SCAS);  for 3GPP virtualized network products

# 3	Rationale

In current TR 33.818 v0.8.0 [1], the analysis of threats on the interface between 3GPP VNF and VNFM is not quite correct. A compromised VNFM can terminate a VNF or tamper with VNFD of a 3GPP VNF. However, the VNFM cannot terminate a VNF via the VNF-VNFM interface. The termination of a VNF is conducted along the way from the VNFM to VIM then to the NFVI. The attack via a tampered VNFD is not conducted via the VNF-VNFM interface either. Therefore, this pCR proposes to remove the current analysis and add some new text for the threat analysis.

# 4	Detailed proposal

*************** Start of the Change ****************

- 5.2.4.2.2.3 Threats relating to ETSI-defined interfaces

Two of the interfaces defined in ETSI NFV specification [11] are identified as the critical assets of GVNP type 1, i.e. interface between VNF and VNFM, interface between 3GPP VNF and virtualisation layer. The threats on these interfaces are as follows.

-	Threats on interface between 3GPP VNF and VNFM: if the interface is not protected, an attacker can attack all the requests/responses sent between the VNF and the VNFM. For example, the attacker can insert, tamper or delete e.g. scaling requests, healing requests, subscribe requests, query requests and other management related requests sent from the VNF to the VNFM, hence the virtualized resource or relevant status information obtained by the VNF is not as requested.

NOTE:	The virtualization layer is out of 3GPP scope, but its protection will affect the security of the upper layer it supports. If the virtualization layer is compromised, the VNF on top of it could also be easily compromised. In such case, the messages sent over the VNF-VNFM interface can be manipulated by the compromised VNF, which is however not a threat coming from the VNF-VNFM interface. The analysis above focuses on the threats directly placed on VNF-VNFM interface, when it is not well protected.

-     Threats on interface between 3GPP VNF and virtualisation layer: an attacker can attack a VNF through a compromised virtualisation layer. For example, cryptographic keys or other security critical data of a VNF could be stolen by an attacker with access to the virtualisation layer.

Editor’s note: The current GVNP model Type1 does not contain VNMF, so the model needs to be updated accordingly to reflect this threat. The threats may need to be revisited once Type 1 model is updated.

Editor’s note: More threats described in 3GPP TR 33.848[9] or/and ETSI specification etc. are to be added if identified as related to the above two interfaces.

*************** End of the Change ****************