3GPP TSG-SA3 Meeting #101-e	S3-203177r3

- e-meeting, 9th - 20th November 2020																		Revision of S3-203177/2877/3088/3296

- Source:	Qualcomm Incorporated, Interdigital, Huawei?, Hsilicon?, Lenovo?, Motorola Mobility?

- Title:	Modification of the pairing key issue

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution proposes a modification of the pairing key issue.

# 2	References

[1]	3GPP TR 33.854 v0.2.0 “Study on security aspects of Unmanned Aerial Systems (UAS)”

# 3	Rationale

This contribution proposes a modification of the pairing key issue in order to clarify the requirements and enable the removal of the Editor’s note on whether the key issue is in scope. There are multiple aspects of pairing in an overall UAS system but the one that is in scope 3GPP is only allowing connectivity between a UAV and UAVC after authorization by the USS/UTM. The threats and requirements are modified to clearly capture this aspect.

# 4	Detailed proposal

It is proposed that SA3 approve the following pCR for inclusion in the TR [1].

**** START OF CHANGES ****

- 5.2	Key Issue #2: Pairing authorization for UAV and UAVC

- 5.2.1	Key issue details

Each UAS consists of one UAV Controller (i.e. UAVC) and one UAV.

It is required in TR 23.754 [3] that

- 3GPP system shall enable UTM to associate/pair the UAV and UAVC.

- Pairing is authorized by the USS/UTM and the result is made known to the PLMN

- Pairing between UAV and UAVC for the use of their connection may be at least authorized

This key issue discuss the detailed 3GPP security procedure for the pairing authorization of UAV and UAVC.

- 5.2.2	Threats

If pairing authorization of UAV and UAVC is not performed securely before establishment of a connection between the UAV and UAVC, an unauthorized UAVC may be able to communicate with the UAV and perform an unauthorised flight which could cause tremendous risks to the security of UAS and public safety.

If 3GPP system is not capable to receive revocation of the connectivity pairing authorisation from UTM/USS, then UTM/USS might not be able to take appropriate measures to deal with misbehaving UAVs and they might cause accidents or become attack vectors.

- 5.2.3		Potential Security requirements

3GPP system shall support enabling authentication and authorization by the USS/UTM of a UAV and UAVC pairing before enabling a data connection between the UAV and UAVC.

3GPP system shall provide means for the UTM/USS to revoke a UAV and UAVC pairing authorization in order to close the connection between the UAV and UAVC.

NOTE: The authorization decision for pairing a UAV and a UAVC is always done by the USS/UTM. The 3GPP system will support the USS/UTM to enforce that decision.

**** END OF CHANGES ****