3GPP TSG-SA3 Meeting #101e 	S3-203185

- e-meeting, 9 - 20 November 2020													      Revision of S3-202064

- Source	:	Qualcomm Incorporated

- Title	:	Key Issue on Impact of Simultaneous CU-UP applying common UP security

- Document for	:	Approval

Agenda Item	:	5.13

# 1	Decision/action requested

This pCR proposes a Key Issue to TR 33.840 on Impact of Simultaneous CU-UP applying common UP security for a UE’s UP security

# 2	References

[1]	3GPP TS 38.401, ‘NG-RAN Architecture Description’, Rel-16

[2]	3GPP TR 38.823, ‘Study of further enhancement for disaggregated gNB’, Rel-16.

[3]	3GPP SP-200349 Study on security aspects of the Disaggregated gNB Architecture.

# 3	Rationale

This key issue was proposed by multiple companies in SA3 #100bis-e (S3-202064) and co-signed by additional companies after revision (i.e., draft_S3-202064-r8) during the e-meeting but was not agreed at the end of the meeting.

This paper proposes to add a security requirement to draft_S3-202064-r8. The new requirement is formulated to address the threat described in clause 5.X.2 in a general way.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.840.

*****Start of Change*****

## 5.X	Key Issue #X: Impact of gNB  applying common User Plane Security on UE simultaneous UP connections with multiple gNB-CU-UPs

### 5.X.1	Key issue details

A gNB may consist of a gNB-CU-CP, multiple gNB-CU-UPs and multiple gNB-DUs. The gNB-CU-CP selects the appropriate gNB-CU-UP(s) for the UE requested services as defined in TS 38.401. To provide different set of services simultaneously for a UE, a gNB-CU-CP can select one or more gNB-CU-UPs at different physical locations (for example one for an URLLC service and the other for an eMBB service) but establishes a common user plane security as in Rel.15 and Rel-16. Applying same user plane security for all simulatenous user plane connections of a UE irrespective of its gNB-CU-UP location may impact the user plane security of the UE UP connection with gNB-CU-UP in other locations.

NOTE: It is FFS if there is any impact on the E1 interface.

### 5.X.2	Security threats

A compromised gNB-CU-UP entity may impact the user plane security of UE(s) which has at least one user plane connection established with other gNB-CU-UP entities other than the compromised gNB-CU-UP.

This

### 5.X.3	Potential security requirements

The 5G system shall support an on demand mechanism to ensure that the compromise of a gNB-CU-UP entity does not impact the security of the user plane traffic passing through other gNB-CU-UPs serving the same UE..

NOTE: Changes are applicable to Rel-17 onwards and solution should support backward compatibility.

Potential solutions shall not compromise the requirement that the NG-RAN internal architecture (i.e., CP/UP separation and DU/CU split) is invisible to the UE.

*****End of Change*****