3GPP TSG-SA3 Meeting #101-e 	S3-203243-r1

- e-meeting, 9th - 20th November 2020												Revision of S3-20xxxx

- Source:	Samsung

- Title:	New KI on Integrity protection for transmitted data between AF and NWDAF

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is proposed to approve this pCR to add a new KI onIintegrity protection of transmitted data between AF and NWDAF in TR 33.866.

# 2	References

[1]	3GPP TR 23.700-91 : " Study on enablers for network automation for the 5G System (5GS) ; Phase 2".

# 3	Rationale

In key issue #8 in 23.700-91 [1] for the UE data collection for deriving the analytics, it is stated that “The network should be able to ensure the privacy, integrity, and Operator-level accessibility of the data provided from the UE”.

Therefore, by considering the above-mentioned requirement, this contribution proposes a new key issue for integrity protection of transmitted data between the AF and NWDAF.

# 4	Detailed proposal

**** START OF CHANGE ****

## 5.X	Key Issue #X: Key issue on integrity protection of data transferred between AF and NWDAF

### 5.X.1	Key issue details

The 5GS supports the collection and utilisation of data provided by the UE in NWDAF in order to provide input information to generate analytics information (to be consumed by other NFs).

As per KI#8 in TR 23.700-91 [1], there is no direct interface between the UE and the NWDAF. When AF is used for the communication between the NWDAF and UE for data collection, there is a need to study the security aspects for the data provided by the UE to NWDAF via AF and vice versa.

This key issue studies the integrity aspects on data collection and utilization of UE data in order to derive the analytics.

### 5.X.2	Security Threats

If the data shared between new interface (i.e between AF and NWDAF) is not secured, it may lead to following issues;

Data can be modified and replayed by any unauthorized parties.

5.X.3	Potential Requirements

Integrity and replay protection shall be supported on the new interface between AF and NWDAF.

**** END OF CHANGE ****