3GPP TSG-SA WG3 Meeting #101-e 	S3-203264

- e-meeting, 9th-20th November 2020													revision of S3-20xabc

- Source:	Ericsson

- Title:	Proposed TR Assumptions on credentials

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is proposed to approve this text as Assumptions on credentials to TR 33.857

# 2	References

[1]	3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

# 3	Rationale

This paper proposes to assume that the authentication methods are based on the EAP framework. For SNPNs it is assumed to be a key generating EAP method as specified in Annex I in 3GPP TS 33.501[2] while for PNi-NPN there is not yet a strong statement on which kind of EAP methods to be supported. It is therefore proposed to distinguish the two deployment options.

During SA3#100-e and SA3#100bis-e it was discussed which credentials to be in scope of provisioning. While there was disagreement on credentials with requirements on USIM storage specified in 33.501[2] it was clear that credentials without USIM storage requirements are in scope.

The intention of the assumptions is to clarify which credentials are in scope of the study.

# 4	Detailed proposal

#### ***** Start of Change *****

# 4	Architectural and security assumptions

Editor's note:	This clause includes the architectural and security assumptions applicable for the study.

## 4.1	Architectural requirements

-	Solutions are built on the 5G System security architectural principles as in TS 33.501 [2] and conclusions drawn in TR 23.700-07 [3], including flexibility and modularity for newly introduced functionalities.

## 4.X	Security assumptions

-	It is assumed for the case where non-USIM credentials are provisioned for SNPN, the non-USIM credentials are basedkey generating EAP method type.

-	It is assumed for the case where non-USIM credentials are provisioned for PNI-NPN, the non-USIM credentials are based oAP method type.

#### ***** End of Change *****