3GPP TSG-SA3 Meeting #101-e 	S3-203319

- e-meeting, 9th - 20th November 2020												Revision of S3-20xxxx

- Source:	Lenovo, Motorola Mobility

- Title:	Removal of EN on AMF routing

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This document proposes a resolution of the EN on AMF routing.

# 2	References

[1]	3GPP TR 33.839 Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC, (Release 17)

# 3	Rationale

It is proposed that the AMF registers the GPSI at the NEF with the AMF ID. This can be also repeated at AMF relocation ot the NEF subscribes for notifications for AMF changes.

# 4	Detailed proposal

Begin of Changes

## 6.7	Solution #7: Authentication and Authorization with the Edge Data Network

### 6.7.1	Solution overview

The solution addresses the following key issues:

-	Key issue #1: Authentication and Authorization between EEC and EES

-	Key issue #2: Authentication and Authorization between EEC and ECS

-	Key issue #6: Transport security for the EDGE-1-9 interfaces

The solution is based on the KAMF generated during the primary authentication. The network function that receives a registration request is querying the previous network function for authentication and the key for setting up an IPsec SA. Messages are protected with a MAC-I, which is also used to authenticate the UE.

### 6.7.2	Solution details

Figure 6.7.2-1: Authentication and Authorization with the Edge Data Network

1. The UE performs normal primary authentication and registration to the network. The UE is MEC capable and may indicate this in the MEC capabilities to the AMF during the registration procedure.

2.	The AMF sends an Identifier Registration Request to the NEF including the GPSI.

NOTE: The solution assumes that the GPSI is either configured in the UE and provisioned in step 1 to the AMF, or configured in the subscriber data and provisioned to the UE after PDU Session establishment, or both.

3.	The NEF stores the GPSI and the AMF ID together.

4. 	The NEF acknowledges the Identifier Registration and may subscribe to AMF changes.

5. The UE establishes a PDU Session for IP connectivity.

6. If the UE is MEC capable, then the UE and the AMF derive a key KECS for authentication with the ECS from the AMF key KAMF.

7. The UE sends an Application Registration Request with a MAC-IECS and a GPSI as well as the Serving Network name to the ECS. The MAC-IECS is computed in a similar way as e.g. the SoR-MAC-IAUSF as defined in Annex A.17 of TS 33.501 [7]. The MAC-IECS is based on the payload of the Application Registration Request, which form the input Application Registration Request Data, and the key KECS to the KDF. The MAC-IECS is identified with the 128 least significant bits of the output of the KDF.

8. The UE is not authenticated at the ECS and the ECS sends a Key Request including the Application Registration Request with the MAC-IECS to the NEF, which is selected based on GPSI and the Serving Network name.

Editor's note: A new network entity to handle the EC service requests to the serving AMF is FFS.

9.	The NEF authorizes the request from the ECS and identifies the AMF ID based on the GPSI. The NEF stores the contact of the ECS (e.g. IP address, source NAI of the ECS etc.) with the GPSI in order to route the answer from the AMF back to the ECS.

10.	The NEF forwards the Key Request including the Application Registration Request with the MAC-IECS as well as the GPSI to the AMF.

11.	The AMF verifies the MAC-IECS of the Application Registration Request, i.e. it computes with the key KECS the MAC-I over the Application Registration Request payload in the similar way as the UE and compares the result with the MAC-IECS included in message. If both are identical, the message can be authenticated to be sent by the UE.

12.	The AMF sends a Key Response to the ECS, including the result of the authentication as well as the KECS.

13.	Based on the authentication result the ECS decides whether to accept or to reject the Application Registration Request from the UE. The ECS sends the Application Registration Response message to the UE including the authentication result and protects the message with a MAC-IECS based on the received key KECS in a similar way as the UE protected the payload of the message in step 4.

14.	The UE verifies the MAC-IECS and if authentication result and verification of the message are successful, then the UE establishes an IPsec SA between the UE and ECS by using the ECS key KECS. All messages are now confidentiality and integrity protected by the IPsec tunnel.

15.	The UE derives the key KEES from the key KECS using a MEC Key Distinguisher flag as input to the KDF.

16.	The UE sends an Application Registration Request with a MAC-IEES to the EES. The MAC-IEES is computed based on the payload of the Application Registration Request, which form the input Application Registration Request Data, and the key KEES to the KDF. The MAC-IEES is identified with the 128 least significant bits of the output of the KDF.

17.	The UE is not authenticated at the EES and the EES sends a Key Request to the ECS. The selection of the ECS may be based on the UE ID.

18.	The ECS identifies the UE based on the UE ID and derives the key KEES in a similar way as the UE in step 10. The ECS verifies the MAC-IEES of the Application Registration Request, i.e. it computes with the key KEES the MAC-I over the Application Registration Request payload in the similar way as the UE and compares the result with the MAC-IEES included in message. If both are identical, the message can be authenticated to be sent by the UE.

19.	The ECS sends a Key Request Response to the EES, including the result of the authentication as well as the KEES.

20.	Based on the authentication result the EES decides whether to accept or to reject the Application Registration Request from the UE. The EES sends the Application Registration Response message to the UE including the authentication result and protects the message with a MAC-IEES based on the received key KEES in a similar way as the UE protected the payload of the message in step 11.

21.	The UE verifies the MAC-IEES and if authentication result and verification of the message are successful, then the UE establishes an IPsec SA between the UE and EES by using the EES key KEES. All messages are now confidentiality and integrity protected by the IPsec tunnel.

22.	The UE derives the key KEAS from the key KEES using a MEC Key Distinguisher flag as input to the KDF.

23.	The UE sends an Application Registration Request with a MAC-IEAS to the EAS. The MAC-IEAS is computed based on the payload of the Application Registration Request, which form the input Application Registration Request Data, and the key KEAS to the KDF. The MAC-IEAS is identified with the 128 least significant bits of the output of the KDF.

24.	The UE is not authenticated at the EAS and the EAS sends a Key Request to the EES. The selection of the EES may be based on the UE ID.

25.	The EES identifies the UE based on the UE ID and derives the key KEAS in a similar way as the UE in step 17. The EES verifies the MAC-IEAS of the Application Registration Request, i.e. it computes with the key KEAS the MAC-I over the Application Registration Request payload in the similar way as the UE and compares the result with the MAC-IEAS included in message. If both are identical, the message can be authenticated to be sent by the UE.

26.	The EES sends a Key Request Response to the EAS, including the result of the authentication as well as the KEAS.

27.	Based on the authentication result the EAS decides whether to accept or to reject the Application Registration Request from the UE. The EAS sends the Application Registration Response message to the UE including the authentication result and protects the message with a MAC-IEAS based on the received key KEAS in a similar way as the UE protected the payload of the message in step 3.

28.	The UE verifies the MAC-IEAS and if authentication result and verification of the message are successful, then the UE establishes an IPsec SA between the UE and EAS by using the EAS key KEAS. All messages are now confidentiality and integrity protected by the IPsec tunnel.

Editor’s Note: Whether the Kamf can be used to derived the Kecs in case ECS is deployed by the home network is FFS.

Editor’s Note: How to solve the replay attack towards the ECS if the counter is not stored in the AMF is FFS

### 6.7.3	Solution evaluation

TBD

End of Changes