3GPP TSG-SA3 Meeting #101-e 	draft_S3-203331-r1

- e-meeting, 9 - 20 November 2020													Revision of S3-20xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Extension of KI on linkability attack

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Additional linkability attacks – extension of KI

# 2	References

[1]	TR 33.846

# 3	Rationale

Another linkability attack is added to KI#2.1.

# 4	Detailed proposal

It is proposed to update the KI#2.1 to include another linkability attack.

************ START OF CHANGES

- 5.2.1	Key Issue #2.1: Linkability by distinguishing MAC failure and synchronization failure

- 5.2.1.1	Issue details

In 5G, 5G AKA and EAP AKA’ are subjected to the linkability attacks like UMTS AKA because they inherit the error messages (MAC failure, Synch failure) from UMTS AKA. Further, tracing a UE may also be possible due to distinguishment of other interactions.

In the linkability attack, the attacker can detect the presence of a victim subscriber, in one of his monitored areas, an active attacker just needs to have previously intercepted one legitimate authentication request message containing the pair (RAND,AUTN) sent by the network to UE. The captured authentication request can now be replayed by the adversary each time he wants to check the presence of UE in a particular area. In fact, thanks to the error messages, the adversary can distinguish any UE from the one the authentication request was originally sent to. On reception of the replayed authentication challenge and authentication vectors (RAND, AUTN), the victim UE successfully verifies the MAC and sends a synchronization failure message. However, the MAC verification fails when executed by any other UE, and as a result a MAC failure message is sent. The implementation of few false base stations would then allow an attacker to trace the movements of a victim UE, resulting in a breach of the subscriber’s untraceability.

The attacker could distinguish the MAC failure and synchronization failure using the several methods, such as according to the failure cause embedded in the authentication failure message sent by the victim UE, or according the difference between the length of the response for MAC failure and the length of the response for SYNC failure. Hence, the mechanism used for mitigating the linkability attack shall take all the above methods into consideration.

However, even if a solution could make the MAC failure and synchronization failure indistinguishable, the attacker could also distinguish other interaction differences between the UE and the network to trace the victim UE, such as whether NAS SMC is triggered, Registration Accept/Reject, or the other message length differences.

- 5.2.1.2	Security Threats

In case the linkability attack occurs, it represents a breach of the user’s untraceability, the attacker can establish the traceability of a subscriber based on the study of the failure messages and can detect subscriber’s presence in a specific area by replaying one old legitimate authentication vector including (RAND and AUTN).

- 5.2.1.3	Potential security requirements

The 5G system shall support mechanisms to mitigate linkability attacks.

************ NEXT CHANGE

- 5.2.x	Key Issue #2.x: Linkability by SUCI

- 5.2.x.1	Issue details

In contrasts to key issue #2.2 where replaying a previously captured SUCI to the network, i.e. replacing a SUCI that a UE sends to the network in some NAS message (e.g. registration request, identity response) with the previously captured SUCI, and observing whether the authentication is successful was addressed, this key issue shows that the linkability attack is also possible without SUCI replay.

This SUCI-based linkability attack does not use a previously captured SUCI but uses a SUCI generated by the attacker from a SUPI. This is possible, if an attacker has gained knowledge of the SUPI of a subscriber, e.g. by IMSI catching in 4G. Since the Home Network Public Key deployed on the USIMs of all subscribers in the Home Network HPLMN, is not secret, even though an operator may attempt to make it not visible to the outside, an attacker can use this "public" key to create a valid SUCIs from a SUPI. With this ability, an attacker can perform a SUCI-based linkability attack without the need for replaying a previously captured SUCI. This means, even if SUCI replay is prevented e.g. by introducing a time stamp, the attacker will be able to carry out the linkability attack by generating valid SUCIs based on the known SUPI/IMSI and the current time.

- 5.2.x.2	Security Threats

In case the linkability attack occurs, traceability of a subscriber becomes possible if the attacker observes that the UE under attack is receiving a response to a registration request. This allows to locate a UE and different cells and represents a breach of the user’s untraceability, thus, this attack compromises privacy.

- 5.2.x.3	Potential security requirements

Same as in 5.2.1.3.

************ END OF CHANGES