3GPP TSG-SA3 Meeting #102-e 	S3-210456-r1

- e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

- Source:	Samsung

- Title:	Authentication and authorization for legacy UE

- Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

It is requested to approve this solution for key issue#4 in TR 33.862.

# 2	References

[1]	3GPP TR 23.700-24: "Study on support of the 5GMSG Service ".

# 3	Rationale

This contribution is proposed to address the security requirement for the Authentication and Authorization between legacy UE and MSGin5G server in key issue #4.

# 4	Detailed proposal

*****Start of Change*****

## 6.Y	Solution #Y: Authentication and authorization for legacy UE

### 6.Y.1	Introduction

This solution proposes to use TLS-PSK based authentication for the authentication between legacy UE and the MSGin5G server where SMSF acts as the key management function.

### 6.Y.2	Solution details

Figure 6.Y.2-1: Procedure for authentication and authorization between legacy UE and MSGin5G server.

Step 1-2: The UE performs the procedures (for example, Initial Registration procedure) as defined in TS 23.502 to get the 5GC network access. At the end of the network access authentication procedure (Primary authentication and key agreement [TS 33.501, clause 6.1]), the UE and the AMF are in possession of the key KAMF derived from KSEAF.

The UE and AMF performs the capability exchange procedure and agree upon the method of authentication for MSGin5G service during the PDU session establishment.

Step 3: The UE initiates the Initial service provisioning procedure with the SMSF.

Step 4:  SMSF sends a key request to the designated AMF. The AMF/SEAF derives the KMSG from the KSEAF/KAMF. The SMSF identifies the AMF using the 5G-GUTI which has the GAUMI which uniquely identifies one AMF.

Step 5a-5b: The UE derives the KMSG key whenever there is trigger to get MSG service. Based on the UE’s capability to support MSGin5G service, the SEAF/AMF derives the KMSG. Key derivation step is skipped, if the UE or AMF holds a valid KMSG.

Step 6: Based on the UE’s capability to support MSGin5G service, the SEAF/AMF derives the KMSG. After key material (KMSG) is generated, the SEAF/AMF sends the generated KMSG and ngKSI to the SMSF together with UE ID and/or SUPI and/or MSGin5G Service ID of the UE in the key response. The SMSF stores the latest information sent by the SEAF/AMF.

Step 7: Once KMSG is derived, PSK is generated from KMSG.

Step 8-10: SMSF sends the initial service provisioning response to the UE. On receiving the initial service provisioning response, the UE derives the KMSG-PSK. On successful initial service provisioning procedure, the UE and SMSF establishes a TLS session using pre-shared key KMSG-PSK.

Step 11-12: On successful TLS session establishment the UE sends an application token request to the SMSF. SMSF provides the access token to the UE.

Step 13: The legacy UE establishes a secure connection with the Legacy 3GPP message gateway. The legacy UE registers with the MSGin5G server via legacy 3GPP message gateway. The communication between legacy UE and the gateway is out of scope.

Step 14: Before sending the access token for authorization to the MSGin5G server, the gateway establishes a secure channel using certificates.

Step 15: The gateway sends the access token in an application registration request to the MSGin5G server along with the MSGin5G service ID.

Step 16-18: SMSF validates the received access token and the MSGin5G server validates the service request by validating the MSGin5G service ID. On successful verification MSGin5G server sends the application registration response.

Editor's Note: How to protect the spoofing of Service ID by a malicious UE is FFS.

### 6.Y.3	Evaluation

TBD

*****End of Change*****