3GPP TSG-SA3 Meeting #103e 	draft_S3-211865-r1

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	Huawei, HiSilicon

- Title:	eNA: New solution to key issue #3.3

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed new solution to key issue #3.3.

# 2	References

# 3	Rationale

This contribution proposes a new solution to key issue #3.3, Ensuring restrictive transfer of ML models between authorized NWDAF instances.

# 4	Detailed proposal

SA3 is kindly requested to approve the following new solution.

******************* Start of Change 1(All texts are new) *******************

## 6.Y	Solution #Y: Solution to ML restrictive transfer

### 6.Y.1	Introduction

This contribution proposes a solution to key issue #3.3, Ensuring restrictive transfer of ML models between authorized NWDAF instances.

This solution leverages existing SBA authorization to fulfil the requirement, i.e. “only authorized NWDAF instances should be allowed to consume ML models from other NWDAF instances.”

### 6.Y.2	Solution details

The solution is predcicated on that NDWAFs instances that are allowed to share the ML models are placed in the same set.  The existing SBA authorization mechanism is then re-used.

Access Token Request

When a service consumer NWDAF requests a token from NRF to access the service from an expected service provider NWDAF for ML provisioning or subscription, the NRF includes in the access token the NF Set ID of the expected service provider NWDAF, as specified in clause 13.4.1.1 of TS 33.501[8].

Service Request

When the service consumer NWDAF request to access the service for ML provisioning or subscription from the service provider NWDAF, the service consumer sends, among others, the access token obtained from NRF which includes the NF Set ID of the expected service provider NWDAF.

The service provider NWDAF then checks that the NF Set ID in the access token matches its own NF Set ID, as specified in clause 13.4.1.1 of TS 33.501 [8]. If the check fails, then the service consumer NWDAF is denied of the requested service.

Editor’s Note: This solution is to be revisited and re-evaluated if the KI is updated due to ongoing SA2 discussion on authorization based on model ID.

### 6.Y.3	System impact

There is no system impact.

### 6.Y.4	Evaluation

This solution requires that NWDAF instances that are allowed to share ML models are placed in the same NF set.

The solution fulfils the security requirement of the key issue #3.3.

*************************** End of Change 1 **************************