3GPP TSG-SA3 Meeting #103 	draft_S3-211877-r1

- e-meeting, 17 - 28 May 2021											Revision of S3-20xxxx

- Source:	Huawei, HiSilicon

- Title:	Updates to solution #3

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

SA3 is kindly requested to approve the updates to solution #3.

# 2	References

# 3	Rationale

This contribution proposes to update the solution #3 based on SA2 study progress, which mainly includes:

- Align the nomenclatures in this solution with the ones updated in TR 23.757, e.g. MBSF, MBSTF.

- Delete the EN on SMF and MB-SMF, according to the conclusion in TR 23.757 that MB-SMF is a functional component which may be standalone or co-located with existing function.

- Delete the multicast join via user plane from the solution, as TR 23.757 has concluded that multicast join via control plane must be used.

- Use TMGI or source specific IP multicast address for identifying a multicast session, as concluded in TR 23.757.

- Update the call flow and MTK distribution to the UE in this solution, according to the concluded call flow in TR 23.757.

Also evaluation to this solution is added.

# 4	Detailed proposal

******************* Start of Change 1*******************

## 6.3	Solution #3: MBS Traffic Protection

### 6.3.1	Solution overview

This solution addresses both KI#2 and KI#3. It is based on the converged architecture in TR 23.757 [2] which is concluded as the adopted architecture for 5G MBS.

According to TR 23.757 [2], in the adopted architecture, the MBSTF (Multicast Broadcast Service Transport Function ) and MBSF(Multicast Broadcast Service Function ) are two functional components introduced at Service Layer. MBSTF is the media anchor for MBS data, performs generic packet transport functionalities such as framing, packet encoding, etc. MBSF provides service level functionality to support MBS, interacts with AF and MB-SMF for session operations and transport, controls MBSTF, and etc. Both MBSTF and MBSF belong to the operator domain.

In this solution, the MBS traffic is protected between the MBSTF and the UE, and the protection is transparent to the content provider.  MBS Traffic Key (MTK) is generated by MBSF and securely distributed to the MBSTF and the UEs through the control plane. MBSTF uses the MTK to protect the MBS traffic before sending them out to the UE.

### 6.3.2	Solution details

Figure 6.3.2-1 shows the call flow for secure MTK distribution and multicast data protetion. The call flow in Figure 6.3.2-1 is based on the concluded multicast call flow in TR 23.757 [2].  In the call flow MB-SMF is another functional component introduced in the adopted architecture in TR 23.757 [2]. MB-SMF is used for MBS session management (including QoS control), control of MBS transport, and etc. MB-SMF may be standalone or co-located with existing network function.

Figure 6.3.2-1: MBS key distribution

Step 1. The AF of the content provider provisions to the MBSF the information on a multicast session including the the multicast group ID and the security policy for the multicast session. The multicast group ID, which identifies the multicast session, can eithr be a TMGI or source specific IP multicast address.  The NEF is involved in the provision if the content provider belongs to a 3rd party.

Step 2. If the security policy indicates the multicast session needs security protection, MBSF will generate a MTK and the associated key identifier (KID) for the multicast session. MBSF sends a request message including the multicast group ID to request MB-SMF to reserve source for the session. MBSF also provisions the MTK and the KID to MB-SMF in the request message.

MTK needs to be different for each multicast session, therefore, the multicast group ID may be used to generate MTK. The KID consists of a Key Domain ID and a MTK ID. The Key Domain ID is MCC|| MNC. A MTK ID is a number that is different for each MTK.

Step 3. MBSF sends the security policy and the MTK and KID to MBSTF.

Step 4. There are other steps defined in concluded multicast call flows in TR 23.757 [2] for the multicast session configuration at the core network (e.g. RAN configuration), the details of which are omitted in this solution as they are irrelevant to solving the key issue #2 and #3.

Step 5. To join a multicast group via control plane, UE sends to SMF  the request for a PDU session establishment/modification and includes in the request the multicast group ID the UE wishes to join.

Step 6. SMF sends to MB-SMF a request for the contexts of the multicast session identified by the multicast group ID.

Step 7. MB-SMF replies to SMF with MTK and KID in the response message.

Step 8. There are other steps for the PDU session establishment/modification (e.g. N4 session creation or modification).

Step 9. SMF  sends the received MTK and the KID to UE.

Step 10. When the multicast data of the session identified by the multicast group ID is received at MBSTF, MBSTF uses the received MTK to protect the multicast data if the received security policy from step 3 indicates security policy is needed. The protected multicast data along with the KID are sent to the UE. Based on the received KID, the UE uses the received MTK in step 9 to decrypt the protected multicast data and/or verify the integrity of the protected multicast data.

### 6.3.3	Solution evaluation

In the solution, the multicast data protection is performed between UE and MBSTF, the anchor point of the multicast data. This is similar to the MBS protection before 5G, in which the protection is performed between BM-SC (the data anchor) and UE.

The MTK and KID，used for the multicast data protection, is generated by MBSF when needed and securely distributed to UE over the control plane, and to MBSTF. The solution conforms to the call flows concluded in TR 23.757 [2].

The solution does not affect RAN or UPF or MB-UPF. Moreover, during the transmission of the multicast data, the solution exposes the unprocted multicast data to the least network functions possible, i.e. only MBSTF has access to the unprotected multicast data.

The protection provided by the solution is transparent to UE mobility.

The solution does not rely on GBA or AKMA.

The solution does not address the update of keys used to protect the MBS traffic.

*************************** End of Change 1 **************************