- 3GPP TSG SA WG3 Meeting #104-e                            	draft_S3-212714-r2

- e-meeting, 16 – 27 August 2021	                         revision S3-21abcd

- Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

- Title:	Conclusion on Key Issue #3

- Document for:	Approval

Agenda Item:	5.14

# Decision/action requested

This contribution proposes a new conclusion in TR 33.867

# References

# Rational

The contribution proposes a new conclusion on key issue #3. The highlighted text in yellow is approved in S3-212123 in the last SA3 meeting.

# Detailed proposal

*************** Start of 1st Change ****************

- 8.3	Conclusion for Key Issue #3: Modification or revocation of user consent

The following specific aspects for use case#1 (NWDAF) are concluded for normative work:

- NWDAF shall subscribe the service for notification of revocation of user consent if the NWDAF processes data subject to user consent, and shall be notified if user consent is changed.

- If user consent is revoked, the NWDAF shall halt analyzing and collecting of data subject to the user consent.

- If user consent is revoked, the NWDAF shall notify NF service consumers to halt processing of data subject to the user consent.

- Editor’s Note: This conclusion is only sufficient for cases in which NWDAF and the data providing NF are operated by the same legal entity, other situations are FFS.

- Editor’s Note: How to delete data subject to the user consent is FFS.

*************** End of 1st Change ****************