3GPP TSG-SA3 Meeting #104e 	S3-213066

# e-meeting, 16 - 27 August 2021															  was S3-212521-r1

# Source:	CableLabs, Ericsson, Charter Communications, Intel

# Title:	pCR: Further conclusions for KI #1

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

Approve this contribution to further conclude on Key issue #1 in TR33.857

# 2	References

[1]

# 3	Rationale

This document proposes additional conclusions for KI#1 Credentials owned by an external entity for the case of AAA not supporting EAP.

# 4	Detailed proposal

***	BEGINNING OF CHANGES ***

## 7.1	Conclusions on KI #1: Credentials owned by an external entity

In case that the external entity is 5GS aware (i.e., has the AUSF/UDM and is capable of deriving 5G key hierarchy after a successful primary authentication), it is concluded that the existing 5GS roaming architecture is reused.

In case that the external entity is non-5GS aware (legacy AAA server), the following is concluded:

- The SNPN access with a credential owned by an external entity is performed via an AUSF in the SNPN that is enhanced to interface with the external entity.

- For potential use of legacy AAA by an external entity (AAA-E) that does not support key generating EAP authentication method, EAP-TTLS as specified in RFC 5281 [5] can be described in an informative annex,  with TLS tunnel terminating within the SNPN. .

Editor’s Note: Further conclusion(s) are FFS.

***	END OF CHANGES ***