3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230078

- Electronic meeting, 16 - 20 January 2023

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Sol#3 Resolution of Editor's Note on Client Credential Grant

- Document for:	Approval, Information, Discussion

Agenda Item:	5.11

# 1	Decision/action requested

Please approve proposed changes to Sol#3 as described below.

# 2	References

[1]	3GPP TR 33.884 v0.3.0 Study on security of application enablement aspects for subscriber-aware northbound API access

[2]	3GPP TS 33.122 Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs

[3]	3GPP TS 33.501 Security architecture and procedures for 5G system.

# 3	Rationale

Sol#3 in T§ 33.884 [1] contains the following EN, which is discussed and resolved by this contribution:

Editor's Note: Whether client credential grant is sufficient for authorization of API invocation to access to resource owner’s resource is FFS

This solution addresses the scenario that the API invoker is part of the UE. Third party applications on a mobile device (which in agreement with SA1 requirement are not visible to the 5G system) do not interact directly with the AEF but utilize the API Invoker on the UE. We are not aware of any issue, which would prevent or limit using the OAuth client credential grant in this scenario.

Obviously, there are other scenarios as well, in which the API Invoker is part of the third-party application (either as part of an application on the mobile device or in an application server outside of the mobile device). It is well understood that these scenarios cannot be tackled by the client credential grant, but rather require for instance the OAuth authorization code grant. However, this aspect does not affect the usability of Sol#3 for the targeted scenario of an API Invoker being part of the UE.

In summary, it is proposed to remove the Editor's Note.

# 4	Detailed proposal

Please accept the following changes to TR 33.884 [1]:

**** START OF CHANGE ****

## 6.3	Solution #3: UE Originated API invocation using OAuth Client Credential Grant

### 6.3.1	Introduction

According to KI#1 and the SA1 requirement referenced in KI#1, a UE shall be able to access a northbound API of the 5G system. The API invocation is triggered by an application, which is not visible to the 5G system.

Therefore, it is assumed that details of the interface between the application and the API invoker on the UE are out of scope of this solution.

It is proposed to use the OAuth client credential grant (specified in clause 4.4. of [4]) as a basis for the solution.

### 6.3.2	Solution details

The context of the components of the solution is visualized in Figure 6.3.2-1.

It is assumed that the application is deployed on the mobile device consisting of the UE. For instance, the application might be an app installed on a smartphone. Note, whether the application is considered being part of the UE or being collocated with the UE on a mobile device is a matter of interpretation and not relevant for the solution.

The API Invoker on the UE is taking the role of the OAuth Client. If CAPIF is applied, the Authorization Server is part the CAPIF Core Function. In this case the interface between API Invoker and Authorization Function is part of the CAPIF-1 interface.

CAPIF onboarding for assignment of Client Id and Client secret, since Client Id and Client secret can be derived from the 5G key hierarchy as explained in more detail in step 2 of the message flow below.

Figure 6.3.2-1: Solution components of UE originated API invocation using OAuth client credential grant.

A typical message flow executed for UE originated API invocation is depicted in Figure 6.3.2-2.

Figure 6.3.2-2: Message flow used for UE originated API invocation using OAuth client credential grant

The individual steps are described below:

1. The flow is triggered by the application sending a request to the API invoker on the UE.

The interface between Application and API invoker including the authorization of the Application to use this interface is out of the scope of this solution.

It is assumed that this interface can be handled like other internal APIs on a mobile device, like access to cameras or GPS location.

Editor's Note: Authorization of application on the UE might have limitations in case an application on a first UE is accessig resources of a second UE. In this case the subscriber or user of the second UE is not able to authorize the application on the first UE. This case is FFS.

2. Based on the received Request the API invoker on the UE (acting as an OAuth Client) sends an OAuth Token Request to the Authorization Server with the grant type set to "client_credentials".

The Token Request is sent as https request. As part of the session establishment the API invoker authenticates the Authorization Server by verifying the Authorization Server's server certificate.

Editor's Note: How the UE Invoker obtains a root certificate for the verification of server certificates is for further study.

The scope parameter is set by the API invoker based on the request of the application such that it covers the necessary scope of the subsequent API request in step 5.

In line with the SA1 requirement that the Application itself is invisible to the 5G system the token request does not contain information related to the identity of the Application, but the solution introduces a new entity, API invoker, in the UE, which is visible to the 5G system.

Editor's Note: Lack of visibility of the application might have implications in case a first UE wants to access resources of a second UE. In this case the subscriber of the second UE is not able to control and restrict which third party applications are used on the first UE to access resources of the first UE. This point is FFS.

The Token Request includes the authentication of the API Invoker using the API invoker's client credentials. The circumstance that the API invoker is located and associated with a UE can be exploited for instance by using AKMA

In this context the Authorization Server takes the role of the AKMA AF. The https protocol is used as Ua* protocol. Depending on whether the Authorization Server is regarded as trusted the Authorization Server can directly interact with the AAnF or via NEF. Depending on the choice the SUPI or the GPSI can be used as OAuth client Id. The AKMA key KAF can be used as client credential. Thus, the authorization header of the http Token Request sent from the UE to the authorization server can be used for http Basic authentication with A-KID and KAF as username and password, respectively.

Editor's Note: Usage of digest authentication instead of basic authenticaton or usage of alternative Ua* protocol, like TLS-PSK; is FFS.

3. The authorization server is authorizing the request based on the identity of the API Invoker and the requested scope by applying stored policies and permissions.

Note, this solution assumes that policies and permissions are pre-arranged at the authorization server. Since the application is not visible, the policies and permissions are in the granularity of UE level, not the application level.

4. In case of successful authorization the authorization server returns the token to the API invoker. The token includes claims, which reflect the granted scopes and permissions.

Editor's Note: Whether 3GPP needs to define additional scopes and claims for token requests and tokens is FFS.

5. The API invoker is sending the actual API request to the AEF. The API request contains the token received in the previous step.

Editor's Note: Mutual authentication between API invoker and AEF is FFS.

6. The AEF is validating the incoming request. That is, the AEF verifies the validity of the token, and the AEF verifies that the request is within the scope described by the claims in the token.

Editor's Note: It is FFS, if the AEF can validate the scope of the API request solely on the content of the token or if other mechanisms are needed and, if so, if further standardization is required for this purpose.

7. In case of successful validation the AEF is executing the request by involving other NFs (not shown in the signalling diagram)

8. The AEF returns the result of the API call to the API invoker.

9. The API invoker completes the flow by sending a response to the application (which is based on and might include results received in the previous step.

NOTE: Authorization from the resource owner (i.e., the user) is not considered as part of the OAuth procedure using Client Credential Grant type.

### 6.3.3	Evaluation

- The presented approach provides a solution for the case that API Invoker is part of the UE and that this API Invoker can be used by applications on a mobile device to utilize 5G northbound APIs. The approach is thus complementary to other solutions which target the case that the API Invoker is part of the third-party application.

- Usage of AKMA guarantees that the client Id of the API Invoker is really bound to the Id of the UE.

- In ths solution the 5G system only authenticates and authorizes the UE. The 5G system does not authenticate or authorize the (invisible) third party application or a user. That is, the solution assumes that the UE is the resource owner or requesting party.

- Editor's Note: Evaluation related to the circumstance that in case of one UE accessing resources of another UE, the subscriber of the second UE is not able to authorize the third party application used on the first UE, are FFS.

- The main changes required for the solution are related to the deployment of the API Invoker on the UE and the definition of interface between API Invoker and the third-party application. However, this interface is out of scope of this solution and out of scope of SA3.

- Potential changes might also be needed with respect to definition of additional scope in the northbound APIs and the way how an AEF is able to restrict the scope of API calls to the invoking UE.

Editor's Note: Furhter evaluations are FFS.

**** END OF CHANGE ****