3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230374

- Electronic meeting, 16 - 20 January 2023

- Source:	Xiaomi

- Title:	KI #1, new Sol on AKMA roaming in multiple registration scenarios

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is requested to approve the new solution for KI #1 of 3GPP TR 33.737

# 2	References

[1]	3GPP TR 33.737: "Study on Authentication and Key Management for Applications (AKMA) phase 2"

# 3	Rationale

This contribution proposes to add a new solution to KI #1 of TR 33.737 [1].

# 4	Detailed proposal

*************** Start of the 1st Change ****************

- 6.X	Solution #X: AKMA roaming in multiple registration scenarios

- 6.X.1	Introduction

This solution addresses the KI #1.

This solution considers AKMA roaming in multiple registration scenarios.

According to TS 33.501 [4], in multiple registration scenarios, UE can be multiple registered in different PLMN's serving networks.

Though UE can connect to two different visited networks, UE may only leverage one visited network to connect to the target AF. In this case, it is unreasonable to send AKMA-related keys to the visited network that does not participate in building the connection to the AF.

This contribution leverages information from PCF to identify the visited network that participates in building the connection between the UE and the AF. And AKMA-related keys are only sent to the identified visited network.

In specific, according to sub clause 6.1.3.18 of 3GPP TS 23.503 [X], AF can obtain visited network identifier from PCF using the AF session related to a specific UE. As described in clause 6.1.3.2.2 of 3GPP TS 23.503 [X], the AF session is associated with one and only one pdu session. And each pdu session is related to one specific UE.

This solution can be combined with Sol #1, #3, #9, #12, #14, and #16 to support the LI requirements.

6.X.2	Solution details

This solution can be combined with Sol #1, #3, #9, #12, #14, and #16 to support the LI requirements.

To support AKMA roaming for external AF in multiple registration scenarios, this solution reuses clause 6.3 TS 33.535 [4] with the following modifications.

- 	In step 1, the AF obtains the visited network identifier (s), which indicates the network participating in building the connection between UE and the AF, from PCF. AF sends the visited network identifier to the NEF.

-    In step 3, NEF sends the visited network identifier(s) to the hAAnF.

hAAnF sends SUPI of the UE, AF_ID, KAF, the KAF expiration time (KAF exptime) to the visited network (s), which is identified by the AF in step 1.

To support AKMA roaming for internal hPLMN AF in multiple registration scenarios, this solution reuses clause 6.2 TS 33.535 [4] with the following modifications.

- The AF obtains the visited network identifier(s), which indicates the network participating in building the connection between UE and the AF, from PCF.

- The AF sends GPSI of the UE, AF_ID, encryption key, and the encryption key expiration time to the identified visited network.

In Multi access pdus session scenarios, information related to AKMA (e.g., GPSI of the UE, AF_ID, encryption key, and the encryption key expiration time) should be sent to both two VPLMNs.

In PDU session moving secnarios, UE can move one pdu session from source access-type to target access-type. When PDU session moving happens, Information related to AKMA (e.g., GPSI of the UE, AF_ID, encryption key, and the encryption key expiration time) should be sent to the VPLMN related to the access-type.

- 6.X.3	Solution Evaluation

This solution can be combined with Sol #1, #3, #9, #12, #14, and #16 to support the LI requirements.

The advantage of this solution is mitigating the risk of sending AKMA related keys to VPLMN, which does not participate in building the connection between UE and AF.

This solution has impact to hAAnF. In specific, for external AF in the data network, external AF sends the identified visited network to the hAAnF. AKMA-related keys are only sent to the identified visited network

In Multi access pdus session scenarios, information related to AKMA (e.g., GPSI of the UE, AF_ID, encryption key, and the encryption key expiration time) should be sent to both two VPLMNs.

In PDU session moving secnarios, UE can move one pdu session from source access-type to target access-type. When PDU session moving happens, Information related to AKMA (e.g., GPSI of the UE, AF_ID, encryption key, and the encryption key expiration time) should be sent to the VPLMN related to the access-type.

*************** End of the 1st Change ****************

*************** Start of the 2nd Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.535: " Authentication and key management for applications based on 3GPP credential in 5G （AKMA）".

[3]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) ".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[5]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security ".

[6]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[7]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[X]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2"

*************** End of the 2nd Change ****************