3GPP TSG-SA3 Meeting #115-adhoc-e 	S3-241207-r1

# Online, April 15 – April 19, 2024

# Source:	Intel

# Title:	Secure Initial Registration for S&F Satellite Operation

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.700-29

# 2	References

# 3	Rationale

This contribution proposes input text for the security assumptions of TR 33.700-29.

# 4	Detailed proposal

SA3 is requested to approve the following pCR.

*****START CHANGE*****

## 6.Y	Solution #Y: Secure Initial Registration for S&F Satellite Operation

### 6.Y.1	Introduction

This solution specifically addresses the security considerations of Key Issue #1, which pertains to supporting Store and Forward Satellite operations. The Initial Attach / Initial Registration process is crucial for all S&F services. It must ensure the network's integrity and security despite the unique challenges posed by satellite communication, such as intermittent connectivity.

### 6.Y.2	Solution details

Considering a scenario with a single Low-Earth Orbit (LEO) satellite providing intermittent coverage, this solution proposes modified MME functionality: one segment aboard the satellite (MME-SAT) and the other on the ground (MME-GND). This split architecture accommodates satellite coverage's intermittent connectivity and facilitates secure communication between the UE and the network.

Figure 6.Y.2-1: Initial Attach in satellite network for S&F operation

1-3	Initial Registration Process: Upon entering the satellite's coverage, the UE initiates an Initial Attach Request. The MME-SAT, unable to immediately establish a ground connection, temporarily stores the UE's International Mobile Subscriber Identity (IMSI) and issues an Attach Reject message.

4-5        Once MME-SAT establishes contact with MME-GND, it forwards the IMSI to request authentication vectors from the Home Subscriber Server (HSS).

6-10 	In subsequent coverage, the UE re-initiates the Attach Request. This time, MME-SAT, equipped with the authentication vectors, proceeds to authenticate the UE, leading to a successful Attach Acceptance.

11-12   Location Update Process: MME-SAT updates the UE's location with the HSS upon establishing ground connectivity, ensuring the UE's subscription permits service in the attempted location. Any discrepancies trigger a detach procedure during the next satellite contact.

Editor’s Note: Aspect related to T3247 needs to be studied further.

Editor’s Note: How to handle Attach requests sent by the UE to separate satellites is FFS

Editor’s Note: how to prevent DoS attacks before the security context is established between UE and network is out of scope of this solution

### 6.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

.

*****END CHANGE*****