3GPP TSG-SA3 Meeting #116	S3-242325

Jeju, South Korea,  20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a new threat on the validity of local certificates at gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei; HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of a new threat related to the potential lack of validity checking of local certificate at gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new threat for the gNB network product class. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete SCAS coverage of the certificate enrolment functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* 1st of Change*************

D.2.2.X	Local certificate validity checking

-	Threat name: Local certificate checking at gNB

-	Threat Category: Denial of Service.

-	Threat Description: If the gNB does not have the capability to check the validity of locally stored certificate prior to usage and to expose such issue (example by raising an alarm or logging), then this may result in the peer (for example, the UPF or the AMF) rejecting the connection with the gNB. Such a late failure and all the corresponding signalling exchange with the peer are completely unnecessary. Furthermore, such issue could remain unnoticed.

-	Threatened Asset: Sufficient Processing Capability.

************* End of Change*************