3GPP TSG-SA3 Meeting #117	S3-243448

Maastricht, 19th - 23th August 2024	revision on S3-243142

# Source:	Ericsson

# Title:	Update of Solution #9: Clarification and removal of Editor's Notes

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR to TR 33.700-32 below.

# 2	References

[1]	3GPP TR 33.700-32: "Study on security aspects of User Identities and Authentication"

# 3	Rationale

Solution #9 "User Plane Based Human User ID Authentication and Authorization" contains several Editor's Notes. The first five Editor's Notes are related to overall clarity of the solution, asking specifically which steps are in SA3 scope, how the 5GC is impacted, and how the solution aligns with SA2.

Editor’s Note: Clarification on which steps are in scope and which steps are out of scope is FFS.

Editor’s Note: Removal of steps that are not related to SA3 scope is FFS.

Editor’s Note: More clarification is FFS.

Editor’s Note: Alignment with SA2 is FFS.

Editor’s Note: How to update 5GC and impact/requirements to 5GC is FFS.

These five Editor's Notes above are addressed by a rewrite of the solution, where specific consideration was put on removing aspects in SA2 remit and clarifying the layers that are affected by each step.

Furthermore, this solution contains the following Editor's Note:

Editor’s Note: How to ensure at step 3 actual user is authenticated is FFS.

This Editor's Note on authentication of the actual user is resolved by a clarification of the credentials in the Preparation step (step 0).

# 4	Detailed proposal

*** BEGIN CHANGES ***

## 6.9	Solution #9: User Plane Based Human User ID Authentication and Authorization

### 6.9.1	Introduction

This solution addresses the requirements identified in key issue #1 (Authentication and Authorization of Human User ID). It is proposed to user plane based authentication and authorization.

### 6.9.2	Solution details

The figure 6.9.2-1 presents the high-level authentication and authorization architecture. The Portal is a trusted entity from the HPLMN perspective, and can e.g. be managed by the HPLMN operator or a trusted partner. The User Identifier is authenticated and authorized by the UIP server. The UIP server is an Application Function (AF) under control of the HPLMN operator.

Figure 6.9.2-1: High-level authentication and authorization architecture

The figure 6.9.2-2 shows a flow for authentication and authorization of the User Identifier in the UIP server via the UE's UIP client.

Figure 6.9.2-2: User ID authentication over user plane

The preparation step 0. is assumed to be performed in advance, and out of 3GPP scope.

0.	 Preparation steps on application layer. The admin user of the subscription logs in at the Portal over user plane. The Portal, UIP server and the admin user create the credentials to be used for authentication of a user, potentially different from the admin user, and download the UE part of the credentials to the UE. For example, the credentials could be a user password chosen by the admin user which could be combined with a certificate provisioned at the UE, e.g. in combination with downloading an application to the UE. Alternatively, the credentials could be a user certificate that is locked with a password or biometric verification locally at the UE. The preparation step also links the credentials to the SUPI, e.g. the admin user indicates that the user is allowed to use the subscription associated with the SUPI. By using credentials in the authentication, which can be directly user provided data such as password or can be accessible with only user provided data such as password or biometric data, it is ensured that the later authentication step actually authenticates the user and not only the UE. An operator can also require control of ID or passport before the operator controlled entities (portal, UIP server) generate the credentials, if desired or required.

Steps 1-5 are performed using a user plane connection to a special DNN that gives the user access to the UIP server. Observe that the registration uses the SUPI and primary authentication is performed between UE and network based on the associated long term key.

1.	[Locally at the UE] User login to UE and the UIP client, possibly including a local authentication of the user. This step is out of 3GPP scope.

2.	[Application layer over 3GPP connection] UIP client requests user login from the UIP server.

3.	[Application layer over 3GPP connection] Authentication between user, UE/UIP client and UIP server is performed, using the credentials established in step 0. The UIP server also authorizes the user and checks which 5GC subscription (identified by the GPSI) the user is using. The UIP server can use the IP translation API to learn the GPSI.

4.	[3GPP scope studied in TS 23.700-32] After successful authentication of the user, procedures as studied in TR 23.700-32 [2] are performed.

5.	[Application layer over 3GPP connection] The UIP server sends the response to the UE/UIP client.

After Step 5, the UE and user have full user plane connectivity.

This solution proposes that Steps 2-5 are performed after activation of AS security. Furthermore, additional security mechanisms on the user plane can be enabled for steps 3 and 5, e.g. TLS with server-side certificates.

Editor’s Note: Clarification on which existing mechanism is reused and how user ID is protected.

### 6.9.3	Evaluation

TBD.

Editor’s Note: Whether the interaction at application layer (steps 2,3,5) are in 3GPP scope is FFS.

*** END CHANGES ***