3GPP TSG-SA3 Meeting #118	S3-244307

Hyderabad, India  14 – 18 October 2024

Title:	Reply-LS on PQC Migration

Response to:	LS PQTN#70 Doc_003 on LS reply to 3GPP Reply-LS on PQC Migration from GSMA

Release:

Work Item:

Source:	SA WG3

To:	GSMA PQTN

Cc:	ETSI TC CYBER QSC

Contact person:	Mohsin Khan

mohsin.a.khan@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 thanks GSMA for the LS on migration to Post-Quantum Cryptography (PQC). SA3 would like to provide the following responses to the raised questions:

- Q1: Regarding “SA3 considers that all the 128-bit algorithms currently used in 5G systems can be considered as secure against attacks from both classical and quantum computing.”, As it is known, the NIST was endorsed the AES-128, quoted “remain secure for decades” [2]. For SNOW-128 and ZUC-128 algorithms, is there any endorsement or analysis that SA3 could provide as the reference to prove these algorithms can be considered quantum-safe?

[SA3]: A very good summary of the impact of Cryptographically Relevant Quantum Computers (CRQCs) on symmetric cryptography was recently given in a statement by the Internet Engineering Task Force (IETF) [1]. The IETF statement refers to UK NCSC whitepaper [3] that says symmetric algorithms with at least 128-bit keys (such as AES) can continue to be used. SA3 agrees with IETF’s analysis. Most other 128-bit algorithms such as SNOW 3G, ZUC, TUAK, KMAC128, Ascon, etc. are likely to have similar quantum overhead for Grover’s algorithm which is known to be optimal [4]. Even if an algorithm has slightly lower quantum overhead than AES-128, SA3 believes the algorithm would still fulfill the requirement (comparable to key search on AES-128) for quantum resistance category 1 [2].

- Q2: Regarding “3GPP profiles of these protocol”, What exact the 3GPP profiles are? Could SA3 provide some guidance on the 3GPP profiles to prioritize the protocols to be migrated/mandatory.

[SA3]: 3GPP profiles to these protocols refer to specifications on the usage of various options and parameters associated with the protocols according to 3GPP needs — for example, which options and parameters available in these protocols are mandatory to use and which are forbidden. The 3GPP profiles for IKEv2, TLS, DTLS, JWS, X.509, CRL, OCSP, CMP and SUPI encryption can be found in 3GPP TS 33.210 [5], 3GPP TS 33.310 [6], and 3GPP TS 33.501 [7]. 3GPP does not yet have an order of priority for the migration work due to dependency on IETF’s work on introduction of PQC algorithms in protocol specifications. The status of IETF’s work on PQC migration is summarized in [1].

## References:

[1] IETF Statement on Quantum Safe Cryptographic Protocol Inventory
https://datatracker.ietf.org/liaison/1942/

[2] NIST FIPS 203

https://doi.org/10.6028/NIST.FIPS.203

[3] UK NCSC Next steps in preparing for post-quantum cryptography
https://www.ncsc.gov.uk/whitepaper/next-steps-preparing-for-post-quantum-cryptography

[4] M. Boyer, G. Brassard, P. Høyer and A. Tapp, “Tight bounds on quantum searching” Fortschritte Der Physik, Vol. 46, No. 4-5, 1998, pp. 493-505. Also available at https://arxiv.org/abs/quant-ph/9605034

[5] 3GPP TS 33.210 Network Domain Security (NDS); IP network layer security
https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=2279

[6] 3GPP TS 33.310 Network Domain Security (NDS); Authentication Framework (AF)
https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=2293

[7] 3GPP TS 33.501 Security architecture and procedures for 5G System
https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3169

# 2	Actions

To GSMA PQTN

ACTION: 	SA3 kindly asks GSMA to take above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#119	11 - 15 November 2024		Orlando (US)

SA3#120	17 – 21 February 2025		Athens (Greece)