3GPP TSG-SA3 Meeting #120	S3-250356

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Pseudo-CR on Proposed Changes to Conclusions

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:

Comments

The analysis on the solution options for inter-CU LTM has shown that RRC based solution concept should be specified. This contribution is adding related text for the conclusions part of the draftCR (see S3-245164).

* * * First Change * * * *

# 5	Conclusions

## 5.1	Conclusion for key issue #1

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles:

-	For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

-	If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch,

-	it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE.

-	it updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

The security analysis of all featured solutions from the draftCR has shown that the solutions can be divided into four different categories/solution classes. RRC-based, MAC-based, N2-based and PDCP-based approaches.

If the RAN design rules [x1] become applicable, the result is the following:

The N2 based approach must be excluded because of the impact on the CN. The PDCP anchor concept seems favourable at first, but the complexity increases if more and more network elements must be integrated.

The MAC CE based concepts is very closed to the inter-CU LTM but has lack of security.

In Release 19 and for the inter-CU LTM handover the RRC based method should be applied. When applying the design rules, the introduction of a new message should be required.

* * * End of Changes * * * *