3GPP TSG-SA3 Meeting #120	S3-250460

Athens, Greece, 17 - 21 February 2025

Source:	Qualcomm Incorporated

Title:	A new solution for network layer security based on the key provided by AF

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_AIOT_Sec

Comments

This contribution proposes a new solution for a key provisioning mechanism to support network layer security for the scenario where long-term AIoT device credentials being managed by the AF. In this solution, the AF generates a key for the AIoT device and provides it to the AIoTF. The key is used to protect the Inventory and Command procedures.

Proposed Changes

* * * First Change * * * *

## 6.x	Solution #x: A key provisioning for network layer security

### 6.x.1	Introduction

This solution addresses key issues #4 and #5.

This solution provides a key provisioning mechanism to support network layer security for the scenario where long-term AIoT device credentials are managed by the AF. In such scenario, the AF generates a key for the AIoT device and provides it to the AIoTF. Then, Inventory and Command procedure is used to provision the key to the AIoT device. This key is used to protect the Inventory and Command procedures between the AIoTF and AIoT device. The AIoTF can perform the Inventory and Command procedure multiple times using the provided key until a new key is provided.

### 6.x.2	Solution details

Figure 6.x.2.1-1:  A key provisioning mechanism for network layer security

0.	Each AIoT device is provisioned with its AIoT device identifier and a AIoT device credential. An Application Function (AF) manages the AIoT device identifier and the associated AIoT device credential.

The AIoTF is configured with the list of the AIoT device identifiers by the AF as part of AIoT service establishment. AIoTF can also be configured with the AIoT device key (KAIoTF) along with the AIoT device ID. When KAIoTF is provisioned during the service establishment, the key derivation parameter (e.g., freshness parameter) is also sent to the AIoTF, which is then included in the Inventory request message so that the AIoT device can derive the same KAIoTF.

The steps 1a to 1c can be performed whenever AIoTF decides to refresh KAIoTF.

1a.	The AIoTF requests a new KAIoTF to the AF when it does not have a valid key for the AIoT device. The request includes the AIoT device ID and a key derivation parameter.

1b. The AF generates a KAIoTF using the AIoT device credential associated with the AIoT device and the key derivation parameter (e.g., freshness parameter).

1c. The AF provides KAIoTF to the AIoTF.

The steps 2-7 (i.e., inventory and command procedure) is performed to provision the same KAIoTF to the AIoT device.

2.	The AIoTF sends the Inventory Request that contains the key derivation parameter and a freshness parameter1 to Reader(s). The Reader(s) broadcasts the received Inventory Request message.

3.	Upon receipt of the Inventory Request message, the AIoT device generates a KAIoTF based on the AIoT device credential, and the received key derivation parameter.

4.	The AIoT device sends an Inventory Response that contains the AIoT device ID, freshness parameter1 and the freshness parameter2.

NOTE 1: The protection of Inventory Response and/or privacy protection of AIoT device ID is not addressed in this solution.

5.	The AIoTF continues with the Command procedure for key confirmation. The Command request and response messages are protected based on the KAIoTF and freshness parameters.

NOTE 2: The AIoTF can perform the Inventory and Command procedure multiple times using the provided key until a new key is provided.

### 6.x.3	Evaluation

This solution addresses key issues #4 and #5 based on network layer security using the AIoTF specific key provided by AF. This solution assumes that the AIoT device credential is managed by AF and AF provides the AIoTF specific key to AIoTF for network layer security. This solution supports provisioning and refresh of the network layer security key at the AIoT device and the AIoTF.

* * * End of Changes * * * *