3GPP TSG-SA3 Meeting #120	S3-250557

# Athens, Greece,  17 - 21 February 2025

Source:	ZTE

Title:	Resolving ENs in solution #25

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_AIOT_Sec

Comments

Solution #25 has the following Editor’s Notes:

Editor’s Note: whether the acknowledgement message needs to be protected is FFS.

Edtor’s Note:  Further evaluation is FFS.

For the first EN, the acknowledgement message contains the refreshment parameter, which are transferred to update the temporary ID. The security of acknowledgement messages are supposed to be aligned with the security between AIOT devices and core network.

So, we delete this EN and add a note says “The protection of the acknowledgement message aligns with the security between the AIoT device and AIOTF. That is, if there is a secure connection between the AIOT device and the AIOTF, ack messages are also protected. If there is no security between the AIOT device and the AIOTF, ack messages are not encrypted.”

For the second EN, no further evaluation is required, so this EN is deleted.

Proposed Changes

* * * First Change * * * *

6.25	Solution #25: Use temporary identifier to protect the privacy of AIoT device identifiers.

6.25.1	Introduction

This solution addresses key issue #3: Privacy by protecting AIoT device identifiers.

6.25.2	Solution details

Figure 1. AIoT Temporary Identifier configuration procedure.

AIoT is an Ambient IoT Device which has ultra-low complexity power, cost and resource-constrained. The reader refers to the base station or UE based on the connectivity topologies.

- Initial temporary identifier (TempID) and crenditials such as TempID generation key are known by both the AIoTF and the AIoT device through onboarding or registration procedure of AIoT device or other NAS/AS procedure.

1. The network sends AIoT request such as inventory request or command request to AIoT device. The indication is optional for TempID synchronisation.

2. Once triggered by AIoT request, the AIoT device sends information including TempID to the reader. If the indication is included in the request, initial TempID is used by AIoT device in this step.

3. The reader transfer the AIoT information to the core network, i.e., the AIoTF.

4. The AIoTF uses tempID to identify devices and perform operations. For example, the core network verifies the validity of the AIoT device based on the TempID and other subscription data of the device.

If the received tempID does not match any one of the TempIDs stored in the network side, the AIOTF stops the subsequent procedure and re-sends the request in step 1 including an indication. By the indication, the AIoT device replies with initial TempID. Correspondingly, the intial TempID is also used in the network side.

5. The network returns an acknowledgement to the reader. And a freshness parameter is included in the response.

6. The reader transfers the acknowledgement to the AIoT device.

7. Both the AIoT device and the core network function generate a new TempID according to the TempID derivation function for later use. The freshness parameter will be used in both sides.

Note: The protection of the acknowledgement message aligns with the security between the AIoT device and AIOTF. That is, if there is a secure connection between the AIOT device and the AIOTF, acknowledgement messages are also protected. If there is no security between the AIOT device and the AIOTF, acknowledgement messages are not encrypted.

TempID derivation function

When deriving the TempID from TempID generation key, the following parameters shall be used to form the input S to the KDF:

-	FC = 0xxx;

-	P0 = "TempID";

-	L0 = length of "TempID"; (i.e. 0x00 0x06)

-	P1 = device ID;

-	L1 = length of device ID.

-	P2 = freshness parameter;

-	L2 = length of freshness parameter.

The input key KEY (i.e. TempID generation key) is long term key pre-configured both in AIoT device and core network function.

Device ID is the fixed identifier of AIoT device.

6.25.3	Evaluation

In this solution, a temporary identifier (temp ID)is used to addresse key issue #3: Privacy by protecting AIoT device identifiers.

The initial temp ID is pre-confiured in both AIoT device and the network, and the temp ID is updated after each time it is used between the device and network.

The AIoT device needs to store the temp ID

* * * End of Changes * * * *