3GPP TSG-SA3 Meeting #120	S3-250658

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	NEF security requirements

Document for:	Approval

Agenda item:	4.24

Spec:	3GPP TS 33.501

Version:	19.1.0

Work Item:	AIML_CN_SEC

Comments

The proposal is to add clause X.12.3 to address privacy across VFL members during sample alignment for scenario when external AF act as a server.

* * * First Change * * * *

## X.12.3	NEF security requirements

### X.12.3.b VFL privacy during sample alignment (External AF as a Server)

Figure X.12.3.b-1 provide the means to enable privacy protection during sample alignment in case external AF act as VFL Server.

Figure X.12.3.b-1: Privacy of VFL participants during sample alignment (External AF Acts as VFL Server)

1. VFL members perform profile registration with supporting VFL attributes as specified in clause 6.2H.2.1 of TS 23.288[105].

2. VFL Server perform discovery with VFL attributes as specified in 6.2H.2.1 of TS 23.288[105].

NOTE: Handling of temporary NWDAF instance ID (s) is left to NEF implementation.

3. The AF acting as VFL server sends Nnef_VFLTraining_Request towards NEF. The Service request shall contain the Analytics ID, VFL interoperability indicator per analytics ID and sample ID(s).

4. The NEF cache’s the set of sample ID(s) proposed by the AF as VFL server.

5. The NEF send Nnwdaf_VFLTraining_Request to NWDAF (VFL Client) with optionally sample ID(s) and while the token includes the Analytics ID, VFL interoperability indicator per analytics ID and AF ID as described in 6.2H.2.2.2 of TS 23.288[105].

6. The NWDAF (VFL as client) sends a Nnwdaf_VFLTraining_Response to the NEF with list of supporting sample ID(s) for this specific VFL process, as described in 6.2H.2.2.2 of TS 23.288[105].

7a.The NEF perform UE ID mapping in different formats (e.g., SUPI, GPSI) from the NWDAF VFL Client(s) and VFL Server into same format (e.g., SUPI, GPSI) for the sample ID(s) received.

7b.The NEF determines the common sample ID(s) list to be used in the VFL procedure based on received results in step 4 and step 6.

8. The NEF forwards the Nnef_VFLTraining response with common sample ID(s) to the AF acting as the VFL server.

9. The AF acting as VFL server will use the list of common samples ID (s) as the final sample list.

10. VFL members determine the overlapped samples ID(s).

Editor’s Note:	This clause provides description of NEF security requirements, and the related conclusion is made in clause 7.2 and clause 7.3 in TR 33.784.

* * * End of Changes * * * *