3GPP TSG-SA3 Meeting #120	S3-250724

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | draftCR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and clarification of TC_AMF_NAS_INTEGRITY_FAILURE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE), Radix Security |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint. |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current test case specification is technically incorrect (i.e. message being sent by wrong party) and unspecific (i.e. common interpretation of missing NAS-MAC would malform the NAS message or be the same as test case 1). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the general procedure to first establish a valid 5G NAS security context, and then send any message. Adjusted the wording from missing NAS-MAC to missing integrity protection. Adjusted general wording. Added the relevant threat reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The execution of the test case specification by different testers could differ greatly. That could lead to testing of non-security relevant subjects such as NAS parsing, non-comparable test results and potentially vulnerable equipment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

4.2.2.1.4	NAS integrity failure

Requirement Name: NAS integrity failure

Requirement Reference: TS 33.501 [2] clause 6.4.3.3.

Requirement Description: In case of failed integrity check (i.e. faulty NAS-MAC or missing integrity protection) is detected after the start of NAS integrity protection, the concerned message is discarded except for some NAS messages specified in TS 24.501.

Threat Reference: TR 33.926 [6], clause K.2.3.2, NAS integrity selection and use

Test Name: TC_AMF_NAS_INTEGRITY_FAILURE

Purpose:

Verify that AMF under test drops messages in case the NAS integrity fails or is missing.

Pre-Conditions:

-	Test environment with UE. The UE may be simulated.

-	AMF under test is connected in emulated/real network environment.

-	NAS Integrity algorithm different than NIA0 is used.

Execution Steps

Test case 1 (wrong NAS-MAC):

1.	The tester triggers the UE to initiate and complete an initial registration procedure with the AMF, which establishes a valid 5G NAS security context with a selected integrity protection algorithm other than NIA0.

2.	The tester sends any UE originating message to the AMF, with a wrong NAS-MAC value. The message used is not listed in the exceptions in TS 24.501 [5] and is a valid message given the communication state (e.g. NAS PDU Session Establishment Request message).

Test case 2 (missing integrity protection):

1.	The tester triggers the UE to initiate and complete an initial registration procedure with the AMF, which establishes a valid 5G NAS security context with a selected integrity protection algorithm other than NIA0.

2.	The tester sends any UE originating message to the AMF, with missing integrity protection (i.e. by encoding it as a Plain 5GS NAS message/NAS message with a security header consisting only of zeros). The message used is not listed in the exceptions in TS 24.501 [5] and is a valid message given the communication state (e.g. NAS PDU Session Establishment Request message).

Expected Results:

In both test cases, the AMF discards the NAS messages. This can be verified by awaiting the message specific response timeout (e.g. T3580 for the NAS PDU Session Establishment Request message as defined in TS 24.501 [5]).

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

********** END OF CHANGE **********