3GPP TSG-SA3 Meeting #120	S3-250876

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Security aspects on store and forward operation with full EPC in each satellite

Document for:	Approval

Agenda item:	5.7

Spec:	3GPP TS 33.401

Version:	18.2.0

Work Item:	FS_5GSAT_Ph3_SEC

Comments

Based on conclusions agreed in the TR 33.700-29 for KI#1, this proposal proposes the security principles for store and forward operation with full EPC in each satellite.

* * * First Change * * * *

Annex X ( informative):
Security for Store and Forward Satellite operation

X.1	General

Editor’s Note: This clause provides general description for Annex X.

X.3	Security aspects on Full EPC in each satellite

In S&F operations involving full EPC in each satellite, authentication and communication protection between the UE and the satellite network reuse security procedures as specified in the present document, including AKA and NAS SMC. The security credentials required for these procedures are stored in the HSS onboard the satellite.

NOTE 1: The security of communications between the proxies on satellite and ground is out of 3GPP scope. Those proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc.

NOTE 2: As an option for the operator, the IOPS-based keying as specified in Annex F can be used to limit further the impact of exposing the long-term key in the HSS in the satellite.

* * * End of Changes * * * *