3GPP TSG-SA3 Meeting #120	S3-250891

# Athens, Greece, 17 - 21 February 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for KI#1

Document for:	Approval

Agenda item:	4.21

Spec:	S3-250779 (DraftCR to TS 33.434)

Version:	18.2.0

Work Item:	Metaverse_Sec

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

This contribution proposes solutions for KI#1.

According to TS 33.434, when CAPIF is used as specified in TS 23.434 [2], the security mechanism for CAPIF specified in TS 33.122 [29] is referred to protect VAL server - SEAL server interface (which is mapped to CAPIF-2/2e) interface between API invoker and AEF). However, how to protect SEAL client – SEAL server interface in CAPIF deployment is not described in TS 33.434. In addition, three methods are specified in clause 6.5.2 of TS 33.122 for CAPIF-2e interface authentication and protection while Authentication and authorization for RNAA specified in clause 6.5.3 of TS 33.122 uses token-based authorization. This contribution proposes following security mechanism for CAPIF to protect spatial anchor/map client - spatial anchor/map server interface, as well as between VAL server - spatial anchor/map server interface. To protect the privacy of spatial anchor/map owner, authentication and authorization for RNAA specified in clause 6.5.3 of TS 33.122 is suggested to protect the interfaces.

When CAPIF is not used, SEAL service authorization procedure to validate the VAL user to access the SEAL services as specified in the present document shall be followed.

Proposed Changes

* * * First Change * * * *

<Proposed change in revision marks>

## 6.X	Authentication and authorization for spatial localization services

Editor’s Note: This clause will be updated to give reference to the procedure for authentication and authorization for spatial localization services.

## 6.x.a Authentication and authorization for spatial localization services when CAPIF is used

When CAPIF is used as specified in TS 23.434 [2], the security mechanism for CAPIF specified in TS 33.122 [29] is referred to protect interfaces between spatial anchor client and spatial anchor server specified in TS 23.437 [x], between VAL server and spatial anchor server, between spatial map client and spatial map server specified in TS 23.437 [x], VAL server and spatial map server. Authentication and authorization for RNAA specified in clause 6.5.3 of TS 33.122 [29] shall be followed to protect the interfaces. Spatial anchor client, spatial map client and VAL server take role of API Invoker in CAPIF. Spatial anchor server and spatial map server take role of AEF in CAPIF.

Editor’s Note: Authorization on service operation level and resource level granularity is FFS.

## 6.x.b Authentication and authorization for spatial localization services when CAPIF is not used

When CAPIF is not used, SEAL service authorization procedure to validate the VAL user to access the SEAL services as specified in the present document shall be followed.

Editor’s Note: The concrete clauses of TS 33.434 will be added.

Editor’s Note: The mapping between entities in SEAL security procedure and entities in spatial anchor/map management procedure will be added.

Editor’s Note: Whether user authentication is needed is FFS.

* * * End of Changes * * * *