3GPP TSG-SA3 Meeting #120	S3-250947

Athens, Greece, 17 - 21 February 2025	(revision of xx-yyxxxx)

# Source:	Ericsson

# Title:	New WID on Protection of XRM Media related information

# Document for:	Agreement

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on Protection of XRM Media related information

Acronym: XRM_Ph2-Sec

Unique identifier:	 TDB

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| XRM_Ph2 | SA WG2 | 1040032 | Extended Reality and Media service (XRM) Phase 2 |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: draft-ietf-masque-quic-proxy-04 - QUIC-Aware Proxying Using HTTP

# 3	Justification

The topic of Extended Reality and Media service (XRM) has been studied by SA2 in TR 23.700-70 and a WID is approved in SP-241385. Normative work is captured in clause 5.37.9 of TS 23.501.

KI#2 of 23.700-70 is about the UPF being able to identify certain XRM Metadata sent from an external Application Server (AS) to the UE together with end-to-end (e2e) encrypted XRM data streams. It was concluded (clause 8.2. of TR 23.700-70) that the XRM metadata shall be encrypted and integrity protected between the UPF and the AS.

It was further concluded by SA2 to support multiple methods for this in normative work:

- Using Media over QUIC (MoQ)

- Using Proxy-UDP-in-HTTP/3 together with QUIC-Aware Proxying,

- Using UDP options

The conclusions of the TR are captured in the WID (SP241385) as WT#1.2:

WT#1.2

KI#2 Enhancement to support QoS control and PDU Set identification for XR traffic stream with e2e encryption.

- Media over QUIC (MoQ), Proxy-UDP-in-HTTP/3+QUIC-Aware Proxying and UDP Options should be supported for XRM metadata delivery for XR traffic stream with e2e encryption.

The requirement (as stated in the conclusion of TR 23.700-70) that XRM metadata shall be encrypted and integrity protected implies that further work is needed by SA3 to specify the details of the security solutions for the three selected methods above.

Note that what the term ‘XRM Metadata’ was changed in normative work to ‘media related information’.

# 4	Objective

This work aims to specify required enhancements on 5GS to support protection of XRM metadata sent between an external Application Server and the UPF.

The following work-items are included based on the conclusions in the TR 23.700-70:

- WT#1 Media over QUIC (MoQ): Specify security details for the case when MoQ is used to transfer the media related information.

-WT#2 Proxy-UDP-in-HTTP/3 + QUIC-Aware Proxying: Specify the security details for the case when QUIC-aware-proxying is used to transfer the media related information.

-WT#3 UDP-option: Specify the security details for the case when UDP-options are used to transfer the media related information.

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | Dependencies |
| --- | --- | --- | --- |
| WT#1 | 0 | <<0.25 | WT is self-contained |
| WT#2 | 0 | <<0.25 | WT is self-contained |
| WT#3 | 0 | <<0.25 | WT is self-contained |

The total sum of TU is expected to be low and handled in one meeting.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501 | New Clause for XRM | TSG#107 (March 2025) |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

Stage 3 aspects covered by CT WGs.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

Lenovo

Nokia

Nokia Shanghai Bell