3GPP TSG-SA3 Meeting #120	S3-250832

Athens, Greece, 17 - 21 February 2025	(revision of xx-yyxxxx)

# Source:	AT&T, Charter Communications, Deutsche Telekom, KDDI, NTT DOCOMO, Telecom Italia, Vodafone

# Title:	New WID on Security Monitoring

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Security Monitoring

Acronym: SECMON

Unique identifier:

TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

# 3	Justification

The Service Based Architecture (SBA) is the dominant method for both control plane and management communications. In additions to the many benefits to using SBA, e.g. agility to increase and decrease the number of service instances in coordination with demand, comes an expanded attack surface. However, visibility in the form of continuous monitoring is missing.

In TS 28.533, section 4.6, it states "The management data analytics utilize the network management data collected from the network (including e.g. service, slicing and/or network functions related data) and make the corresponding analytics based on the collected information. For example, the information provided by PM data analytics services can be used to optimize network performance, and the information provided by FM data analytics services can be used to predict and prevent failures of the network. Security benefits like more efficient security detection and higher protection can also be achieved in a similar way by utilizing security data collected from the network.

Generally, traditional security requirements include among others authentication, confidentiality and integrity protection, authorization, and security assurance aspects. While those current security requirements are still valid and practical today, they might not be sufficient to address more sophisticated risks like those derived from emerging Advanced Persistent Threats (APTs). More visibility into the SBA is required to detect them.

Service-based architecture enhances flexibility and scalability of 5G network, although it also introduces additional threats. Although there are TLS and OAuth security features to enhance the protection of SBA interfaces, the architecture still faces the problem that a breach of the network can be used to attack other NFs. The network lacks a security detection functionality into NFs, and the traffic analysis alone is not efficient enough to detect compromised NFs. Security operations centers require more “security” visibility into NFs.

This argument also applies to east-west traffic as well as network management operations.

In the SBA there are gaps in the current security requirements that make detection of threats difficult.

The security incidents or scenarios in the network where data can be collected include:

1)	authentication and authorization failure event;

2)	unexpected setup of TLS session and API invocation related to unauthorized reconnaissance;

3)	malformed message event;

4)	high service load;

5)	unexpected SBI call flows; and

6) 	unexpected use of APIs exposed by services in SBA layer

4	Objectives

The objective of this WID is to specify data collection requirements and solution for continuous security monitoring, that can be used by an operator security function.  Following objectives are proposed:

WT1:

-	Specification of the general requirements for security events handling and collection.

-	Specification of the security requirements to transfer or communicate security events.

WT2:

This work task will specify the events that need to be reported as part of continuous security monitoring.

Examples for security relevant events data for monitoring:

- 	The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

- 	The NF collects information about events involving receiving a massive number of incoming messages on the SBA layer.

- 	The NF collects information about failed authentication and authorization attempts from inbound connections on the SBA layer.

- 	The NF collects information about potential replay attacks on the SBA layer.

- 	The NF collects information about potential abnormal SBI call flows as defined for the communication models in Annex E of TS 23.501.

This work task will also specify the collection and reporting of sensitive network management operations at SBA or platform level like management login/logoff events, configuration modification, etc.

For each of the events, it will be specified which set of data (e.g., information elements as applicable) needs to be reported.

NOTE 1: The format of this reporting will be selected by SA5. Coordination with SA5 will be required.

NOTE 2: The method for collecting this data in the NF is up to implementation.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- |
| WT1: Security reporting requirements |  | 0.25 | No |
| WT2: Security events definition |  | 3.75 | No |

Total TU estimates for the study phase:   N/A

Total TU estimates for the normative phase:   4

Total TU estimates: 4

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TS | 33.abc | Security monitoring | SA#108 (June 2025) | SA#109 (September 2025) |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBA

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

Event reporting mechanism to be defined by SA5

# 9	Supporting Individual Members

Supporting IM name

AT&T

Charter Communications

Deutsche Telekom

KDDI

NTT Docomo

Telecom Italia

T-Mobile USA?

Verizon?

Vodafone