3GPP TSG-SA3 Meeting #120	S3-250837

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi Technology Spain S.L

Title:	Pseudo-CR on updates to conclusion for KI#1 in TR 33.743

Document for:	Approval

Agenda item:	5.12

Spec:	3GPP TR 33.743

Version:	1.1.0

Work Item:	FS_5G_ProSe_Ph3_SEC

Comments

With the SA2 response in LS S2-2501335, the assumption that the Intermediate Relays and the UE-to-Network Relay belong to the same PLMN is not valid. If the Intermediate Relays and the UE-to-Network Relay can belong to different PLMNs, the Remote UE and Intermediate Relay will obtain multiple sets of discovery security materials. Therefore, it is proposed to update the conclusion for multi-hop UE-to-Network relay discovery.

Editor’s Note: How the PC5 link establishment is performed based on the optional procedure clause 6.3.2.5.2 of TS 23.304 [4]) is FFS.

As specified in TS 23.304,

RSC based configuration for the 5G ProSe Intermediate UE-to-Network Relay indicates whether it should establish the Layer-2 Link with a parent relay before it can participate in the discover operation.

Whether the PC5 link establishment procedure is performed or not is decided by the UE by using the RSC based configuration. If the PC5 link establishment procedure should be performed before it can participate in the discovery procedure, the existing CP/UP procedure defined in clause 6.3.3 of TS 33.503 is reused. Otherwise, the Intermediate Relay broadcasts the updated Announcement message without performing the link establishment procedure.

Therefore, it is proposed to remove this Editor’s Note.

Editor’s Note: Further work on authentication and authorization between the 5G ProSe Remote UE and potential 5G ProSe UE-to-Network Relay via the network is FFS.

For the multi-hop UE-to-Network Relay communication, the authentication and authorization between the 5G ProSe Remote UE and 5G ProSe UE-to-Network Relay are achieved during the link establishment for each hop, which follows the principle used in UE-to-UE Relay communication.

Therefore, it is proposed to remove this Editor’s Note.

* * * First Change * * * *

7.2	Key issue #1: Security for multi-hop UE-to-Network Relay

The following statements are agreed as a basis for normative work:

For 5G ProSe Multi-hop UE-to-Network Relay discovery:

- The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay’s HPLMN respectively, based on the procedure specified in clause 6.1.3.2 of TS 33.503 [5]. The Intermediate UE-to-Network Relay is provisioned with multiple sets of discovery security materials and the associated HPLMN IDs of the potential Intermediate UE-to-Network Relays/5G ProSe UE-to-Network Relay.

- The discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of discovery messages.

- The Intermediate UE-to-Network Relay processes the received discovery message using the provisioned discovery security materials associated with the RSC as specified in clause 6.1.3.2 of TS 33.503[5]. If the processing is successful, the Intermediate UE-to-Network Relay updates the path information (e.g., hop count, Relay Info.) and protects the updated message using the security materials associated with the RSC as specified in clause 6.1.3.2 of TS 33.503 [5].

- Both discovery with Model A and discovery with Model B are supported.

- For discovery with Model A, the Intermediate UE-to-Network Relay may (as specified in step 6 in clause 6.3.2.5.2 of TS 23.304 [4]) protect and send the Announcement message after a secure PC5 link establishment with the upstream Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay.

- For discovery with Model B, the 5G ProSe Remote UE protects the discovery message (i.e., Relay Discovery Solicitation) reusing the procedures as specified in clause 6.1.3.2.3 of TS 33.503[5], i.e., using the discovery security materials associated with the RSC and HPLMN ID of the potential Intermediate UE-to-Network Relay.

For Key issue #1 on the support Security for multi-hop UE-to-Network Relay communication, the following principles apply:

1)	For UE-to-Network Relay communication security following Model A discovery:

- The security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay is based on CP-based/UP-based procedures as specified in clause 6.3.3 of TS 33.503 [5].

2)	For UE-to-Network Relay communication security following Model B discovery, the existing CP-based/UP-based procedures as specified in the in clauses 6.3.3 of TS 33.503 [5] are used as baseline for the normative work with the following principles:

- To trigger the multi-hop U2NW Relay link security establishment, the Remote UE sends a DCR message as specified in the in clauses 6.3.3 of TS 33.503 [5] to the Intermeditate UE-to-Network Relay.

- The Intermediate UE-to-Network Relay plays the role of the Remote UE to establish secured link with the UE-to-Network Relay or with the next Intermediate UE-to-Network Relay, reusing the procedure as specified in the in clauses 6.3.3 of TS 33.503 [5], if the Intermediate UE-to-Network Relay does not have an existing PC5 link with the UE-to-Network Relay or the next Intermediate UE-to-Network Relay, and the next Intermediate UE-to-Network Relay is in-coverage.

NOTE 1: The next Intermediate UE-to-Network Relay are considered in-coverage if they have connection to the network.

NOTE 2: 	Whether a separate RSC for multi-hop or the same RSC as for single hop is provisioned is to be determined during normative phase in coordination with SA2.

3)	For authentication and authorization, the existing CP-based/UP-based procedures as specified in clause 6.3.3 of TS 33.503 [5] can be used as baseline for each hop during the normative work.

4)	For Remote UE report procedure, the existing procedures for Remote UE Report as specified in clause 6.3.3 of TS 33.503 [5] can be used as baseline for the normative work.

Editor’s Note: Further work on Remote UE report procedure is FFS.

NOTE 3: How the Remote User ID of the Remote UE is obtained by the 5G ProSe UE-to-Network Relay for the Remote UE Report procedure is to be determined during normative phase in coordination with SA2.

NOTE 4: For the security procedure over User Plane, it is assumed that an Intermediate UE-to-Network Relay is able to access to the 5G PKMF in its HPLMN (i.e., PC8 establishment via Intermediate UE-to-Network Relay(s) or UE-to-Network Relay).

* * * End of Changes * * * *