3GPP TSG-SA3 Meeting #120	S3-251013

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon, Nokia

Title:	Resolve EN’s to KI#2 conclusion

Document for:	Approval

Agenda item:	5.16

Spec:	3GPP TR 33.766

Version:	0.6.0

Work Item:	FS_Energy_Sec

Comments

This contribution resolves the ENs for KI#2 conclusion. Without granular authorization, the AF may gain access to more data than necessary, violating principles of least privilege and increasing the risk of misuse or accidental exposure or lead to UE’s profiling or unauthorized analytics using UE’s energy related data. Thus, fine grain authorization mitigates this by enforcing strict boundaries for what level of UE’s energy related information can be accessed and which type of energy related information is made available to the AFs based on its subscription.

This is a merger of S3-250664 and S3-250823.

* * * First Change * * * *

## 7.2	Conclusion for KI#2: Security aspects of exposure of energy related information.

Based on solution 2 and 3, the following conclusion is drawn. The exposure of energy related information to 3’rd party AFs happens through the NEF, this implies that the following principles are reused without need for normative work. For authentication aspects TS 33.501 [3] clause 12.2 applies, for protection of the interface between the NEF and AF TS 33.501 [3] clause 12.3 applies and for authorisation of the AF TS 33.501 [3] clause 12.4 applies. The corresponding AF authorisation granularities for energy data related exposure can reuse the principles already defined in TS 33.501 [3]. Authorisation with further granularity (e.g., UE, PDU Session and/or QoS flow) may be needed to avoid  the risk of unauthorized service access that may violating principles of least privilege that may result in AF gaining access to more data than necessary. By incorporating these granularities, the NEF ensures that external AF access is policed. The definition of token claims and corresponding authorisation granularities is left for implementation. Thus, no normative work is needed.

* * * End of Changes * * * *