# 3GPP TSG-SA3 Meeting #125																			S3-254577

# Dallas, US, 17 – 21 November 2025

Source:	Huawei, HiSilicon

Title:	Solution to key issue#2

Document for:	Approval

Agenda item:	5.2.6

Spec:	3GPP TR 33.785

Version:	0.2.0

Work Item:	Study on Security for Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2

Comments

As per KI#1 in TR 23.700-04, which aims to investigate and develop architectural enhancements to support UE data collection to meet the requirements for AI/ML for NR air interface with UE-side model training. This proposal bring solution to address CN-OTT interaction, more specifically with Data collection function (Naming and role of data collection function is TBD and subject to progress of TR 23.700-04 [2]). Aspects covered as part of the solution are CN-OTT Authentication, Authorization and the Communication security. As outlined in the high-level principle of TR 23.700-04 clause 7.1.1 UP path is used between the UE and a 5GC Data collection Function, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

* * * First Change * * * *

## 6.Y	Security, and Authorization for Exposure of UE Data towards OTT Servers

### 6.Y.1	Introduction

This clause outlines the security considerations for AF authentication, communication security between NEF and AF and authorization for data collection procedure between both AFs and DCF utilizing NRF token-based approach. General mechanism to utilise the token-based authorization is specified in clause 13.4.1 of TS 33.501 [x].

As specified in clause 12.2 of TS 33.501 for authentication between NEF and an AF that resides outside the 3GPP operator domain, mutual authentication based on client and server certificates shall be performed between the NEF and AF using TLS.

As specified in clause 12.3 of TS 33.501 TLS shall be used to provide integrity protection, replay protection and confidentiality protection for the interface between the NEF and the AF to secure the communication data.

### 6.Y.2	Solution details

Editor's note:	Whether the DCF is a new NF or the existing one is FFS and is depending on SA2 progress.

Figure 6.Y.2.x, Authorization for Exposure of UE Data towards OTT Servers

1a.	In case of untrusted AF OTT server, the NEF registers its NF profile at the NRF with AF’s related parameters that includes AF ID, data context ID(s), and Filter info (e.g. AoI(s)).

1b.	DCF register its NF profile at the NRF that consists of allowed list which contains AF ID, data context ID(s), Filter info (e.g. AoI(s)) that indicate whether the specific AF is allowed to access specific set of UE.

Editor’s Note: What constitutes the allowed list during the DCF profile registration how this is implemented is FFS.

2. The OTT server sends Service Request towards NEF. The Service request shall contain the AF ID, and data context ID(s), Filter info (e.g. AoI(s)).

3. The NEF requests an access token from the NRF. The access token request shall contain AF ID, data context ID(s), Filter info (e.g. AoI(s)).

Editor’s Note: Which input parameters are mandatory is FFS

4. The NRF checks whether NEF on behalf of OTT Server is authorized to access DCF services by comparing the allowed list parameters that contains AF ID, data context ID(s) and Filter info (e.g. AoI(s)). received in step 1b and one received in step 3.

5. If the NEF is authorized, the NRF will issue an access token(s) in response. The token claim shall include parameters as part of the allowed list i.e AF ID, data context ID(s) and Filter info (e.g. AoI(s)).

Editor’s Note: What constitute the access token claims is FFS.

6. The NEF send service request with parameters as received in step 2 and the access token claims received in step 5 to DCF.

7. The DCF verifies the received access token as specified in clause 13.4.1 and checks for the token claims. The DCF extracts the allowed list parameters i.e AF ID, and data context ID(s) and Filter info (e.g. AoI(s)). and check whether it matches the one received in step 6.

8. In case of successful access token verification, the DCF returns the requested data collected to the NEF and that is subsequently returned to the OTT server.

Editor’s Note: Utilizing specific authorization parameters and overall solution alignment, e.g. data collection procedure is subject to SA2 progress.

Editor’s Note: NRF based OTT server authorization is FFS.

### 6.Y.3	Evaluation

TBD

* * * End of Change * * * *