# 3GPP TSG-SA3 Meeting #125																				S3-254583

# Dallas, US, 17 – 21 November 2025 																 was S3‑254377

Source:	Xiaomi

Title:	Pseudo-CR on Update TR 33.785 sol#5

Document for:	Approval

Agenda item:	5.2.6

Spec:	3GPP TR 33.785

Version:	0.2.0

Work Item:	FS_AIML_CN_Ph2_SEC

Comments

This contribution proposes to update TR 33.785 sol#5.

This solution only addresses the secure communication requirement and mutual authentication between UE and the data collection function requirement of KI#1, thus the following EN is automatically resolved.

Editor's note: How the solution covers all the requirements of KI#1 is FFS.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-04: "Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)".

[3]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[4]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[5]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[X]				IETF RFC 4279: "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[Y]				IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

* * * Second Change * * * *

6.6	Solution #5: Secure communication between UE and the data collection function

6.6.1	Introduction

This solution addresses part of KI#1, i.e. secure communication and mutual authentication between UE and the data collection function.

Secure connection is required between the UE and the data collection function, the connection between the UE and the data collection function can be secured by the TLS or NDS/IP and UP security in this solution.

The generation of shared key depends on other solutions in this document.

According to RFC 4279 [X], to enable data collection function to identify the shared key, UE needs to send the PSK identity to the data collection function via ClientKeyExchange message.

According to RFC 8446 [Y], to enable the data collection function to identify the shared key, UE needs to send the PSK identity to the data collection function via pre_shared_key extension.

In this solution, the PSK identity is set as SUCI to enable the data collection function to identify the correct shared key.

6.6.2	Solution details

For connection between UE and the data collection function located in home network, the TLS connection between the UE and the data collection function can be used for protecting the UE data. The TLS can be established based on key shared between the UE and the data collection function.

The shared key is generated based on mechanisms defined in other solutions in this document (e.g., data collection function generates the shared key, the AKMA based mechanism).

Since the PSK identity is delivered in an insecure environment (i.e., the TLS tunnel between UE and the data collection function is not established yet), the PSK identity is set as the SUCI of the UE when the AKMA/GBA based mechanism is not used.

After receiving the PSK identity (i.e., the SUCI), the data collection function interacts with the UDM/UDR/ADRF to retrieve the SUPI and uses the SUPI to identify the shared key.

Editor’s Note: Whether TLS implementations support exporting PSK identity for retrieving PSK is FFS

Editor's note: How the shared key is generated is FFS.

Editor’s Note: how SUCI and other user identifiers are used to identify PSK and how and if those are utilized for key derivation are FFS.

For connection between UE and the data collection function located in home network, the connection between the UE and base station, the connection between base station and the UPF, the connection between the UPF and the data collection function will be secured. Then the following mechanisms can be reused.

-	The connection between UE and base station can be secured based on user plane related security algorithms defined in clause 6.6 of TS 33.501[3].

-	The connection between base station and UPF can be secured based on existing NDS/IP as specified in clause 9.3 of TS 33.501 [3].

-	The connection between UPF and the data collection function can be secured based on existing NDS/IP as specified in clause 9.3 of TS 33.501 [3].

6.6.3	Evaluation

Editor's note: Further evaluation is needed.

Editor's note: How the UE perform data collection and its dependency on the solution is subject to SA2 progress.

* * * End of Changes * * * *