3GPP TSG-SA WG3 Meeting #55	S3-09xxxx

- Shanghai, China, 11-15 May 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.223 | CR |  |  rev | - |  Current version: | 8.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Editorial corrections on 33.223 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | GBA Push |  |  |  |  |  |  | Date:  |  | 12/5/2009 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | The parameter name App_Lbl of UICC application identifier in GPI information is inconsisitent with the parameter name Appl_Lbl in NAF SA definition. the description in NAF GPI request. It would be confused. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Correction on the parameter name Appl_Lbl in NAF SA definition |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The specification would be inconsistent and confused on the parameter name of UICC application identifier. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

**** Start of 1st change *****

### 5.1.4	UE processing of GPI

When the UE receives a GPI it performs the following steps.

1.	UE receives GPI. The parameters of the GPI are defined in clause 5.2.1

2.	If the App_Lbl in the GPI is undefined, the UE determines the UICC application to use from used delivery channel of the GPI (e.g. SMS, MMS, SIP Message, etc) or from other context information.

3.	UE checks if it has received the same GPI earlier.

a.	If the GPI corresponds to an already existing NAF SA, then the GPI is silently dropped and the GPI processing terminated.

b.	If the GPI corresponds to an incomplete NAF SA, the Ks indicated by GPI is activated and processing continues from step 6 (step 7 describes how an incomplete SA may appear).

NOTE 1:	To handle retransmissions efficiently the UE should only invoke a UICC application after checking that the GPI does not correspond to an already existing NAF SA. The check can be done by comparing the received (RAND, AUTN(*),App_Lbl) triplet with the corresponding triplets associated with existing NAF SAs.

**** End of 1st change *****

**** Start of 2nd change *****

### 5.2.3	NAF SA

The NAF needs to keep some additional information in its NAF SA compared with the UE. The UE identity used in the BSF request for GPI must be stored to allow the NAF to determine from which UE a response is coming and also to link sequences of SA's for the same UE. The NAF also needs to store the transport address to which the GPI should be directed. If the NAF uses retransmission to achieve better delivery reliability, it also needs to store the encrypted version of the part of the GPI, which is confidentiality protected. It also has to store the GPI MAC.

Table 5.2.3-1: NAF SA definition

| Parameter name | NAF | UE | Description | Notes |
| --- | --- | --- | --- | --- |
| UE_ Id | m | o | The user identity used in NAF request. |  |
| UE_Priv_Id | o | - | Private user identity (IMSI/IMPI) for used UE_Id |  |
| UE_Trp | m | - | Transport address to which GPI should be delivered | The transport address used by the NAF when pushing GPI to the UE |
| RAND | m | m | RAND in UMTS AKA | From GPI |
| AUTN(*) | m | m | AUTN or AUTN* | From GPI |
| App_Lbl | m | m | UICC application identifier | From GPI or other implicit agreement or information. |
| NAF_Id | m | m | Concatenation of NAF FQDN and Ua security protocol Id |  |
| Enc_GPI | m | - | Encrypted part of GPI plus MAC |  |
| Mac_GPI | m | - | BSF generated MAC over GPI |  |
| UL_SA_Id | m | m | Uplink NAF SA identity |  |
| DL_SA_Id | m | m | Downlink NAF SA identity |  |
| Ks_NAF / Ks_ext_NAF | m | m | External NAF-key | Ks_NAF is generated in GBA_ME based GBA-Push Ks_ext_NAF is generated in GBA_U based GBA_Push |
| Ks_int_NAF | o | o | UICC internal NAF-key | Ks_int_NAF is generated in GBA_U based GBA_Push |
| Key_LT | m | m | Requested NAF-Key life time |  |
| Repl_Cnt | o | o | Replay counter for outbound messages: First outbound message gets value 1 | Outbound; Defined when Ua protocol is GPL |
| Repl_Win | o | o | Replay window for incoming messages: Initially the window is empty with highest received replay counter =0 | Inbound; Defined when Ua protocol is GPL |

**** End of 2nd change *****