- 3GPP TSG-SA3 (Security)	S3-100189

- SA3#58, 1-5 Feb 2010, Xi’an, China	revision of S3-10xyzw

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.310 | CR | CRNum |  rev | - |  Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Alignment of TLS profile in NDS with TR-069 profile in H(e)NB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | BT |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | NDS_Backhaul |  |  |  |  |  |  | Date:  |  | 19/01/2010 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | 9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | SA3 has decided to use to TLS to secure the management protocol in the context of H(e)NB and has agreed that only certain versions of TLS and cipher suites shall be supported to ensure a minimum level of security. The same minimum standards should apply to the use of TLS in NDS ie when securinfg signaling and management traffiic for NE,s in general |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Added relevant statements from TS33.320 clause 8.3.4 TR-069 protocol profile to TLS protocol profile in Annex E TS33.310 |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | - Inconsistent levels of securtity within an operators network - Vendors will have to implement multiple TLS profiles in SEG’s etc which may conflict. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 5.2.2.1 ( now renumbered correctly as 5.2.2.2), Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | "The RSA key length shall be at least 1024-bit" in TS33.310. Should we be making this 2048- bit minimum for new uses of NDS? I note that we are silent on this in the H(e)NB spec TS 33.320 Minimum of TLS1.1 in TS33.320 and in this CR, but TS33.310 allows TLS 1.0 do we want to allow 1.0? |  |  |  |  |  |  |  |  |

************************** start of changes ************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 33.210: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Network domain security; IP network layer security".

[2]	IETF RFC 2986: "PKCS#10 Certification Request Syntax Specification Version 1.7".

[3]	Void.

[4]	IETF RFC 4210: "Internet X.509 Public Key Infrastructure Certificate Management Protocol".

[5]	IETF RFC 2252: "Lightweight Directory Access Protocol (v3): Attribute Syntax Definitions".

[6]	IETF RFC 1981: "Path MTU Discovery for IP version 6".

[7]	"PKI basics – A Technical Perspective", November 2002, http://www.pkiforum.org/pdfs/PKI_Basics-A_technical_perspective.pdf

[8]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[9]	3GPP TS 33.203: "Access security for IP-based services".

[10]	3GPP TS 33.220: "Generic Authentication Architecture: Generic Bootstrapping Architecture".

[11]	IETF RFC 2246: "The TLS Protocol Version 1.0".

[12]	IETF RFC 2560 (1999): "X.509 Internet Public Key Infrastructure Online Certificate Status Protocol - OCSP".

[13]	Void.

[14]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[15]	IETF RFC 4945: "The Internet IP Security PKI Profile of IKEv1/ISAKMP, IKEv2, and PKIX".

[16]				IETF RFC 5246:  “The Transport Layer Security (TLS) Protocol Version 1.2”

************************** start of next change *********************

#### 5.2.2.2	TLS case

After establishing a interconnect agreement and finishing the required preliminary certificate management operations as specified in clause 5.2.1, the operators configure their TLS entities for secure interconnection. The exact process for establishing the TLS connections is dependent on the application protocol and is outside the scope of this specification. However, the general flow is described in the remainder of this clause.

The local Interconnection CA and TLS client/server CAs are configured as trusted CAs in the TLS entity typically by storing them in a file of trusted CAs on the TLS entity. The cross-certificates on the TLS client/server CAs of the remote operator are also made available to the TLS entity, e.g. by storing them in a file of trusted CAs on the TLS entity, or by storing them in a local CR (Certificate Repository) which all TLS entities that need to communicate with the other domain shall access e.g. using LDAP. Because of the cross-certification, any operator whose TLS client CA or TLS server CA has been cross-certified by another operator can establish TLS connections with that other operator.

The following is the connection establishment from the point of view of a TLS client in Operator A (TLSa) and a TLS server in Operator B (TLSb). The case where the TLS client is in Operator B and the TLS server is in Operator A is treated in a similar fashion. The flow is based on the TLS handshake protocol as described in RFC 2246 [11]

The flow for RFC 5246:  “The Transport Layer Security (TLS) Protocol Version 1.2” [16] is assumed to be similar.

************************** start of next change *********************

Annex E (normative):
TLS protocol profile

TLS end points shall support TLS with the following resrictions and extenstions, except those TLS endpoints that have been defined for GBA in TS 33.220 [10] and for IMS in TS 33.203 [9]. Both TS 33.203 [9] and TS 33.220 [10] contain their own TLS CipherSuite requirements.

- SSL 3.0 shall not be used as it is outdated.

- At least TLS 1.1 [11] shall be supported. TLS 1.2 [16] should be supported.

- The support of TLS cipher suite RSA_WITH_RC4_128_SHA shall not be mandatory.

- The support of TLS cipher suite RSA_WITH_AES_128_CBC_SHA shall be mandatory.

- Only TLS cipher suites listed in TLS 1.2 [16] shall be used. Ciphersuites with RC4 should not be used.

NOTE: 	This specification contains TLS profile requirements when both TLS endpoints belong to an NE. However if a new TLS based UE to NE profile needs to defined, then this can be based on the existence of the above ciphersuites, if this NE needs to implement a TLS profile for NE to NE communication

*****************End of changes*************************