3GPP TSG SA WG3 (Security) Meeting #70 	S3-130035

21-25 January 2012; Sophia Antipolis, France

Source:	Alcatel-Lucent, China Mobile

Title:	SECAM - Section 4.3.1: Addition of text for Threats

Document for:	Discussion and Approval

Agenda Item:	8.3 Study Item on Security Assurance Methodology for 3GPP Network Products

Work Item / Release:	FS_SECAM / R12

Abstract of the contribution:

|  | Addition of text which describes threat categories to be considered for 3GPP nodes.. |
| --- | --- |

4.3.1 Threats based on ITU-T X.800

The threats against 3GPP Network elements or an individual 3GPP functional entity or a group of 3GPP functional entity can be categorized under the headings listed below. Examples for each threat category are listed and in some cases more detailed specific threats are included. The examples listed are representative of the threat category but not an exhaustive list.

-1. Destruction:

- Destruction of information &/or other resources

- Examples:	 Destruction of isolated base stations or devices

Destruction of configuration data/billing data/service data/user data

This includes deliberate acts of destruction due to vandalism, attempts to deny service to an area or acts of terrorism. Accidental destruction can occur during environmental incidents such as storms, flooding etc or during maintenance or construction projects. Destruction of configuration data/billing data/service data/user data may occur accidentally during OAM activities or deliberately as a result of an insider attack.

2. Corruption:

- Tampering with an asset

- Examples: 	Changing configuration information

Configuration settings that control or impact the service of a user or of a node may be changed accidentally or deliberately potentially impacting a user’s service or a node’s service level.

Changing data as it is being transmitted across the network

      - User Plane packet injection attacks [TR 33.821]

      - User plane packet modification attacks [TR 33.821]

Modifying data or settings on a base station

Compromised/Stolen base station used to access operator network

A modified base station maybe used to send message to end-users, or interrupt or steal service from adjacent nodes or to launch attacks against other nodes including the core network

Backdoors

APT

Targeted malicious apps.

3 Removal:

- Theft, removal or loss of information &/or other resources

- Examples:	Theft of base station

Removal of confidential information or software

Theft of subscriber data (personal information, location information, authentication data, etc) may breach regulatory laws thus leading to financial penalties.  Removal of base stations may result in reduced availability, breaking SLA’s not withstanding the cost of replacement of equipment.

Removal of charging information

Operation Administration and Maintenance(OAM) could be interrupted and can be used to  removal charging information. Removal charging information would cause wrong charging and then cause to operator financial loss and user complains.

4. Disclosure:

- An unauthorized access to an asset

- Examples:    	Unauthorized data capture (data sniffing)

Discovery of private keys

Unauthorized access to sensitive data

The disclosure of private keys (e.g. keys for OAM authentication to a base station) may lead to further attacks on nodes if keys were shared amongst nodes.

Unauthorized copying of data to external location via a network channel or removable media

Location disclosure

Unauthorized session keys access - Brute force attacking of weak private keys

The brute force attacking on weak private keys can leak the private keys and result to unauthorized session key leakage. Unauthorized session key access would cause user’s message leakage even when the information is encrypted.

5. Interruption:

- Network becomes unavailable or unusable

- Examples:	 Denial of service attack

RF jamming of a radio access point

Signaling Storm from UE/Internet

Unusable application, data, device

End-User device not able to process information in timely manner

Network throughput is reduced

DoS attacks on user plane or control plane can degrade or render NE’s unusable for intermittent or longer periods of time. Targeted attacks against applications supported by 3GPP networks may result in increased traffic thus leading to reduced capacity for other services on the 3GPP network.