3GPP TSG SA WG3 (Security) Meeting #74-BIS	S3-140383

31 March – 2 April, 2014;Sophia-Antipolis (France)	revision of S3-14abcd

Source:	Alcatel Lucent

Title:	WebRTC security:- pCR to TR 33.871

Document for:				Approval

Agenda Item:			  4.5 WebRTC security

Work Item / Release:	IMS_WebRTC / Rel-12

Abstract of the contribution: This pCR updates section 4.1.2 WebRTC Control Plane with required clarifications on DTLS and ICE

### Introduction

This pCR clarifies the requirements for the WebRTC IMS Client when SIP-based SDP transport over the W2 interface is used.

*** BEGIN CHANGES ***

*********************************************************

4.1.2	WebRTC Control plane

The WebRTC control plane is sent over HTTP/WebSocket and is controlled by the WebRTC IMS Client (WIC) application of the UE. While HTTP is a request-response protocol, WebSocket provides a full-duplex communication channel over TCP. Current WebRTC specifications [7] do not specify any control-plane protocol to establish the connection between WIC peers. The WIC application can implement any signalling protocol such as SIP or RESTful HTTP or JSON over WebSocket. The WIC exchanges WebRTC User plane parameters with the peer WIC over the chosen signalling protocol.  These parameters are retrieved from the WebRTC compliant browser in the form of SDPs.

- SDP must be used with the following security requirements (see [10]):

  - DTLS-SRTP must be used. DTLS certificate fingerprint are shared with the peer DTLS endpoint. This fingerprint binds the DTLS key exchange in the media plane to the WebRTC control plane.

  - ICE must be used. ICE candidates are needed for the WebRTC Media plane to traverse through firewalls and NAT equipments.

  - The WebRTC Control plane that transports SDP between two W2 end points shall be integrity-protected.

*********************************************************

*** END CHANGES ***