3GPP TSG SA WG3 (Security) ad-hoc Meeting	S3-140619

(revision of S3-140405)

31 March- 2 April, 2014, Sophia Antipolis, France

Source:	China Mobile

Title:	A new solution—WIC direct authentication using web credentials

Document for:	Discussion & Approval

Agenda Item:	4.5

Work Item / Release:	Rel-12

Abstract of the contribution: This contribution proposes a WIC direct authentication using web credentials.

Introduction

This contribution proposes a direct authentication between the WIC and the IMS when the user uses web credentials to register with IMS, and provides the related pCR.

Discussion

In the clause 6.1.2.1 of TR33.871, there is an Editor’s Note like:

It is assumed that the user does not have access to IMS credentials and that the eP-CSCF authenticates to the IMS on behalf of the user. The user may use some other form of credentials to authenticate.

It implies that:

Conclusion 1: The authentication service and the operator should map the user’s Web identity and IMS identity in advance, and then the user can have access to Web credentials instead of IMS credentials.

Furthermore, clause 6.1.2.1 contains another Editor’s Note:

It is desirable for 3GPP to provide a security mechanism for the interface between WIC and eP-CSCF in Rel-12, but it is ffs whether this goal can be achieved in Rel-12.

Conclusion  2: Based on conlusion 1 and the above Editor’s Note, it leaves the feasibility to make a security mechanism which the WIC could use web credentials to register with the IMS directly.

As far as we concerned, it can make a direct authentication scheme for the WIC by using the web credentials.

Proposal

It is proposed to update TR 33.871 as follows.

pCR

***	BEGIN CHANGES	***

- 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	3GPP TR 23.701: "Study on the Support of WebRTC IMS Client access to IMS".

[5]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[6]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[7]	W3C Web Real-Time Communications Working Group,
http://www.w3.org/2011/04/webrtc-charter.html

[8]	IETF Real-Time Communication in WEB-browsers Working Group,
http://tools.ietf.org/wg/rtcweb/

[9]	IETF RFC 5763: " Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS)".

[10]	draft-ietf-rtcweb-security: "Security Considerations for WebRTC".

[11]	3GPP TS 33.823: "Security for usage of Generic Bootstrapping Architecture (GBA) with a User Equipment (UE) browser".

[12]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[15]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents".

[16]	3GPP TS 24.292: "IP Multimedia (IM) Core Network (CN) subsystem Centralized Services (ICS);Stage 3".

[17]	IETF RFC 5764: "Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[18]	draft-ietf-rtcweb-data-protocol: "RTCWeb Data Channel Protocol ".

[19]	draft-ejzak-dispatch-webrtc-data-channel-sdpneg: "SDP-based WebRTC data channel negotiation".

[20]	RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[X1]	IETF RFC 2617: "HTTP Authenticaiton: Basic and Digest Access Authencication".

***	NEXT CHANGE	***

      - Use of direct authentication between WIC and eP-CSCF

In this solution, user uses the web credentials to register with IMS directly, which WIC executes the SIP Digest mechanism to perform the authentication procedures.

Editor’s note: It is ffs whether this solution is in the scope of scenario 2 described in Clause U.2.1.3 of the TS 23.228.

As shown in Figure 6.1.2.4-1, the signalling flows can be demonstrated as follows:

- REGISTER request (WebRTC IMS Client to eP-CSCF)

The WebRTC IMS Client establishes a secure WebSocket connection with the eP-CSCF and sends a REGISTER request. The REIGISTER request message includes the user’s Web indentity and the WWSF identity.

Note: WWSF identity is used to distinguish the Web service provider, and serves for the proxy to find the AVs. If the Web identity is in the format of bob@socialnet.com which has already clearly shown the specific web service provider, then the WWSF identity may not be needed.

- eP-CSCF forwards the REGISTER Request

The eP-CSCF and the I-CSCF respectively forwards the REIGSTER Request towards the S-CSCF as described in Annex N of TS33.203 [5].  The I-CSCF queries the HSS to find the address of the S-CSCF. At this stage, the HSS shall check the user’s Web identity maps with a specific IMPU (e.g., via databace of the mapping relationship between user’s Web identity and IMS identity). The S-CSCF checks whether the REGISTER Request is related to SIP Digest in terms of the rules in Annex P of TS33.203. If it is, the procedures apply below.

Editor Note: Routing of incoming and outgoing calls are ffs.

Editor Note: Whether inter domain calls (i.e. between two IMS operators) are possible when web identities are used for addressing is ffs.

- Authentication Vector Request

After receiving the REGISTER request, the S-CSCF shall send the Authentication Vector Request (AV Req) towards HSS. The AV Req message includes the user’s Web indentity and the WWSF identity.

- Authentication Vector Reponse

Upon receiving the request from the S-CSCF, the HSS forwards the request towards the proxy. The proxy sends the AV consists of Web identity, qop value, the algorithm, realm, and H(A1) based on the user’s Web identity and the WWSF identity to the HSS, H(A1) consists of the Web identity, realm, and password. And the HSS passes the AV to the S-CSCF.

Note : The proxy is the intermediate entity between the IMS and the authentication service and it’s a component of the HSS. The proxy serves for sending the AV which has been synchronized for each Web user with the authentication service offline in advance. How does the synchronization process is left out of scope.

- 401 Auth_Challenge (S-CSCF to eP-CSCF)

The S-CSCF sends a SIP 401 Auth_Challenge to the eP-CSCF, including the Web identity, realm, qop, algorithm, and new generated random number nonce. Meanwhile, the S-CSCF stores H(A1).

- 401 Auth_Challenge (eP-CSCF to WebRTC IMS Client)

The eP-CSCF forwards the 401 Auth_Challenge to the WebRTC IMS Client.

- REGISTER Digest-Response (WebRTC IMS Client to eP-CSCF)

Upon the receipt of the challenge, the WIC generates a cnonce, and uses the cnonce as well as paramenters in the receive challenge to compute an authentication response refer to RFC 2617 [X1].

- eP-CSCF forwards the Digest-Response

The eP-CSCF forwards the authentication response to the I-CSCF, which queries the HSS to find the address of the S-CSCF. And then, the I-CSCF forwards the authentication response to the S-CSCF. The S-CSCF calculates the expected response using the previously stored H(A1) and stored nonce together with other parameters (e.g.,  cnonce, nonce-count, qop, as specified in RFC 2617 [X1]) and uses this to check against the response sent by the WIC.

- 200 (OK) response (S-CSCF to eP-CSCF)

The S-CSCF sends a 200 (OK) response to the eP-CSCF (via I-CSCF) indicating that Registration was successful.

- 200 (OK) response (eP-CSCF to WebRTC IMS Client)

The eP-CSCF forwards the 200 (OK) response to the WebRTC IMS Client indicating that Registration was successful.

Figure 6.1.2.4-1: WIC direct authentication using web credentials

***	END OF CHANGES	***