3GPP TS 33.107 V11.5.0 (2014-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G security;

Lawful interception architecture and functions

(Release 11)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

LTE, UMTS, Security, Architecture

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

Introduction	8

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Abbreviations	10

4	Functional architecture	12

5	Activation, deactivation and interrogation	16

5.1	Activation	17

5.1.1	X1_1-interface	17

5.1.2	X1_2-interface (IRI)	19

5.1.3	X1_3-interface (CC)	19

5.2	Deactivation	20

5.2.1	X1_1-interface	20

5.2.2	X1_2-interface (IRI)	21

5.2.3	X1_3-interface (CC)	21

5.3	Interrogation	21

5.3.1	Interrogation of the 3G ICEs	22

5.3.2	Interrogation of Delivery Functions	22

6	Invocation of Lawful Interception for Circuit Switched Services	23

6.1	Provision of Intercept CC - Circuit Switched	24

6.2	Provision of CC - Short Message Service	24

6.3	Provision of Intercept Related Information	25

6.3.1	X2-interface	25

6.3.2	Structure of the events	26

6.3.3	Call Related events	28

6.3.3.1	Call establishment	28

6.3.3.2	Answer	28

6.3.3.3	Supplementary Services	29

6.3.3.4	Handover	29

6.3.3.5	Release	29

6.3.4	Non Call Related events	30

6.3.4.1	SMS	30

6.3.4.2	Location update	30

6.3.4.3	Subscriber Controlled Input (SCI)	30

6.4	Intercept cases for circuit switched supplementary services	31

6.4.1	Interception of Multiparty call	31

6.4.2	Interception for Call Forwarding / Call Deflection / ECT	31

7	Invocation of Lawful Interception for GSN Packet Data services	32

7.1	Provision of Intercept Product - Short Message Service	33

7.2	Provision of Intercepted Content of Communications – Packet data GSN services	34

7.2.1	X3-interface	34

7.3	Provision of Intercept Related Information	34

7.3.1	X2-interface	35

7.3.2	Structure of the events	35

7.4	Packet Data related events	38

7.4.1	Mobile Station Attach	38

7.4.2	Mobile Station Detach	38

7.4.3	Packet Data PDP context activation	38

7.4.4	Start of interception with PDP context active	39

7.4.5	Packet Data PDP context deactivation	39

7.4.6	RA update	40

7.4.7	SMS	40

7.4.8	Packet Data PDP context modification	40

7.4.9	Serving System	41

7.4.10	Start of interception with mobile station attached	41

7.5	Void	41

7.6	Interception of the Multimedia Messaging Service (MMS)	41

7A	Invocation of Lawful Interception for Packet Data Multi-media Service	42

7A.1	Provision of content of communications	42

7A.2	Provision of IRI	42

7A.3	Multi-media events	42

7A.4	Multi-media Call State Control Service Scenarios	43

7A.5	Push to talk over Cellular (PoC)	43

7A.6	SMS over IMS	43

7A.7	LI for KMS based IMS Media Security	43

7A.7.1	LI Architecture and functions	43

7A.7.2 	Signalling over the Xk interfaces and LI events	44

7A.7.3	Cooperating KMSs	45

7A.7.4	Security	45

7A.8	IMS IMEI Interception	45

8	Security	46

8.1	Administration security	46

8.2	IRI security	46

8.2.1	Normal operation	46

8.2.2	Communication failure	46

8.3	CC security	47

8.4	Security aspects of Lawful Interception billing	47

8.5	Other security issues	47

8.5.1	Log files	47

8.5.2	Data consistency	47

9	Invocation of Lawful Interception for 3GPP WLAN Interworking Services	47

9.1	Provision of Intercept Product - Short Message Service	48

9.2	Provision of Intercepted Content of Communications - 3GPP WLAN Interworking services	48

9.2.1	X3-interface	49

9.3	Provision of Intercept Related Information	49

9.3.1	X2-interface	50

9.3.2	3GPP WLAN Interworking LI Events and Event Information	50

9.4	Structure of I-WLAN Events	54

9.4.1	I-WLAN Access Initiation	54

9.4.2	WLAN Access Termination	55

9.4.3	I-WLAN Tunnel Establishment	55

9.4.4	I-WLAN Tunnel Disconnect	56

9.4.5	Start of Intercept with I-WLAN Communication Active	57

10	Interception of Multimedia Broadcast/MultiCast Service (MBMS)	59

10.1	Provision of Content of Communications	59

10.2	Provision of Intercept Related Information	59

10.2.1	X2-interface	60

10.2.2	MBMS LI Events and Event Information	60

10.3	Structure of MBMS Events	62

10.3.1	Service Joining	62

10.3.2	Service Leaving	62

10.3.3	Start of Interception with Service Active	63

10.3.4	Subscription Activation	63

10.3.5	Subscription Modification	63

10.3.6	Subscription Termination	64

11	IMS Conference Services	65

11.1	Background for IMS Conference Services	65

11.1A	Start of Interception for IMS Conference Services	65

11.2	Provision of Intercepted Content of Communication – IMS Conference Services	65

11.2.1	X3-interface	66

11.3	Provision of Intercept Related Information for IMS Conference Service	66

11.3.1	X2-interface	67

11.3.2	IMS Conference Events and Event Information	67

11.3.3	Structure of Conference Events	70

11.3.3.1	Start of Conference	70

11.3.3.2	Party Join	70

11.3.3.3	Party Leave	71

11.3.3.3A	Conference Bearer Modification	71

11.3.3.4	Start of Intercept on an Active Conference	72

11.3.3.5	Conference End	72

11.3.3.6	Creation of Conference	73

11.3.3.7	Update of Conference	73

12	Lawful Interception for Evolved Packet System	75

12.1	LI functional architecture for EPS	75

12.2	Functional requirements for LI in case of E-UTRAN access and GTP based S5/S8.	76

12.2.1	Provision of Intercept Related Information	77

12.2.1.1	 X2-interface	77

12.2.1.2	 Structure of the events	77

12.2.2	X3-interface	80

12.2.3	EPS related events	81

12.2.3.1	Attach	81

12.2.3.2	Detach	81

12.2.3.3	Bearer activation	81

12.2.3.4	Bearer deactivation	82

12.2.3.5	Bearer modification	83

12.2.3.6	 Start of interception with active bearer	83

12.2.3.7	Tracking Area Update	83

12.2.3.8	Serving Evolved Packet System	84

12.2.3.9	UE requested PDN connectivity	84

12.2.3.10	UE requested PDN disconnection	84

12.2.3.11	UE requested Bearer Resource Modification	84

12.2.3.12	Void	85

12.2.3.13	Start of interception with E-UTRAN attached UE	85

12.3	Functional requirements for LI in case of E-UTRAN access and PMIP based S5/S8 interfaces	85

12.3.1	Provision of Intercept Related Information	86

12.3.1.1	X2 interface	86

12.3.1.2	Structure of the events	86

12.3.2	X3-interface	88

12.3.3	LI events for E-UTRAN access with PMIP-based S5 or S8	88

12.3.3.1	Initial E-UTRAN Attach and UE PDN requested connectivity with PMIP-based S5 or S8	88

12.3.3.2	Detach and PDN disconnection for PMIP-based S5/S8	89

12.3.3.3	Start of interception with active tunnel for PMIP based S5/S8	89

12.3.3.4	Dedicated Bearer Procedures for E-UTRAN Access with PMIP-based S5/S8	89

12.3.3.5	PDN-GW initiated PDN-disconnection Procedure	89

12.3.3.6	PMIP Session modification	90

12.4	Functional requirements for LI in case of trusted non-3GPP IP access	90

12.4.1	Provision of Intercept Related Information	90

12.4.1.1	X2-interface	91

12.4.1.2	 Structure of the events	91

12.4.2	X3-interface	94

12.4.3	LI events for trusted Non-3GPP IP access	94

12.4.3.1	Initial Attach and PDN connection activation with PMIPv6 on S2a	94

12.4.3.2	Initial Attach and PDN connection activation procedures with MIPv4 FACoA on S2a	95

12.4.3.3	Initial Attach and PDN connection activation procedures with DSMIPv6 over S2c	95

12.4.3.4	Detach and PDN disconnection with PMIPv6 on S2a	96

12.4.3.5	Detach and PDN disconnection with MIPv4 FACoA	96

12.4.3.6	Detach and PDN disconnection with DSMIPv6 on S2c	96

12.4.3.7	PDN-GW reallocation upon initial attach on s2c	96

12.4.3.8	PDN GW initiated Resource Allocation Deactivation with S2a PMIP	97

12.4.3.9	PDN GW initiated Resource Allocation Deactivation with S2a MIP v4	97

12.4.3.10	Serving Evolved Packet System	97

12.4.3.11 	Start of interception with active tunnel	97

12.4.3.12	PMIP session modification	98

12.4.3.13	DSMIP session modification	98

12.5	Functional requirements for LI in case of untrusted non-3GPP IP access	98

12.5.1	Provision of Intercept Related Information	98

12.5.1.1	X2-interface	99

12.5.1.2	Structure of the events	99

12.5.2	X3-interface	101

12.5.3	LI events for untrusted Non-3GPP IP access	101

12.5.3.1	Initial Attach and PDN connection activation with PMIPv6 on S2b	101

12.5.3.2	Initial attach and PDN connection activation for S2c in untrusted non-3GPP IP access	102

12.5.3.3	UE/ePDG-initiated Detach Procedure and UE Requested PDN disconnection with PMIP	102

12.5.3.4	Detach and PDN Disconnection for S2c in Un-trusted Non-3GPP IP access	102

12.5.3.5	Serving Evolved Packet System	103

12.5.3.6 	Start of interception with active tunnel	103

12.5.3.7	PDN-GW reallocation upon initial attach on s2c	103

12.5.3.8	PDN GW initiated Resource Allocation Deactivation with S2b PMIP	103

12.5.3.9	PMIP session modification	104

12.5.3.10	DSMIP session modification	104

12.6	Functional requirements for LI in case of Handovers between E-UTRAN and CDMA2000 Accesses.	104

12.7	Functional requirements for LI in case of interworking between SGSN and EPS nodes over S4/S12 interfaces	105

12.8	Functional requirements for LI in case of interworking between SGSN and PDN-GW over Gn/Gp interfaces	105

Annex A (informative):	Information flows for Lawful Interception invocation of circuit switched services	106

A.1	Mobile originated circuit switched calls	106

A.2	Mobile terminated circuit switched calls	107

A.3	Call hold / call waiting	108

A.4	Multiparty calls	110

A.5	Call forwarding / call deflection	113

A.5.1	Unconditional call forwarding	113

A.5.2	Call forwarding on not reachable (IMSI detached)	113

A.5.3	Call forwarding on busy (network determined)	114

A.5.4	Call forwarding on not reachable (no response to paging/radio channel failure)	115

A.5.5	Call forwarding on no reply	115

A.5.6	Call forwarding on busy (user determined)/call deflection	116

A.5.7	Call waiting / call forwarding on no reply	117

A.6	Explicit call transfer	120

Annex B (informative):	Information flows for Lawful Interception invocation of GSN Packet Data services	122

B.1	Mobile Station Attach	122

B.2	Mobile Initiated Mobile Station Detach	123

B.3	Network initiated Mobile Station Detach	123

B.4	Intra 3G GSN Routing Area Update	124

B.5	Inter 3G GSN Routing Area Update	124

B.6	PDP Context Activation	125

B.7	Start of interception with PDP context active	125

B.8	MS initiated PDP Context Deactivation	126

B.9	Network initiated PDP Context Deactivation	126

B.10	SMS	127

Annex C (informative):	Information flows for the invocation of Lawful Interception for Packet Data with multimedia	129

C.1	Multimedia registration	129

C.2	Multimedia Session Establishment and Answer	131

C.3	Multimedia Release	132

C.4	Multimedia with Supplementary Service – Call Forwarding	132

C.5	Multimedia with Supplementary Service – Explicit Call Transfer	132

C.6	Multimedia with Supplementary Service – Subscriber Controlled input	132

Annex D (informative):	Information flows for Lawful Interception invocation at the MGW using H.248	133

D.1	Mobile to Mobile call, originating side is target	133

Annex G (informative):	Change history	135

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This Technical Specification has been produced by the 3GPP TSG SA to allow for the standardisation in the area of lawful interception of telecommunications. This document describes in general the architecture and functions for lawful interception. Laws of individual nations and regional institutions (e.g. European Union), and sometimes licensing and operating conditions define a need to intercept telecommunications traffic and related information in modern telecommunications systems. It has to be noted that lawful interception shall always be done in accordance with the applicable national or regional laws and technical regulations.

# 1	Scope

The present document describes the architecture and functional requirements within a Third Generation Mobile Communication System (3GMS) and the Evolved Packet System (EPS).

The specification shows the service requirements from a Law Enforcement point of view only. The aim of this document is to define a 3GMS and EPS interception system that supports a number of regional interception regulations, but these regulations are not repeated here as they vary. Regional interception requirements shall be met in using specific (regional) mediation functions allowing only required information to be transported.

The handover interfaces for Lawful Interception (LI) of Packet-Data Services, Circuit Switched Services, and Multimedia Services within the UMTS network and Evolved Packet System for Stage 3 are described in TS 33.108 [11].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Telecommunications security; Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[4]	GSM 01.33: "Lawful Interception requirements for GSM".

[5]	GSM 02.33: "Lawful Interception - stage 1".

[6]	GSM 03.33: "Lawful Interception - stage 2".

Editor's note: 
References 4, 5 & 6 are not cited in the body of the present TS, and anyway they do not exist in this Release.

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A: "Lawfully Authorised Electronic Surveillance".

[9]	IETF RFC 2806: "URLs for Telephone Calls ".

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122: "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123: "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	(void)

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: “Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2”

[31]		3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [13] and the following apply.

Application layer: As defined by Internet Engineering Task Force (IETF) in RFC 1123 [19].

IP layer: As defined by Internet Engineering Task Force (IETF) in RFC 1122 [18]

Interception Area: is a subset of the network service area comprised of a set of cells which defines a geographical zone.

Location Dependent Interception: is interception of a target mobile within a network service area that is restricted to one or several Interception Areas (IA).

Other LI specific definitions are given in TS 33.108 [11].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [13] and the following apply:

3GMS	3rd Generation Mobile Communications System

3G GGSN	3rd Generation Gateway GPRS Support Node

3G GSN		3rd Generation GPRS Support Node (GGSN/SGSN)

3G MSC		3rd Generation Mobile Switching Center

3G SGSN	3rd Generation Serving GPRS Support Node

3G UMSC	3rd Generation Unified Mobile Switching Centre

AAA	Authentication, Authorization, and Accounting

ADMF	Administration Function

AN	Access Network

AP	Access Provider

BM-SC	Broadcast-Multicast Service Centre

CC	Content of Communication

CS	Circuit Switched

CSCF	Call Session Control Function

DF	Delivery Function

DSMIP		Dual Stack Mobile IP

ECT	Explicit Call Transfer

EPC	Evolved Packet Core

ePDG	Evolved PDG

EPS	Evolved Packet System

E-UTRAN	Evolved UTRAN

FTP	File Transfer Protocol

GGSN	Gateway GPRS Support Node

GPRS	General Packet Radio Service

GSM	Global System for Mobile communications

GSN	GPRS Support Node (SGSN or GGSN)

HA	Home Agent

HI	Handover Interface

HRPD	High Rate Packet Data

HSS	Home Subscriber Server

IA	Interception Area

ICEs	Intercepting Control Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, HLR, AAA Server, PDG, MME, S-GW, PDN-GW, HSS)

IETF	Internet Engineering Task Force

IMEI	International Mobile station Equipment Identity

IMS	IP Multimedia Core Network Subsystem

IMSI	International Mobile Subscriber Identity

INEs	Intercepting Network Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, MGW, HLR, AAA Server, PDG)

IP	Internet Protocol

IRI	Intercept Related Information

I-WLAN	Interworking WLAN (3GPP WLAN interworking subnetwork)

LAN	Local Area Network

LDI	Location Dependent Interception

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

MBMS	Multimedia Broadcast/Multicast Service

ME	Mobile Entity

MIP	Mobile IP

MME	Mobility Management Entity

MN	Mobile Node

MSISDN	Mobile Subscriber ISDN Number

NAI	Network Access Identifier

NO	Network Operator

PDG	Packet Data Gateway

PDN 	Packet Data Network

PDN-GW	PDN Gateway

PMIP	Proxy Mobile IP

PoC	Push to talk over Cellular

PS	Packet Switched

RA	Routing Area

RAI	Routing Area Identity

SAI	Service Area Identity

SGSN	Serving GPRS Support Node

SIP	Session Initiation Protocol

SMS	Short Message Service

S-GW	Serving Gateway

TEL URL	"tel" URL, as defined in RFC 2806 [9]

TLS	Transport Layer Security

UE	User Equipment

UMTS	Universal Mobile Telecommunication System

URI	Universal Resource Identifier

URL	Universal Resource Locator

VoIP	Voice over IP

WLAN	Wireless LAN

# 4	Functional architecture

The following figures contain the reference configuration for the lawful interception. The circuit-switched configuration is shown in figure 1a. The packet-switched configuration is shown in figure 1b. Intercept configurations for HLR and IMS are shown in figures 1c and 1d. The WLAN interworking configuration is shown in figure 1e. The various entities and interfaces are described in more detail in the succeeding clauses. The additional intercept configurations for Evolved 3GPP Packet Switching Domain are described in clause 12.

PS domain of the UMTS system (GSN and Multimedia Packet Data services), 3GPP-WLAN interworking network and Evolved Packet Switching Domain provide UMTS/GSM/EPS customer's mobile equipment (UE) with connectivity service to another end of the communication. Another end of the communication may be a network element (server) or another UE. Therefore, UMTS/EPS system provides IP layer TS 23.008 [15] services. Hence, UMTS/EPS NO/AP is responsible only for IP layer interception of CC data. In addition to CC data, the LI solution for UMTS/EPS offers generation of IRI records from respective control plane (signalling) messages. The IP layer connectivity service is needed to support application layer TS 29.234 [16] service provision to UMTS/GSM/EPS customers. For instance, the following are examples of application layer services: email service; web browsing service; FTP service; audio services  (e.g. VoIP, PoC); other multimedia services (MBMS, video telephony); The majority of the application layer services require addition of respective server functionality to the network. Note that it is not necessary that such application layer SP should be the same commercial entity as the UMTS/EPS AP/NO in question.

NOTE 1:	For instance in MBMS a BM-SC and especially content providing server may be operated by different commercial entity than UMTS network.

Figure 1a: Circuit switched intercept configuration

Figure 1b: Packet Switched Intercept configuration

Figure 1c: HLR Intercept configuration

Figure 1d: IMS-CSCF Intercept configuration

Figure 1e: WLAN Interworking Intercept configuration

Figure 1f: IMS Conferencing Intercept configuration

The reference configuration is only a logical representation of the entities involved in lawful interception and does not mandate separate physical entities.

Regional Mediation Functions, which may be transparent or part of the administration and delivery functions, are used to convert information on the HI1, HI2 and HI3 interfaces in the format described in various national or regional specifications. For example, if ETSI ES 201 671 [3] or ANSI J-STD-025 [8] is used, then the adaptation to HI1, HI2 and HI3 will be as defined in those specifications.

There is one Administration Function (ADMF) in the network. Together with the delivery functions it is used to hide from the 3G ICEs that there might be multiple activations by different Law Enforcement Agencies (LEAs) on the same target. The administration function may be partitioned to ensure separation of the provisioning data from different agencies.

See the remaining clauses of this document for definitions of the X1_1, X1_2, X1_3, X2 and X3 interfaces.

Interception at the Gateways is a national option. However, if 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then the GGSN shall perform the interception of IRI and the content of communications.

In figure 1a DF3 is responsible for two primary functions:

-	Call Control (Signalling) for the Content of Communication (CC); and

-	Bearer Transport for the CC.

HI3 is the interface towards the LEMF. It must be able to handle the signalling and the bearer transport for CC.

In figures 1a, 1b, 1e and 1f, the HI2 and HI3-interfaces represent the interfaces between the LEA and two delivery functions. The delivery functions are used:

-	to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2 (based on IAs, if defined);

-	to distribute the Content of Communication (CC) to the relevant LEA(s) via HI3 (based on IAs, if defined).

In figures 1c and 1d the HI2 interface represents the interface between the LEA and the delivery function. The delivery function is used to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2.

NOTE 2:	With reference to figure 1c, CC interception does not apply to HLR.

NOTE 3:	For IMS, figure 1d relates to the provision of IRI for SIP messages handled by the CSCF. Interception of CC for this case can be done at the GSN under a separate activation and invocation, according to the architecture in Figure 1b (see also clause 7.A.1).

# 5	Activation, deactivation and interrogation

Figure 2 is an extraction from the reference intercept configuration shown in figures 1a through to 1e which is relevant for activation, deactivation and interrogation of the lawful interception.

Figure 2: Functional model for Lawful Interception activation, deactivation and interrogation

In addition to the typical 3G ICEs functional entities, a new functional entity is introduced - the ADMF - the Lawful Interception administration function. The ADMF:

-	interfaces with all the LEAs that may require interception in the intercepting network;

-	keeps the intercept activities of individual LEAs separate;

-	interfaces to the intercepting network.

Every physical 3G ICE is linked by its own X1_1-interface to the ADMF. Consequently, every single 3G ICE performs interception (activation, deactivation, interrogation as well as invocation) independently from other 3G ICEs. The HI1-interface represents the interface between the requester of the lawful interception and the Lawful administration function; it is included for completeness, but is beyond the scope of standardisation in this document.

The target identities for 3GMS CS and PS interception at the SGSN, GGSN, 3G MSC Server and 3G GMSC Server can be at least one of the following: IMSI, MSISDN or IMEI.

NOTE 1:	Some communication content during a mobility procedure may not be intercepted when interception is based on MSISDN (only PS interception) or IMEI. The use of the IMSI does not have this limitation. For the availability of the target identities IMSI, MSISDN and IMEI (PS interception), refer to TS 23.060 [10].

The target identities for multi-media at the CSCF can be one or more of the following: SIP URI, TEL URL, or IMEI. Other identities are not defined in this release.

The target identities for 3GPP WLAN Interworking interception can be MSISDN, IMSI or NAI. For the availability of the target identities in the I-WLAN nodes (AAA server, PDG, WAG), refer to TS 23.234 [14], TS 23.008 [15], TS 29.234 [16] and TS 24.234 [17].

NOTE 2:	The NAI may be a temporary ID, therefore the use of MSISDN or IMSI is recommended.

NOTE 3:	 Void

In the case of location dependent interception the following network/national options exist:

-	target location versus Interception Areas (IAs) check in the 3G ICEs and Delivery Functions (DFs);

-	target location versus IAs check in the DFs (physical collocation of the DFs to the 3G ICEs may be required by national law);

-	location dependent interception is not applicable to CSCF.

NOTE 4:	The IA is previously defined by a set of cells. From the location of the target this set of cells permits to find the relevant IA.

NOTE 5:	It is not required that the 3G GMSC or the 3G GGSN are used for interception when Location Dependent Interception is invoked and the location of the target is not available.

Editors' note:	Location dependent intercept for the 3G MSC Server is not defined for this release.

The ADMF shall be able to provision P-CSCFs independently from S-CSCFs. If both P-CSCFs and S-CSCFs are administered within the network for intercept, redundant multi-media IRI may be presented to the agency as a result.

## 5.1	Activation

Figures 3, 4 and 5 show the information flow for the activation of Lawful Interception.

### 5.1.1	X1_1-interface

The messages sent from the ADMF to the 3G ICEs (X1_1-interface) contain the:

-	target identities (MSISDN, IMSI, IMEI, SIP URI or TEL URL, NAI) (see notes 4, 5, 6);

-	information whether the Content of Communication (CC) shall be provided (see note 1);

-	address of Delivery Function 2 (DF2) for the intercept related information (see note 2);

-	address of Delivery Function 3 (DF3) for the intercepted content of communications (see note 3);

-	IA in the case of location dependent interception.

NOTE 1:	As an option, the filtering whether intercept content of communications and/or intercept related information has to be provided can be part of the delivery functions. (Note that intercept content of communications options do not apply at the CSCF, HLR and AAA server). If the option is used, the corresponding information can be omitted on the X1_1-interface, while "information not present" means "intercept content of communications and related information has to be provided" for the ICE. Furthermore the delivery function which is not requested has to be "pseudo-activated", in order to prevent error cases at invocation.

NOTE 2:	As an option, only a single DF2 is used by and known to every 3G ICE. In this case the address of DF2 can be omitted.

NOTE 3:	As an option, only a single DF3 is used by and known to every 3G ICE (except at the CSCFs, HLR and AAA server). In this case the address of DF3 can be omitted.

NOTE 4:	Since the IMEI is not available, interception based on IMEI is not applicable at the 3G Gateway. Moreover, in case the IMEI is not available, interception based on IMEI is not applicable at 3G ICEs.

NOTE 5:	Interception at the CSCFs is based upon either SIP URI, TEL URL, or IMEI. SIP URI and TEL URL as target identities are not supported by the other ICEs.

NOTE 6:	Interception based on NAI is only applicable at AAA server, PDG, and WAG. As the NAI could be encrypted or based on temporary identity at the PDG and WAG, interception based on the NAI is not applicable in those cases in these nodes.

NOTE 7:	Void

If after activation subsequent Content of Communications (CC) or Intercept Related Information (IRI) has to be activated (or deactivated) an "activation change request" with the same identity of the target is to be sent.

Figure 3: Information flow on X1_1-interface for Lawful Interception activation

Interception of a target can be activated on request from different LEAs and each LEA may request interception via a different identity. In this case, each target identity on which to intercept will need to be sent via separate activation messages from ADMF to the 3G ICEs on the X1_1-interface. Each activation can be for IRI only, or both CC and IRI.

When several LEAs request activation on the same identity and the ADMF determines that there is an existing activation on the identity, the ADMF may (as an implementation option) send additional activation message(s) to the 3G ICEs. When the activation needs to change from IRI only to CC and IRI an activation change message will be sent to the 3G ICEs.

In the case of a secondary interception activation only the relevant LEAs will get the relevant IRIs.

### 5.1.2	X1_2-interface (IRI)

For the activation of IRI the message sent from the ADMF to the DF contains:

-	the target identity;

-	the address for delivery of IRI (= LEMF address);

-	which subset of information shall be delivered;

-	a DF2 activation identity, which uniquely identifies the activation for DF2 and is used for further interrogation or deactivation, respectively;

-	the IA in case of location dependent interception;

-	the warrant reference number if required by national option.

If a target is intercepted for several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.

Figure 4: Information flow on X1_2-interface for Lawful Interception activation

### 5.1.3	X1_3-interface (CC)

For the activation of intercepted Content of Communications the message sent from the ADMF to the Delivery Function contains:

-	the target identity;

-	the address of delivery for CC (= LEMF address);

-	a DF3 activation identity, which uniquely identifies the activation for DF3 and is used for further interrogation or deactivation, respectively;

-	the IA in case of location dependent interception;

-	the warrant reference number if required by national option.

If a target is intercepted by several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.

Figure 5: Information flow on X1_3-interface for Lawful Interception activation

## 5.2	Deactivation

Figures 6, 7 and 8 show the information flow for the deactivation of the Lawful interception.

### 5.2.1	X1_1-interface

The messages sent from the ADMF to the 3G ICEs for deactivation contain:

-	the target identity;

-	the possible relevant IAs in case of location dependent interception.

Figure 6: Information flow on X1_1-interface for Lawful Interception deactivation

If interception of a target has been activated via different identities then a separate deactivation message will need to be sent from the ADMF to the 3G ICEs for each identity.

When several LEAs requested activation on the same identity and subsequently request deactivation then the ADMF determines that there are remaining activations on the identity. In this case, the ADMF will not send a deactivation message to the 3G ICEs except when the activation needs to change from CC and IRI to IRI only. In that case an activation change message will be sent to the 3G ICEs.

### 5.2.2	X1_2-interface (IRI)

The message(s) sent from the ADMF to Delivery Function 2 for the deactivation of the Intercept Related Information contains:

-	a DF2 activation ID, which uniquely identifies the activation to be deactivated for DF2.

If a target is intercepted by several LEAs and/or several identities simultaneously, a single deactivation is necessary for each combination of LEA and identity.

Figure 7: Information flow on X1_2-interface for Lawful Interception deactivation

### 5.2.3	X1_3-interface (CC)

For deactivating the delivery of the CC the message(s) sent from the ADMF to DF3 contains:

-	a DF3 activation ID, which uniquely identifies the activation to be deactivated for DF3.

Figure 8: Information flow on X1_3-interface for Lawful Interception deactivation

## 5.3	Interrogation

Interrogation provides the current status of the interception activation in the system. Interrogation of all activations for a given LEA is an ADMF function.

### 5.3.1	Interrogation of the 3G ICEs

Figure 9 shows the information flow for the interrogation of the Lawful Interception. It shall be possible to interrogate:

-	a specific activation at each relevant 3G ICEs;

-	all activations at each relevant 3G ICEs.

As a result of the interrogation the activation status and data are returned.

Figure 9: Interrogation of the Lawful Interception (3G ICEs)

### 5.3.2	Interrogation of Delivery Functions

Figure 10 shows the information flow for the interrogation of the Lawful Interception. It shall be possible to interrogate:

-	a specific activation at a DF;

-	all activations at a DF for a given target identity;

-	all activations at a DF.

As a result of the interrogation the activation status and data are returned.

Figure 10: Interrogation of the Lawful Interception (Delivery Functions)

# 6	Invocation of Lawful Interception for Circuit Switched Services

Figure 11 shows an extraction from the reference configuration in figure 1a which is relevant for the invocation of the lawful interception.

Figure 11: Functional model for Lawful Interception invocation

The HI2 and HI3 interfaces represent the interfaces between the LEMF and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of standardization in this document. The delivery functions are used:

-	to convert the information on the X2-interface to the corresponding information on the HI2-interface;

-	to convert the information on the X3-interface to the corresponding information on the HI3-interface;

-	to distribute the intercept related information to the relevant LEA(s) (based on IAs, if defined);

-	to distribute the intercept content of communicationsto the relevant LEA(s) (based on IAs, if defined).

For the delivery of the CC and IRI, the 3G MSC Server provides a correlation number and target identity to the DF2 and DF3 which is used to select the different LEAs to which the product shall be delivered.

NOTE:	If interception has been activated for both parties of the call both CC and IRI will be delivered for each party as separate intercept activity.

The Mc interface between the 3G MSC Server and MGW is used to establish intercept and deliver the bearer to DF3.

For Location Dependent Interception, the location dependency check occurs at the establishment of each call. Subsequent dependency checks for simultaneous calls are not required, but can be a national option.

If a target is marked using an IA in the 3G MSC Server, the 3G MSC Server shall perform a location dependency check at call set-up. Only if the target's location matches the IA then the call is intercepted.

If a target is marked using an IA in the DF2, the DF2 shall perform a location dependency check at reception of the first IRI for the call. Only if the target's location matches the IA for certain LEAs is IRI the relayed to these LEAs. All subsequent IRIs for the call are sent to the same LEAs.

If a target is marked using an IA in the DF3, the DF3 signalling function shall perform a location dependency check at reception of the CC. Only if the target's location matches the IA for certain LEAs is the CC relayed to these LEAs.

## 6.1	Provision of Intercept CC - Circuit Switched

Figure 12 shows the access method for the delivering of CC. The access method shall be a bridged/ T-connection.

Figure 12: Delivery configuration to the LEMF for the interception of a circuit switched call

The signals of both parties of the configuration to be intercepted are delivered separately to the LEMF. The delivery function has no impact on the connection between the subscribers.

The two stublines towards the LEMF are established in parallel to the call set up. For both stublines the address is used which has been provided during activation.

Bearer, and only bearer, is sent from the MGW to the bearer function of DF3.

NOTE 1:	For data calls it is necessary to provide means for fast call establishment towards the LEMF to help ensure that the beginning of the data transmission is delivered.

The following information needs to be transferred from the 3G MSC Server to the DF3 in order to allow the DF3 to perform its functionality:

-	target identity (MSISDN, IMSI or IMEI); note 2

-	the target location (if available) or the IAs in case of location dependent interception. note 2

-	correlation number (IRI <-> CC);

-	direction indication - (Signal from target or signal to target).

NOTE 2:	For DF3 internal use only.

Additional information may be provided if required by national laws.

## 6.2	Provision of CC - Short Message Service

Figure 14 shows an SMS transfer from the 3G MSC Server to the LEMF. Quasi-parallel to the delivery from / to the mobile subscriber a message, which contains the contents of the SMS with the header, is generated and sent via the Delivery Function 2 to the LEMF in the same way as the Intercept Related Information.

The IRI will be delivered to the LEMF:

-	for a SMS-MO. Dependent on national requirements, delivery shall occur either when the 3G MSC receives the SMS from the target MS, or when the 3G MSC receives notification that the SMS-Centre successfully received the SMS;

-	for a SMS-MT. Dependent on national requirements, delivery shall occur either when the 3G MSC receives the SMS from the SMSC, or when the 3G MSC receives notification that the target MS successfully received the SMS.

Figure 14: Provision of Content of Communication - Short Message Service

## 6.3	Provision of Intercept Related Information

Intercept Related Information (Events) are necessary at the Begin and End of the call, for all supplementary services during a call and for information which is not call associated. There are call related events and non call related events.

Figure 15 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the 3G MSC Server sends the relevant data to the DF2.

Figure 15: Provision of Intercept Related Information

### 6.3.1	X2-interface

The following information needs to be transferred from the 3G MSC Server to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (MSISDN, IMSI or IMEI);

-	in case of location dependent interception, the IAs and/or target cell ID shall be provided;

-	events and associated parameters as defined in clauses 6.3.3 and 6.3.4 may be provided.

The IRI should be sent to DF2 with a reliable transport mechanism.

### 6.3.2	Structure of the events

The information sent to DF2 is triggered by up to eight different call related and non-call related events. Details are described in following clause. The events for interception are configurable (if they are sent to DF2) in the 3G MSC Server and can be suppressed in the DF2. The events are listed as follows:

Call Related Events:

-	Call Establishment

-	Answer

-	Supplementary Service

-	Handover

-	Release

Non Call Related Events:

-	SMS

-	Location Update

-	Subscriber Controlled Input

Table 1 below shows the set of information that can be associated with the events. The events trigger the transmission of the information from the 3G MSC Server to DF2. Available IEs from this set of information can be extended in the 3G MSC Server, if this is necessary in a specific country. DF2 can extend available information if this is necessary in a specific country e.g. a unique number for each surveillance warrant.

Table 1: Information Elements for Circuit Event records

Observed MSISDN Target Identifier with the MSISDN of the target subscriber (monitored subscriber).

Observed IMSI Target Identifier with the IMSI of the target subscriber (monitored subscriber).

Observed IMEI Target Identifier with the IMEI of the target subscriber (monitored subscriber), It shall be checked for each call over the radio interface

event type Description which type of event is delivered: Establishment, Answer, Supplementary service, Handover, Release, SMS, Location update, Subscriber controlled input

event date Date of the event generation in the 3G MSC Server

event time Time of the event generation in the 3G MSC Server

dialled number Dialled phone number before digit modification, IN-modification etc.

Connected number Number of the answering party

other party address Directory number of the other party for MOC Calling party for MTC

call direction Information if the monitored subscriber is calling or called e.g. MOC/MTC or originating/ terminating In or/out

Correlation number Unique number for each call sent to the DF, to help the LEA, to have a correlation between each Call and the IRI

Network Element Identifier Unique identifier for the element reporting the ICE.

Location Information Location information is the service area identity and/or location area identity that is present at the 3G MSC Server at the time of event record production. In some traffic cases the available location information can be the one received from the MME, i.e. the Tracking Area Identity (TAI) and/or the E-UTRAN Cell Global Identification (ECGI) as specified in the 3GPP TS 23.272 [30].

basic service Information about Tele service or bearer service.

Supplementary service Supplementary services used by the target e.g. CF, CW, ECT

Forwarded to number Forwarded to number at CF

call release reason Call release reason of the target call

SMS initiator SMS indicator whether the SMS is MO, MT, or undefined

SMS Message The SMS content with header which is sent with the SMS-service

Redirecting number The number which invokes the call forwarding towards the target. This is provided if available.

SCI Non call related Subscriber Controlled Input (SCI) which the 3G MSC Server receives from the ME

### 6.3.3	Call Related events

#### 6.3.3.1	Call establishment

For call establishment a call establishment-event is generated. This event is generated at the beginning of a call when the 3G MSC Server attempts to reach the subscriber. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

event type

event date

event time

dialled number

other party address

call direction

Correlation number

Redirecting number

Network Element Identifier

Location Information

basic service

Supplementary service

#### 6.3.3.2	Answer

If the called party answers, an answer- event is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

event type

event date

event time

dialled number

other party address

Connected party

call direction

Correlation number

Redirecting number

Network Element Identifier

Location Information

basic service

Supplementary service

#### 6.3.3.3	Supplementary Services

For supplementary services events are generated with the information which supplementary service is used e.g. Call Forwarding (CF), Call Waiting (CW), Explicit Call Transfer (ECT), Multi Party (MPTY), Call Hold and information correlated to the service like the forwarded to number. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

event type

event date

event time

dialled number

other party address

call direction

Correlation number

Network Element Identifier

Location Information

basic service

Supplementary service

Forwarded to number

#### 6.3.3.4	Handover

For each handover that is realised at the 3G MSC Server due to a change in target location information, a handover-event with the new location information is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

event type

event date

event time

Correlation number

Network Element Identifier

Location Information

#### 6.3.3.5	Release

For the release or failed attempt of a target call, a release event with the following information is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

event type

event date

event time

dialled number

other party address

call direction

Correlation number

Network Element Identifier

Location Information

basic service

call release reason

### 6.3.4	Non Call Related events

#### 6.3.4.1	SMS

For MO-SMS the event is generated in the 3G MSC Server. Dependent on national requirements, event generation shall occur either when the 3G MSC Server receives the SMS from the target MS or when the 3G MSC Server receives notification that the SMSC successfully receives the SMS; for MT-SMS the event is generated in the 3G MSC Server. Dependent on national requirements, event generation shall occur either when the 3G MSC Server receives the SMS from the SMSC or when the 3G MSC Server receives notification that the target MS successfully received the message. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

event type

event date

event time

Network Element Identifier

Location Information

SMS initiator

SMS Message

#### 6.3.4.2	Location update

For location updates a Location update-event is generated, with the new location information. This information will be delivered to the DF2 if available:

Observed MSISDN

observed IMSI

event type

event date

event time

Network Element Identifier

Location Information

#### 6.3.4.3	Subscriber Controlled Input (SCI)

SCI includes subscriber initiated changes in service activation and deactivation. SCI does not include any information available in the CC. For subscriber controlled inputs - a SCI-event is generated with information about the SCI. This information will be delivered to the DF2 if available:

observed MSISDN

observed IMSI

event type

event date

event time

Network Element Identifier

Location Information

SCI

## 6.4	Intercept cases for circuit switched supplementary services

### 6.4.1	Interception of Multiparty call

Figure 16: Interception of Multiparty for CC

Figure 16 shows the delivery of CC from intercepted multiparty call where party A is the target of interception.

One pair of call content channels are delivered to the delivery function. Party A is delivered to the DF3 on one channel and the sum of the balance of the parties, B,C and D is delivered on the second channel.

It should be noted that if parties B,C or D is a target of interception, that intercept is treated as a simple call intercept.

The events contain information about B, C and D if subscriber A is monitored. If one of B, C or D is monitored, events contain the information about A but not the other parties of the conference.

### 6.4.2	Interception for Call Forwarding / Call Deflection / ECT

Figure 17: Interception for Call Forwarding / Deflection / ECT

The interception of party B once the supplementary service is invoked is a national option.

For Intercept Related Information it depends who is monitored:

-	If subscriber A is monitored the number of A and B are mandatory in the event information and the number of C if available.

-	If subscriber B is monitored the number of B and C are mandatory in the event information and the number of A if available.

-	If subscriber C is monitored the number of C is mandatory in the event information and the number of A and B if available.

Intercept requirements for CS multi-media is not defined in this release.

# 7	Invocation of Lawful Interception for GSN Packet Data services

Figure 18 shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the packet data GSN network.

Delivery

Function 2

3G

GSN

X2

X3

Delivery

Function 3

HI2

HI3

LEAs

Figure 18: Functional model for Packet Data GSN Network Lawful Interception invocation

The HI2 and HI3 interfaces represent the interfaces between the LEA and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of this specification. The delivery functions are used:

-	to convert the information on the X2-interface to the corresponding information on the HI2 interface;

-	to distribute the intercept related information to the relevant LEA(s);

-	to distribute the intercept product to the relevant LEA(s).

For the delivery of the CC and IRI the 3G SGSN and/or, per national option 3G GGSN provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered. When the SGSN connects an UE to a S-GW through the S4 interface ([10], see also note 3), the SGSN is not required to provide CC for that communication (see note 4).

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one PDP context.

- The correlation number shall be generated by using existing parameters related to the PDP context.

- When the SGSN connects an UE to a S-GW through the S4 interface ([10], see also note 3), the SGSN is not required to provide IRIs for PDP contexts associated with CC and correlation for that communication (see note 4).

NOTE 1:	If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

In case of location dependent interception:

-	for each target, the location dependency check occurs at each Packet Data session establishment or release and at each Routing Area (RA) update to determine permanently the relevant IAs (and deduce, the possible LEAs within these IAs);

-	concerning the IRI:

-	when an IA is left, either a Mobile Station Detach event is sent when changing servicing 3G GSNs, or an RA update event is sent;

-	RA update event is sent to DF2 when changing IAs inside the same servicing 3G SGSN;

-	when a new IA is entered a RA update event is sent to DF2 and, optionally, a "Start of interception with PDP context active" event for each PDP context;

-	concerning the CC, when crossing IAs, the CC is not sent anymore to the DF3 of the old IA but sent to the DF3 of the new IA.

Both in case of location dependent and location independent interception:

"Start of interception with PDP context active" event is sent by the new SGSN if an Inter-SGSN RA update procedure, which involves different PLMNs, takes place for a target, which has at least one active PDP context.

NOTE 2:	An SGSN can differentiate "Inter PLMN" type of Inter-SGSN RA update procedure from "Intra PLMN" type of Inter-SGSN RA update procedure by inspecting the old RAI parameter, which is being received by the SGSN as part of the procedure (see TS 23.060 [10], clause 6.9.1.2.2 and TS 23.003, clause 4.2).

Optionally, it is possible to send "Start of interception with PDP context active" for all cases of inter- SGSN RA update when at least one PDP context is active.

NOTE 3: S4 is an intra-PLMN reference point between the SGSN and the S-GW.

NOTE 4: When the SGSN connects an UE to a S-GW through the S4 interface, the S-GW provides IRI, CC and correlation for the EPS bearer associated to the PDP context, as specified in clause 12.

## 7.1	Provision of Intercept Product - Short Message Service

Figure 19 shows an SMS transfer from the 3G SGSN node to the LEA. Quasi-parallel to the delivery from / to the mobile subscriber a SMS event, which contains the content and header of the SMS, is generated and sent via the Delivery Function 2 to the LEA in the same way as the Intercept Related Information. National regulations and warrant type determine if a SMS event shall contain only SMS header, or SMS header and SMS content.

The IRI will be delivered to the LEA:

-	for a SMS-MO. Dependent on national requirements, delivery shall occur either when the 3G SGSN receives the SMS from the target MS or when the 3G SGSN receives notification that the SMS-Centre successfully received the SMS;

-	for a SMS-MT. Dependent on national requirements, delivery shall occur either when the 3G SGSN receives the SMS from the SMS-Centre or when the 3G SGSN receives notification that the target MS successfully received the SMS.

3G GSN

intercepted

subscriber

SMS

service

center

LEA

delivery

function 2

Figure 19: Provision of Intercept Product - Short Message Service

## 7.2	Provision of Intercepted Content of Communications – Packet data GSN services

The access method for the delivering of Packet Data GSN Intercept Product is based on duplication of packets without modification at 3G GSN. The duplicated packets with additional information in a header, as described in 7.2.1, are sent to DF3 for further delivery to the LEA.

3G GSN

intercepted

subscriber

other

party

Delivery

Function 3

LEA

Duplicator

of

packets

Figure 20: Configuration for interception of Packet Data GSN product data

### 7.2.1	X3-interface

In addition to the intercepted content of communications, the following information needs to be transferred from the 3G GSN to the DF3 in order to allow the DF3 to perform its functionality:

-	target identity;

-	correlation number;

-	time stamp - optional;

-	direction (indicates whether T-PDU is MO or MT) - optional;

-	the target location (if available) or the IAs in case of location dependent interception.

As a national option, in the case where the 3G GGSN is performing interception of the content of communications, the intercept subject is handed off to another SGSN and the same 3G GGSN continues to handle the content of communications subject to roaming agreements, the 3G GGSN shall continue to perform the interception of the content of communication.

If 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then the GGSN shall perform the interception of the content of communications.

## 7.3	Provision of Intercept Related Information

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, Serving System and SMS events.

Serving System event reporting is a national option.

Figure 21 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the 3G GSN or the Home Location Register (HLR) sends the relevant data to the DF2.

See clause 7A for multi-media Intercept Related Information produced at the CSCF.

intercepted

subscriber

HLR

other

party

Delivery

Function 2

LEMF

3G GSN

Figure 21: Provision of Intercept Related Information

### 7.3.1	X2-interface

The following information needs to be transferred from the 3G GSN or the HLR to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (MSISDN, IMSI, IMEI);

-	events and associated parameters as defined in clauses 7.3.2 and 7.4 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	Correlation number;

-	Quality of Service (QoS) identifier;

-	Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

### 7.3.2	Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. Details are described in the following clause. The events for interception are configurable (if they are sent to DF2) in the 3G GSN or the HLR and can be suppressed in the DF2.

The following events are applicable to 3G SGSN:

-	Mobile Station Attach;

-	Mobile Station Detach;

-	PDP context activation;

-	Start of interception with mobile station attached (national option);

-	Start of intercept with PDP context active;

-	PDP context modification;

-	PDP context deactivation;

-	RA update;

-	SMS.

NOTE:	3G GGSN interception is a national option. Location information may not be available in this case.

If 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then both the SGSN and the GGSN shall perform the interception of intercept related information.

When the SGSN connects an UE to a S-GW through the S4 interface ([10]), the SGSN is not required to report events PDP context activation (successful), Start of intercept with PDP context active, PDP context modification, PDP context deactivation; the SGSN shall report unsuccessful PDP context activation event.

The following events are applicable to the 3G GGSN:

-	PDP context activation;

-	PDP context modification;

-	PDP context deactivation;

-	Start of interception with PDP context active.

The following events are applicable to the HLR:

-	Serving System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from 3G GSN or HLR to DF2. Available IEs from this set of elements as shown below can be extended in the 3G GSN or HLR, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

Table 2: Information Events for Packet Data Event Records

Observed MSISDN MSISDN of the target subscriber (monitored subscriber).

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed IMEI IMEI of the target subscriber (monitored subscriber),it shall be checked for each activation over the radio interface.

Event type Description which type of event is delivered: MS attach, MS detach, PDP context activation, Start of intercept with PDP context active, PDP context deactivation, SMS, Serving System, Cell and/or RA update.

Event date Date of the event generation in the 3G GSN or the HLR.

Event time Time of the event generation in the 3G GSN or the HLR. Timestamp shall be generated relative to GSN or HLR internal clock.

PDP address The PDP address of the target subscriber. Note that this address might be dynamic. In case the PDP type is IPv4v6, the parameter may carry two IP addresses.

Access Point Name The APN of the access point. (Typically the GGSN of the other party).

Location Information Location Information is the Service Area Identity (SAI), RAI and/or location area identity that is present at the GSN at the time of event record production.

Old Location Information Location Information of the subscriber before Routing Area Update

PDP Type The used PDP type.

Correlation Number The correlation number is used to correlate CC and IRI.

SMS The SMS content with header which is sent with the SMS-service. The header also includes the SMS-Centre address.

Network Element Identifier Unique identifier for the element reporting the ICE.

Failed attach reason Reason for failed attach of the target subscriber.

Failed context activation reason Reason for failed context activation of the target subscriber.

IAs The observed Interception Areas.

Initiator The initiator of the PDP context activation, deactivation or modification request either the network or the 3G MS.

SMS Initiator SMS indicator whether the SMS is MO or MT.

Deactivation / termination cause The termination cause of the PDP context.

QoS This field indicates the Quality of Service associated with the PDP Context procedure.

Serving System Address Information about the serving system (e.g. serving SGSN number or serving SGSN address).

NSAPI Network layer Service Access Point Identifier The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane. This is a optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks when the GGSN is used as element of the PDG according TS 23.234 [14], Annex F.

## 7.4	Packet Data related events

### 7.4.1	Mobile Station Attach

For attach an attach-event is generated. When an attach activation is generated from the mobile to serving 3G G SN this event is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

Failed attach reason

IAs (if applicable)

### 7.4.2	Mobile Station Detach

For detach a detach-event is generated, this is for the common (end) detach. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

IAs (if applicable)

### 7.4.3	Packet Data PDP context activation

When a PDP context activation is generated a PDP context activation-event is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Location Information

Failed context activation reason

IAs (if applicable)

Initiator (optional)

QoS (optional)

NSAPI (optional)

### 7.4.4	Start of interception with PDP context active

This event will be generated if interception for a target is started and if the target has at least one PDP context active. If more then one PDP context are open, for each of them an event record is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Location Information

Old Location Information (optional)

IAs (if applicable)

QoS (optional)

Initiator (optional)

NSAPI (optional)

Presence of the optional Old Location Information field indicates that PDP context was already active, and being intercepted. However, the absence of this information does not imply that interception has not started in the old location SGSN for an active PDP context.

Start of interception with PDP context active shall be sent regardless of whether a Start of interception with mobile station attached has already been sent.

### 7.4.5	Packet Data PDP context deactivation

At PDP context deactivation a PDP context deactivation-event is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access point name

Network Element Identifier

Location Information

IAs (if applicable)

Deactivation cause

Initiator (optional)

NSAPI (optional)

### 7.4.6	RA update

For each RA update an update-event with the elements about the new location is generated. New SGSN shall send the event, and the old SGSN may optionally send the event as well. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Location Information (only for the new SGSN)

Old Location Information (only for the old SGSN)

IAs (if applicable)

NOTE:	Once target moves out of the interception area, old SGSN may report the RAU event. Normally, however, the old SGSN does not receive the new SGSN's RAI, while the new SGSN does receive the old SGSN's RAI from UE with the RAU Request message.

### 7.4.7	SMS

For MO-SMS the event is generated in the 3G SGSN. Dependent on national requirements, event generation shall occur either when the 3G SGSN receives the SMS from the target MS or when the 3G SGSN receives notification that the SMS-Centre successfully receives the SMS; for MT-SMS the event is generated in the 3G SGSN. Dependent on national requirements, event generation shall occur either when the 3G SGSN receives the SMS from the SMS-Centre or when the 3G SGSN receives notification that the target MS successfully received the message. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

SMS

SMS Initiator

IAs (if applicable)

### 7.4.8	Packet Data PDP context modification

This event will be generated if an active PDP context for the target subscriber is modified. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Location Information

IAs (if applicable)

Initiator

QoS

### 7.4.9	Serving System

The Serving System report event is generated at the HLR, when the HLR has detected that the intercept subject has roamed. The elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Serving System Address

### 7.4.10	Start of interception with mobile station attached

This event will be generated if interception has started for the already attached target. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

IAs (if applicable)

## 7.5	Void

## 7.6	Interception of the Multimedia Messaging Service (MMS)

The Multimedia Messaging Service (MMS) is a service running over the 3GPP PS-domain. Both mobile originating and mobile terminating MMS messages must pass through PS domain GSN nodes en route to or from Multimedia Message Service Centres (MMSCs). Therefore, interception of MMS messages shall be performed at the GSN in exactly the same way as for other PS-domain bearer services.

The GSN is not responsible for recovering individual MMS messages from the user PDP context IP stream.

No MMS specific HI2 records are defined to be delivered to the LEMF over the DF2 other than those listed in clause 7.4 of this specification. CC records shall be sent to the LEMF over the DF3 as specified in clause 7.3.

Interception of a user PDP context IP stream will occur as described in clause 7.2. Such a stream may or may not contain MMS messages.

# 7A	Invocation of Lawful Interception for Packet Data Multi-media Service

## 7A.1	Provision of content of communications

Interception of the content of communications for GSN packet data services is explained in clause 7.2. No additional content of communications intercept requirements are identified. Activation and invocation of multi-media service does not produce interception of content of communications, which must be in case of GPRS / UMTS (PS domain) intercepted at the GSN under a separate activation and invocation.

Note: Interception at the GSN is only possible for a basic call. Content of communication for forwarded or transferred calls is for further study.

## 7A.2	Provision of IRI

SIP messaging is reported as Intercept Related Information for the interception of multi-media service. As shown in figure 22 below, all SIP messages executed on behalf of a target subscriber are subject to intercept at the S-CSCF and Optionally P-CSCF. Based upon network configuration, the ADMF shall provision P-CSCFs, or S-CSCFs, or both P-CSCFs and S-CSCFs with SIP URI, TEL URL, or IMEI target identifiers. These resulting intercepted SIP messages shall be sent to DF2 for mediation prior to transmittal across the HI2 interface.

For roaming scenarios, interception at the P-CSCF shall be Mandatory, in order to provide IRI Interception in the visited network, where the P-CSCF is located in the Visited Network. Where the P-CSCF is located in the Home Network, interception at the P-CSCF shall be Optional, subject to national regulation.

ADMF

P

-

CSCF

S-CSCF

DF2

LEMF

Intercepted

Subscriber

Other

party

Figure 22: Provision of Intercept Related Information for multi-media

## 7A.3	Multi-media events

-	All SIP messages to or from a targeted subscriber, and all SIP messages executed on behalf of a targeted subscriber for multi-media session control are intercepted by the S-CSCF and Optionally P-CSCF and sent to DF2. The target identifier used to trigger the intercept will also be sent with the SIP message. This standard does not require nor prohibit redundant information from being reported to DF2.

-	Where a CSCF which provides lawful interception makes changes to a SIP message, sent to or from or executed on behalf of a target subscriber then the CSCF shall report both the original message and the modified message to the DF2.

-	Where a CSCF which provides lawful interception changes identities within a SIP message (eg IMPI/IMPU changes or due to call forwarding etc) and the new identity is the subject of interception, then both the original and modified SIP messages shall be reported to DF2.

-	Where a CSCF which provides lawful interception changes identities within a SIP message (eg IMPI/IMPU changes or due to call forwarding etc) and the new identity is not the subject of interception, then both the original and modified SIP messages shall be reported to DF2.

-	P-CSCF event reports may be redundant with S-CSCF event reports when the P-CSCF and S-CSCF reside in the same network, however, this standard does not require nor prohibit redundant information from being reported to DF2.

-	The IRI should be sent to DF2 with a reliable transport mechanism.

-	Correlation for SIP to bearer shall be supported within the domain of one provider.

-	An intercepted SIP event sent to DF2 is shown below:

-	Observed SIP URI

-	Observed TEL URL

-	Observed IMEI

-	Event Time and Date

-	Network element identifier

-	SIP Message Header

-	SIP Message Payload

NOTE: The Observed IMEI is obtained from the +sip.instance.id of the intercepted SIP message (as defined in 3GPP TS 24.229 [49]).

## 7A.4	Multi-media Call State Control Service Scenarios

Annex C shows examples of the delivery of intercepted events and product under various call scenarios.

## 7A.5	Push to talk over Cellular (PoC)

PoC is a service of the IMS Domain and interception is done according the definitions in clause 7A.3. Interception of CC is available with the current implementations in the GSNs.

## 7A.6	SMS over IMS

SMS over IMS shall be intercepted in accordance with normal IMS interception as described in 7A.3.  SMS IRI (including originating and destination addresses, SMS direction, and SMS Centre Address) are reported, if available, for IRI-only intercepts.

## 7A.7	LI for KMS based IMS Media Security

### 7A.7.1	LI Architecture and functions

KMS based IMS media security is specified in [25]. The present clause specifies LI architecture and functions needed to provide session encryption keys generated by the KMS to protect IMS media for a subscriber who is a target for interception in the IMS nodes. This section is applicable to the cases in which the KMS is under responsibility of the Operator providing the IMS network infrastructure. Other scenarios such as the one in which the KMS is run by an independent legal entity are outside the scope of this specification.

NOTE 1: It is FFS whether the Xk interface defined in this section can be used also by the LEMF to directly query the KMS as an additional option.

NOTE 2: This section covers the scenario in which encrypted content of communication is provided to the LEMF together with encryption keys, to allow decryption at LEMF. Decryption done in the network (i.e. at DF/MF) is FFS.

Figure 7A.7.1 shows the LI architecture for the case in which a KMS is used to support IMS media security, with a Xk interface defined between the DF2/MF and the KMS, in addition to the interfaces and functional entities needed to support LI in the P-CSCF/S-CSCF.

Figure 7A.7.1: KMS Intercept configuration

When LI has been activated in the P/S-CSCF for a target subscriber, the node will report SIP messages events on the X2 interface, as specified in section 7.A and subsections. The DF2/MF shall extract from the intercepted SIP signaling the information related to the encryption and send a request over the Xk interface to the KMS to derive the encryption keys; the request will carry also the reference to the ticket transferred by the SIP signalling between the parties involved in the communication. The KMS shall then, based on the information received from the DF2, resolve the ticket and provide the session keys to the DF2/MF over the Xk interface.

### 7A.7.2 	Signalling over the Xk interfaces and LI events

The following messages are defined over the Xk interface:

- get_keys

- get_keys_response

The message get_keys shall be sent by the DF2/MF to the KMS in order to ask the KMS to provide session keys for an ongoing communication.

The message get_keys_response shall be sent by the KMS to the DF2/MF in order to provide the session keys.

The message get_key_response defines a LI event provided by the KMS to the DF2/MF which shall then be sent by the DF2/MF to the LEMF in a proper IRI record over the HI2 interface.

Table 7A.7.2.1 provides the list of parameters, which shall be carried by the message get_keys, in order to transfer to the KMS the information, as specified in [25], needed to provide the session encryption keys:

Table 7A.7.2.1 Parameters and information in message get_keys

Public KMS Identity of the target user

TRANSFER_INIT

TRANSFER_RESP

Upon reception of get_keys message, the KMS shall verify that the key management information is related to the targeted user.

Table 7A.7.2.2 provides the list of parameters, which shall be carried by the message get_keys_response, in order to provide the DF2/MF with the session keys:

Table 7A.7.2.2 Parameters and information in message get_keys_response

Crypto Session ID

Session key

Salt

The following information shall be added by the DF2/MF to the session keys retrieved in get_keys_response and provided to the LEMF:

- Lawful interception identifier

- Observed target identity(ies)

- Correlation number (in order to correlate the keys to IMS session under interception at the CSCF(s))

- Event type (session encryption keys available).

### 7A.7.3	Cooperating KMSs

As specified in [25], in some scenarios the parties involved in an encrypted IMS based communication may use two different KMSs. In these cases, no additional LI specific signalling between the KMSs shall take place. The KMS may need to cache the session keys retrieved as result of the ticket resolution for possible LI needs at later stage.

### 7A.7.4	Security

Xk interface and its configuration shall only be accessible to authorized personnel.

The Xk interface shall have strong integrity and confidentiality protection. The Xk interface shall be protected by TLS  unless protected by IPsec for LI purposes. TLS and certificate profiling shall be according to TS 33.310 [28]; IPsec profiling shall be according to TS 33.310 [28] and TS 33.210 [29].

## 7A.8	IMS IMEI Interception

The use of Instance ID in TS 24.229[31] is mandatory in IMS in order to support IMEI based LI in the CSCF. The CSCF is required to have access to the Instance IDs for all active IMS registrations regardless of whether LI has been requested prior to UE registration for a given IMEI. The CSCF shall be responsible for extracting the IMEI from the Instance ID where required for a specific target interception and providing the IMEI to the DF2. The IMEI (when available) shall be provided by the CSCF to the DF2 for all intercepted communication regardless of whether the IMEI or another identifier has been used as the target for interception.

Based on the national regulations, IMEI-based LI shall be possible for IMS sessions originated from, or terminated to, the UE with that IMEI.

# 8	Security

The security requirements are valid for the whole Lawful Interception system, i.e. rules and procedures shall be used for all involved entities, 3G GSN and the DF.

## 8.1	Administration security

The administration of the LI function, i.e. Activation, Deactivation and Interrogation of Lawful Interception, in the 3G ICEs and the DFs shall be done securely as described below:

-	It shall be possible to configure the authorised user access within the serving network to Activate, Deactivate and Interrogate Lawful Interception separately for every physical or logical port at the 3G ICEs and DF. It shall be possible to password protect user access.

-	Only the ADMF is allowed to have access to the LI functionality in the 3G ICEs and DF.

-	The communication links between ADMF, 3G GSN,3G MSC Server, CSCF, DF2, and DF3 may be required by national option to support security mechanisms. Options for security mechanisms include:

-	CUG / VPN;

-	COLP;

-	CLIP;

-	authentication;

-	encryption.

Through the use of user access restrictions, no unauthorised network entities or remote equipment shall be able to view or manipulate LI data in the 3G GSN, 3G MSC Server, CSCF or the DFs.

## 8.2	IRI security

### 8.2.1	Normal operation

The transmission of the IRI shall be done in a secure manner.

When DFs are physically separate from the 3G ICEs, the X2-interface may be required by national option to support security mechanisms. Options for security mechanisms include:

-	CUG/VPN;

-	COLP;

-	CLIP;

-	authentication;

-	encryption.

### 8.2.2	Communication failure

Depending on the national law in case of communication failure IRI may be buffered in the 3G INEs. After successful transmission of IRI the whole buffer shall be deleted. It shall be possible to delete the content buffer via command or a timer, in an un-restorable fashion.

## 8.3	CC security

The transmission of the CC shall be done in a secure manner.

When DFs are physically separate from the 3G INEs, the X3-interface may be required by national option to support security mechanisms. Options for security mechanisms include:

-	CUG/VPN;

-	COLP;

-	CLIP;

-	authentication;

-	encryption.

In case of transmission failure no buffering is required within the intercepting network.

## 8.4	Security aspects of Lawful Interception billing

Billing information may be suppressed or made available at the DFs and the ADMF. Billing information for Lawful Interception shall be separated from "regular" billing data.

Billing data transmission to the Lawful Interception billing system may be done in a secure manner per national option.

In case of transmission failure billing-data shall be buffered/stored in a secure way. After successful transmission billing data shall be deleted in an un-restorable fashion.

## 8.5	Other security issues

### 8.5.1	Log files

Log files shall be generated by the ADMF, DF2, DF3, 3G MSC Server, CSCF and the 3G GSN. All log files are retrievable by the ADMF, and are maintained by the ADMF in a secure manner.

### 8.5.2	Data consistency

The administration function in the 3GMS shall be capable of performing a periodic consistency check to ensure that the target list of target identities in all involved 3G MSC Servers, CSCFs, 3G GSNs in the 3GMS and the DFs contain the appropriate target Ids consistent with the intercept orders in the ADMF. The reference data base is the ADMF data base.

# 9	Invocation of Lawful Interception for 3GPP WLAN Interworking Services

Figure 23 shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the packet data 3GPP WLAN Interworking network.

Delivery

Function 2

3G ICE

X2

X3

Delivery

Function 3

HI2

HI3

LEAs

Figure 23: Functional model for invocation of Lawful Interception for 3GPP WLAN Interworking Services

The HI2 and HI3 interfaces represent the interfaces between the LEA and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of this specification.

The delivery functions are used:

-	to convert the information on the X2-interface to the corresponding information on the HI2 interface;

-	to distribute the intercept related information to the relevant LEA(s);

-	to distribute the intercept product to the relevant LEA(s).

Interception at a WAG applies for the roaming users where the PDG is not in the visited network.

For most WLAN Interworking cases, the Packet Data Gateway (PDG) handles the bearer level interception, specifically interception of CC and IRI related to tunnel establishment and release in which case there is no need to perform interception at a WAG.  This includes the case where the PDG is in the intercepting carrier’s network (whether it be home or visited).  For the case where a visited network is to intercept WLAN related tunnel and the PDG for the tunnel is not in the visited network, the Wireless Access Gateway (WAG) is used to intercept the CC and IRI related to tunnel establishment and release. It should be noted that the CC available at the WAG may be encrypted.

## 9.1	Provision of Intercept Product - Short Message Service

LI for SMS in the 3GPP-WLAN Interworking case is described in Clause 7A.4.

## 9.2	Provision of Intercepted Content of Communications - 3GPP WLAN Interworking services

The access method for the delivering of 3GPP WLAN Interworking Intercept Product is based on duplication of packets without modification at the PDG or WAG. The duplicated packets with additional information in the header, as described in the following sections, are sent to DF3 for further delivery. Note that CC available at the WAG is likely to be encrypted.

PDG

intercepted

subscriber

other

party

Delivery

Function 3

LEA

Duplicator

of

packets

Figure 24: Configuration for interception of 3GPP WLAN Interworking product data

### 9.2.1	X3-interface

In addition to the intercepted content of communications, the following information needs to be transferred from the PDG or WAG to the DF3 in order to allow the DF3 to perform its functionality:

-	target identity;

-	correlation number;

-	time stamp - optional;

-	direction (indicates whether T-PDU is MO or MT) - optional;

-	the target location (if available in the intercepting node).

## 9.3	Provision of Intercept Related Information

Figure 25 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the PDG, WAG, or the AAA Server sends the relevant data to the DF2.

AAA Server

PDG/WAG

Other

Party

DF2

Intercept

Subject

LEMF

Figure 25: Provision of Intercept Related Information

### 9.3.1	X2-interface

The following information needs to be transferred from the PDG, WAG or the AAA server to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (IMSI, NAI, or MSISDN);

-	events and associated parameters as defined in section 9.3.2 may be provided;

-	the target location (if available);

-	Correlation number;

-	Quality of Service (QoS) identifier (if available).

The IRI should be sent to DF2 using a reliable transport mechanism.

### 9.3.2	3GPP WLAN Interworking LI Events and Event Information

The following events are applicable to AAA Server:

-	I-WLAN Access Initiation;

-	I-WLAN re-authentication,

-	I-WLAN Access Termination;

-	I-WLAN Tunnel Establishment;

-	I-WLAN Tunnel Disconnect;

-	Start of Intercept with I-WLAN Communication Active;

The following events are applicable to the PDG and WAG:

-	I-WLAN Tunnel Establishment;

-	I-WLAN Tunnel Disconnect;

-	Start of Intercept with I-WLAN Communication Active.

A set of possible elements as shown below is used to generate the events. Information associated with the events are transmitted from the PDG, WAG or AAA server to DF2.

NOTE:	Some of these parameters apply to the PDG or WAG and some apply to the AAA server.  Parameters sent from the PDG, WAG or AAA server is dependent on what is available at the network element.

Table 3: Information Events for WLAN Interworking Event Records

| Element | PDG | AAA Server |
| --- | --- | --- |
| Observed MSISDN MSISDN of the target subscriber (monitored subscriber). | Available, see TS 29.234 | Available, see TS 29.234 |
| Observed NAI NAI of the target subscriber (monitored subscriber). | Not available | Available, see TS 29.234 |
| Observed IMSI IMSI of the target subscriber (monitored subscriber). | Available, see TS 24.234 | Available, see TS 29.234 |
| Event type Description which type of event is delivered: I-WLAN Access Initiation, I-WLAN Access Termination, I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active. | Available from ICE | Available from ICE |
| Event date Date of the event generation in the PDG or the AAA server. | Available from ICE | Available from ICE |
| Event time Time of the event generation in the PDG or the AAA server. Timestamp shall be generated relative to the PDG or AAA server internal clock. | Available from ICE | Available from ICE |
| WLAN UE Local IP address The WLAN UE Local IP addess of observed party. The WLAN UE Local IP address field specified in TS 24.234 and IETF RFC 2409, represents the IPv4/IPv6 address of the WLAN UE in the WLAN AN. It is an address used to deliver the packet to a WLAN UE in a WLAN AN. Note that this address might be dynamic. | Available, see TS 24.234 and IETF RFC 2409 | Not available |
| WLAN UE MAC address The WLAN MAC address of the target subscriber. Note that this address might be dynamic and the validity of the MAC Address is outside of the scope of 3GPP. | Not available | Available, see TS 29.234 |
| WLAN UE Remote IP address The WLAN UE Remote IP addess of observed party. The WLAN UE Remote IP address field specified in TS 24.234, represents the IPv4/IPv6 address of the WLAN UE in the network being accessed by the WLAN AN. It is an address used in the data packet encapsulated by the WLAN UE-initited tunnel and is the source address used by applications in the WLAN UE. Note that this address might be dynamic. | Available, see TS 24.234 | Not available |
| WLAN Access Point Name The W-APN of the access point. | Available, see TS 24.234 | Available, see TS 29.234 |
| WLAN Operator Name The name of the WLAN operator name serving the target subscriber. | Not available | Available, see TS 29.234 |
| WLAN Location Name The name of the location of the WLAN serving the target subscriber (e.g., string like "coffee shop" or "airport", etc.). | Not available | Available, see TS 29.234 |
| WLAN Location Information Location Information regarding the WLAN as provided in RADIUS or DIAMETER signalling exchanged with the AAA server. | Not available | Available, see TS 29.234 |
| Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records. In case of the AAA server, the Correlation Number is only used to correlate IRI records. | Generated for LI by PDG | Generated for LI by AAA server |
| Network Element Identifier Unique identifier for the element reporting the ICE. | Generated for LI by PDG | Generated for LI by AAA server |
| Initiator The initiator of the request either the network or the WLAN UE. | Generated for LI by PDG | Generated for LI by AAA server |
| NAS IP/IPv6 address The IP or IPv6 address of the NAS in the WLAN. | Not available | Available, see TS 29.234 |
| Visited PLMN ID Identity of the visited PLMN to which the user is terminating their WLAN tunnels or through which the user is establishing their WLAN tunnels. | Not available | Available, see TS 29.234 |
| Session Alive Time The amount of time in seconds during which the target subscriber can be registered for WLAN access. | Not available | Available, see TS 29.234 |
| Failed access reason Provides the reason for why a WLAN access attempt failed ("Authentication Failed"). | Not available | Available from ICE |
| Session termination reason Provides a reason for why a WLAN access session is terminated. | Not available | Available, see TS 29.234 |
| Failed tunnel establishment reason Provides a reason for why a WLAN tunnel establishment failed ("Authentication failed" or "Authorization failed"). | Available from ICE | Available from ICE |
| NSAPI Network layer Service Access Point Identifier The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane. This is a optional parameter to help DF/MF and LEA's to distinguish between the sending mobile access networks | Optional available according 23.234 Annex F; defined 29.060 7.7.17 | Not available |

Table 3a: Information Events for WLAN Interworking Event Records - WAG

| Element | WAG |
| --- | --- |
| Observed MSISDN MSISDN of the target subscriber (monitored subscriber). | Available, see 3GPP TS 29.234 |
| Observed IMSI IMSI of the target subscriber (monitored subscriber). | Available, see 3GPP TS 29.234 |
| Event type Description which type of event is delivered: I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active. | Available from ICE |
| Event date Date of the event generation in the PDG/WAG or the AAA server. | Available from ICE |
| Event time Time of the event generation in the PDG/WAG or the AAA server. Timestamp shall be generated relative to the PDG/WAG or AAA server internal clock. | Available from ICE |
| WLAN UE IP address The WLAN UE IP addess of observed party. The WLAN UE IP address field contains the IPv4/IPv6 address (specified by 3GPP TS 29.234) of the WLAN UE tunnel endpoint as seen by the WAG. Note that this address might be dynamic. | Available, see 3GPP TS 29.234 |
| WLAN PDG Tunnel Endpoint IP address The WLAN PDG Tunnel Endpoint IP address field contains the IPv4/IPv6 address of the PDG (as specified in 3GPP TS 29.234) as seen by the WAG. Note that this address might be dynamic. | Available, see 3GPP TS 29.234 |
| WLAN Access Point Name The W-APN of the access point. | Available, see 3GPP TS 29.234 |
| Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records. | Generated for LI by WAG |
| Network Element Identifier Unique identifier for the element reporting the ICE. | Generated for LI by WAG |
| NAS IP/IPv6 address The IP or IPv6 address of the NAS in the WLAN. | Available, see 3GPP TS 29.234 |
| Tunnel Protocol The Tunnel Protocol as defined in the Routing-Policy AVP in 3GPP TS 29.234. | Available, see 3GPP TS 29.234 |
| Source Ports The list or range of source ports as specified in the Routing-Policy AVP provided by the AAA server in 3GPP TS 29.234. | Available, see 3GPP TS 29.234 |
| Destination Ports The list or range of destination ports as specified in the Routing-Policy AVP provided by the AAA server in 3GPP TS 29.234. | Available, see 3GPP TS 29.234 |
| Session Alive Time The amount of time in seconds during which the target subscriber can be registered for WLAN access. | Available, see 3GPP TS 29.234 |

## 9.4	Structure of I-WLAN Events

### 9.4.1	I-WLAN Access Initiation

For I-WLAN Access Initiation including I-WLAN re-authentication, for both  I-WLAN Access Initiation-event is generated. The elements, shown in Table 4, will be delivered to the DF2, if available, by the AAA server.

Table 4: I-WLAN Access Initiation – AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Network Element Identifier

WLAN Operator Name

WLAN Location Name

WLAN Location Information

NAS IP/IPv6 Address

WLAN UE MAC Address

Visited PLMN ID

Session Alive Time

Failed Access reason

### 9.4.2	WLAN Access Termination

For WLAN Access Termination or the immediate purging of a user from a WLAN access, a WLAN access termination-event is generated. The elements, shown in Table 5, will be delivered to the DF2, if available, by the AAA server.

Table 5: I-WLAN Access Termination – AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Network Element Identifier

WLAN Operator Name

WLAN Location Name

WLAN Location Information

NAS IP/IPv6 Address

WLAN UE MAC Address

Session Termination reason

### 9.4.3	I-WLAN Tunnel Establishment

For I-WLAN Tunnel Establishment, a I-WLAN tunnel establishment-event is generated. The elements, shown in Table 6, 6a, and Table 7, will be delivered to the DF2 if available, by the PDG, WAG or AAA server, respectively.

Table 6: I-WLAN Tunnel Establishment - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN UE Local IP address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

Failed tunnel establishment reason

NSAPI (optional)

Table 6a: I-WLAN Tunnel Establishment - WAG

Observed MSISDN

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

WLAN UE IP address

WLAN PDG Tunnel Endpoint IP address

WLAN Access Point Name

NAS IP/IPv6 address

Tunnel Protocol

Source Ports

Destination Ports

Session Alive Time

Network Element Identifier

Table 7: I-WLAN Tunnel Establishment - AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN Access Point Name

Network Element Identifier

Visited PLMN ID

Failed tunnel establishment reason

### 9.4.4	I-WLAN Tunnel Disconnect

At I-WLAN Tunnel Disconnect, a I-WLAN tunnel disconnect event is generated. The elements, shown in Table 8, 8a, and Table 9, will be delivered to the DF2, if available, by the PDG, WAG or AAA server, respectively.

Table 8: I-WLAN Tunnel Disconnect - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN UE Local IP Address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

Initiator (optional)

Table 8a: I-WLAN Tunnel Disconnect – WAG

Observed MSISDN

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

WLAN UE IP address

WLAN PDG Tunnel Endpoint IP address

WLAN Access Point Name

NAS IP/IPv6 address

Tunnel Protocol

Source Ports

Destination Ports

Network Element Identifier

Table 9: I-WLAN Tunnel Disconnect - AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

Tunnel address of observed party

WLAN Access Point Name

Network Element Identifier

Initiator (optional)

### 9.4.5	Start of Intercept with I-WLAN Communication Active

This event will be generated if interception for a target is started and if the target has one or more active I-WLAN Access sessions or one or more I-WLAN Tunnels established. The elements, shown in Table 10,10a, and Table 11, will be delivered to the DF2, if available, by the PDG, WAG or AAA server, respectively.

Table 10: Start of Intercept with I-WLAN Communication Active - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation Number

WLAN UE Local IP Address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

Table 10a: Start of Intercept with I-WLAN Communication Active – WAG

Observed MSISDN

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

WLAN UE IP address

WLAN PDG Tunnel Endpoint IP address

WLAN Access Point Name

NAS IP/IPv6 address

Tunnel Protocol

Source Ports

Destination Ports

Session Alive Time

Network Element Identifier

Table 11: Start of Intercept with I-WLAN Communication Active - AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation Number

WLAN Access Point Name

Network Element Identifier

WLAN Operator Name

WLAN Location Name

WLAN Location Information

NAS IP/IPv6 address

Visited PLMN ID

# 10	Interception of Multimedia Broadcast/MultiCast Service (MBMS)

MBMS provides video or similar streamed services via either point to point multicast or cell broadcast mechanisms between an operator content server (BM-SC) and UEs as defined in TS 23.246 [20]. This section details the stage 2 Lawful Interception requirements for MBMS.

NOTE:	Generic Broadcast services where the UE receives the broadcast in IDLE mode and there is no subscription relationship between the UE and the BM-SC are out of scope. In addition 3rd party BM-SC services where the operator is not responsible for content encryption and subscription management are out of scope.

Figure 10.1 shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the MBMS Services.

Delivery

Function 2

3G ICE

X2

X3

Delivery

Function 3

HI2

HI3

LEAs

Figure 10.1: Functional model for invocation of Lawful Interception for MBMS Services

## 10.1	Provision of Content of Communications

Interception of the content of communications for MBMS services if available, may be provided by the underlying transport bearer interception functionality (e.g. GSN, PDG or NGN network) and is therefore subject to the current transport bearer interception functionality detailed in other parts of this specification.

## 10.2	Provision of Intercept Related Information

Figure 10.2 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the BM-SC shall send the relevant data to the DF2.

intercepted

subscriber

Delivery

Function 2

LEMF

BM

-

SC

Figure 10.2: Provision of Intercept Related Information

### 10.2.1	X2-interface

The following information needs to be transferred from the BM-SC to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clauses 10.3.2 may be provided;

-	For Further Study:- Encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

### 10.2.2	MBMS LI Events and Event Information

Intercept Related Information (Events) are necessary are necessary for the following;

- Service Joining.

- Service Leaving

- Start of Interception with Service Active

- Subscription Activation

- Subscription Modification

- Subscription Termination

Events shall include changes resulting from direct communication between the UE and BM-SC and off-line subscription changes (e.g. changes made by operator customer services on behalf of the subscriber).

A set of possible elements as shown in Table 10.2.2 are used to generate the events.

Table 10.2.2: Information Events for MBMS Event Records

Element

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed Other Identity Other Identity of the target subscriber (monitored subscriber).

Event type Description which type of event is delivered:- Service Joining; Service Leaving; Subscription Activation; Subscription Modification; Subscription Termination.

Event date Date of the event generation in the BM-SC.

Event time Time of the event generation in the BM-SC. Timestamp shall be generated relative to the BM-SC server internal clock.

MBMS Subscribed Service Details of the MBMS Service to which the Target Subscriber has subscribed.

MBMS Service Joining Time Requested MBMS Service Joining Time

MBMS Service Subscription List List of all users subscribed to MBMS Service to which Target Subscriber has requested Joining.

Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.

Network Element Identifier Unique identifier for the element reporting the ICE.

Initiator The initiator of the request either the UE or Off-line BM-SC access (eg customer services agent or internet).

Visited PLMN ID Identity of the visited PLMN to which the user is registered

APN Access Point Name on which this IP multicast address is defined.

Multicast/Broadcast Mode MBMS bearer service in broadcast or multicast mode

IP IP/IPv6 multicast address(multicast mode only) IP or IPv6 multicast address identifying the MBMS bearer described by this MBMS Bearer Context.

List of Downstream Nodes List of downstream nodes that have requested the MBMS bearer service and to which notifications and MBMS data have to be forwarded.

MBMS Leaving Reason Indicates whether UE initiated/requested leaving, or whether BM-SC/network terminated the Service to the UE (e.g. GSN session dropped or BM-SC subscription expired etc).

NOTE: Generation of Correlation Number is FFS.

## 10.3	Structure of MBMS Events

### 10.3.1	Service Joining

For MBMS Service Joining, a Service Joining event is generated. The elements, shown in Table 10,3.1 will be delivered to the DF2, if available, by the BM-SC. A new Service Joining Event shall be generated for each individual service joined.

Table 10.3.1: Service Joining

Observed IMSI

Event Type

Event Time

Event Date

MBMS Subscribed Service

MBMS Service Joining Time

Network Element Identifier

Initiator

IP/IPv6 Multicast Address (If Applicable)

Visited PLMN ID (If Applicable)

Multicast/Broadcast Mode

APN (If Available)

List of Downstream Nodes (If Available)

MBMS Service Subscription List (Optional)

### 10.3.2	Service Leaving

For MBMS Service Leaving, a Service Leaving event is generated. The elements, shown in Table 10.3.2 will be delivered to the DF2, if available, by the BM-SC. A new Service Leaving Event shall be generated for each individual service leaving.

Table 10.3.2: Service Leaving

Observed IMSI

Event Type

Event Time

Event Date

MBMS Subscribed Service

Network Element Identifier

Initiator

IP/IPv6 Multicast Address (If Applicable)

Visited PLMN ID (If Applicable)

MBMS Service Subscription List (Optional)

MBMS Service Leaving Reason

### 10.3.3	Start of Interception with Service Active

For Start of Interception where MBMS Service Joining has already occurred prior to start of interception, a Start of Interception with Service Active event is generated. The elements, shown in Table 10.3.3 will be delivered to the DF2, if available, by the BM-SC. A new Start of Interception with Service Active Event shall be generated for each individual service the target is subscribed to.

Table 10.3.3: Start of Interception with Service Active

Observed IMSI

Event Type

Event Time

Event Date

MBMS Subscribed Service

MBMS Service Joining Time

Network Element Identifier

Initiator

IP/IPv6 Multicast Address (If Applicable)

Visited PLMN ID (If Applicable)

Multicast/Broadcast Mode

APN (If Available)

List of Downstream Nodes (If Available)

MBMS Service Subscription List (Optional)

### 10.3.4	Subscription Activation

For MBMS Subscription Activation, a Subscription Activation event is generated. The elements, shown in Table 10.3.4 will be delivered to the DF2, if available, by the BM-SC. If Subscription Activation is performed simultaneously for more than one service, a separate event shall be generated for each service activated.

Table 10.3.4: Subscription Activation

Observed IMSI

Event Type

Event Time

Event Date

MBMS Subscribed Service

Network Element Identifier

Initiator

IP/IPv6 Address (If Applicable)

Visited PLMN ID (If Applicable)

MBMS Service Subscription List (Optional)

### 10.3.5	Subscription Modification

For MBMS Subscription Modification, a Subscription Modification event is generated. The elements, shown in Table 10.3.5, will be delivered to the DF2, if available, by the BM-SC. If Subscription Modification is performed simultaneously for more than one service, a separate event shall be generated for each service modified.

Table 10.3.5: Subscription Modification

Observed IMSI

Event Type

Event Time

Event Date

MBMS Subscribed Service

Network Element Identifier

Initiator

IP/IPv6 Address (If Applicable)

Visited PLMN ID (If Applicable)

MBMS Service Subscription List (Optional)

### 10.3.6	Subscription Termination

For MBMS Subscription Termination, a Subscription Termination event is generated. The elements, shown in Table 10.3.6 will be delivered to the DF2, if available, by the BM-SC. If Subscription Termination is performed simultaneously for more than one service, a separate event shall be generated for each service performed.

Table 10.3.6: Subscription Modification

Observed IMSI

Event Type

Event Time

Event Date

MBMS Subscribed Service

Network Element Identifier

Initiator

IP/IPv6 Address (If Applicable)

Visited PLMN ID (If Applicable)

MBMS Service Subscription List (Optional)

# 11	IMS Conference Services

## 11.1	Background for IMS Conference Services

The entire clause 11 is a national option and is subject to national regulations. The covered cases are where the conference services are in the domain of the intercepting operator. The following cases are covered.

- A target’s conference call is the subject of interception. This may be where the intercept subject is the head of the conference. IRI and CC for this conference is reported. The following are examples of information that is reported.

  - For example, the starting and ending of a conference as well as any parties joined or removed from the conference call are reported.

  - Reporting of CC for held conferences initiated by the intercept subject.

- A conference that itself is directly the target of interception.  This case is applicable only provided that the conference is identified by a proper identity for LI in IMS domain (Conference URI or Conference Factory URI).  The IRI and CC for this conference is reported.

  - For example, the starting and ending or a conference as well as any parties joined or removed from the conference call are reported.

The case when an intercept subject joins an associate’s conference is for further study.

The key elements for interception of conference services are the AS/MRFC and MRFP. IRI associated with the conference services that are to be intercepted is reported by the AS/MRFC while the CC associated with the conference service is reported by the MRFP.

## 11.1A	Start of Interception for IMS Conference Services

Interception (as defined in 11.1) for IMS Conference Services is started when the first of any one of the following occurs:

-	When a target requests that a conference be created

-	When a target successfully provisions a conference

-	When a target provisioned or requested conference is started (i.e., the first party is joined to the conference)

-	When a conference that is a target of interception is started (i.e., the first party is joined to the conference)

-	When interception is activated (on a conference or a conference owner) during an ongoing conference

-	When parties have joined a conference and communication is started or enabled by the conference server in cases where the conference is a target of interception or when it is a target’s conference.

If the target of interception has provisioned or requested a conference to be created, interception on IMS Conference Services shall begin regardless whether the target of interception has joined the conference. Interception of IMS Conference Services shall continue if the target of interception is on hold and the conference continues.

## 11.2	Provision of Intercepted Content of Communication – IMS Conference Services

The access method for the delivery of IMS conference services intercept content of communication (CC) is based on duplication of packets without modification at the MRFP for conferences that are to be intercepted. The duplicated packets with additional information in the header, as described in the following sections, are sent to DF3 for further delivery. For a target’s conference call held by the target, the MRFP duplicates the CC for conference call held by the target, in accordance with national regulations.  For a conference call that is the target of interception, the MRFP duplicates the CC for the conference.

NOTE:	There is an issue of combined versus separated delivery. With combined delivery, one method for intercepting the CC would be to create a virtual conference port (not visible to others) through which a copy of the combined CC could be passed over the X3 interface (Y conferees means 1 content stream).  With the separated delivery approach, each conferee’s connection to the conference would need to be intercepted and passed over the X3 interface (Y conferees, means Y pairs of bi-directional content streams).

Figure 11.1: Configuration for interception of IMS Conference Services CC

### 11.2.1	X3-interface

In addition to the intercepted content of communications, the following information may need to be transferred from the MRFP to the DF3 in order to allow the DF3 to perform its functionality:

-	identity used for interception;

-	correlation number;

NOTE 1:	Information passed between the MRFC and MRFP for correlation has to uniquely identify the mixing of associated media streams for a conference distinct from any other mixing or media handling.  An example is how H.248 uses a context identifier to do this.

-	the identity of source (i.e., conference party identity) of a media stream;

-	time stamp - optional;

-	direction (incoming or outgoing stream) – optional.

NOTE 2:	When the media is delivered in a mixed format, the identity of the media stream source may be unknown.

## 11.3	Provision of Intercept Related Information for IMS Conference Service

Figure 11.2 shows the transfer of intercept related information to the DF2. If an event for / from or associated with a conference server occurs, the AS/MRFC sends the relevant data to the DF2.

NOTE:	Reporting of non-transmission related actions of a target’s subscriber controlled input (e.g., signalling "mute" commands) is for further study.

AS/MRFC

Other

Party

DF2

Intercept

Subject

LEMF

Other

Party

…

Figure 11.2: Provision of Intercept Related Information for IMS Conferencing

### 11.3.1	X2-interface

The following information may need to be transferred from the AS/MRFC to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (IMPU, IMPI, Conference URI);

-	events and associated parameters as defined in section 11.3.3 "Structure of Conference Events" may be provided;

-	Correlation number;

-	Bandwidth and media descriptions (e.g., as associated with SDP negotiation) associated with the parties’ bearer connection to the conference.

The IRI should be sent to DF2 using a reliable transport mechanism.

### 11.3.2	IMS Conference Events and Event Information

The following events are applicable to AS/MRFC:

-	Start of Conference

-	Party Join;

-	Party Leave;

-	Conference Bearer Modification;

-	Start of Intercept on an Active Conference;

-	End of Conference;

-	Creation of Conference;

-	Update of Conference.

NOTE 1:	Reporting of Floor Control events from the MRFP is FFS.

A set of possible elements as shown below that may be reported with the events. Information associated with the events is transmitted from the AS/MRFC server to DF2.

Table 11.3.1 Information Elements for Conference Events

Element

Observed IMPU IMS Public User identity (IMPU) of the target subscriber (monitored subscriber). In some cases, this identity may not be observed by the MRFC. Also see Note 1.

Observed IMPI IMS Private User identity (IMPI) of the target subscriber (monitored subscriber). In some cases, this identity may not be observed by the MRFC. Also see Note 1.

Observed Other Identity Target Identifier with the NAI of the target subscriber (monitored subscriber).

Event Type Description which type of event is delivered: Start of Conference, Party Join, Party Leave, Start of Intercept on an Active Conference, Conference End.

Event Date Date of the event generation in the AS/MRFC.

Event Time Time of the event generation in the AS/MRFC server. Timestamp shall be generated relative to the AS/MRFC internal clock.

Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.

Network Element Identifier Unique identifier for the element reporting the ICE.

Initiator The initiator of a request, for example, the target, the network, a conferee.

Join Party ID Identity of the party successfully joining or attempting to join the conference.

Leave Party ID Identity of the party leaving or being requested to leave the conference.

List of Potential Conferees Identifies each of the parties to be invited to a conference or permitted to join the conference (if available).

Observed Conference URI A URI associated with the conference being monitored.

Temporary Conference URI A temporarily allocated URI associated with a conference being monitored.

List of Conferees Identifies each of the conferees currently on a conference (e.g., via SIP URI or TEL URL).

Failed Conference Start Reason Provides a reason for why a conference start attempt failed.

Failed Party Join Reason Provides a reason for why a party join attempt failed.

Party Leave Reason Provides a reason for the party leaving.

Failed Party Leave reason Provides a reason for why a party leave attempt failed.

Conference End Reason Provides a reason for why the conferece ended.

Potential Conference Start Date and Time The expected start date and time of the conference, if start time information is configured in the system.

Potential Conference End Date and Time The expected end date and time of the conference, if such end information is configured in the system.

Recurrence Information Information indicating the recurrence pattern for the event as configured for the created conference.

Identity(ies) of Conference Controller Identifies the parties that have control privileges on the conference, if such information is configured in the system.

Bearer Modify ID Identifies the party modifying a conference bearer.

Failed Bearer Modify Reason Provides a reason for a bearer modification attempt failed.

Failed Conference End Reason Provides a reason why a conference end attempt failed.

Join Party Supported Bearers Identifies the bearer types supported by the party joining the conference.

List of Waiting Conferees Identifies each of the parties that have called into a conference but have not yet joined.

Media Modification Identifies how the media was modified (i.e., added, removed, changed)

Parties Affected by Bearer Modification Identifies all conference party identities affected by the bearer modification.

Supported Bearers Identifies all bearer types supported by a conferee in a conference.

Update Type Indicates what update was done to a conference (e.g., update List of Potential Conferees, update of Start Time, update of End Time, Update of Recurrence Information, Cancellation of Conference, etc.).

Editor’s Note:  We should consider whether H.248 Context Identifier should be added to help correlate CII and CC

NOTE 2:	In most cases, either the IMPU or IMPI may be available, but not necessarily both.

### 11.3.3	Structure of Conference Events

#### 11.3.3.1	Start of Conference

For the start of a conference, a Start of Conference-event is generated in the following cases:

-	When a target provisioned or requested conference or a conference that is the target of interception is started. The conference is started when the first party is joined to the conference.;

-	When a conference that is a target of interception or when a target provisioned or requested conference fails to start.

The fields, shown in Table 11.3.2, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.2. Start of Conference

Observed IMPU

Observed IMPI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

List of Potential Conferees

List of Conferees

List of Waiting Conferees

Supported Bearers

Observed Conference URI

Temporary Conference URI

Failed Conference Start Reason

#### 11.3.3.2	Party Join

A Party Join-event is generated in the following cases:

-	When a party successfully joins the target’s conference or a conference that is the target of interception.

-	When a party unsuccessfully attempts to join the target’s conference or a conference that is the target of interception.

The fields, shown in Table 11.3.3, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.3 Party Join

Observed IMPU

Observed IMPI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Join Party ID

Join Party Supported Bearers

Initiator (of the Party Join request)

Observed Conference URI

Temporary Conference URI

Failed Party Join Reason (e.g., not available)

#### 11.3.3.3	Party Leave

A Party Leave-event is generated in the following cases:

-	When a party leaves a target’s conference or a conference that is the target of interception. This includes situations where the party simply disconnects themselves from the conference (hang up), the party’s connection to the conference is broken (e.g., party leaves wireless coverage area), and where the party’s connection to the conference is forcefully terminated due to another party’s drop request or operator policy.

-	When a party unsuccessfully attempts to drop another party from the conference. This applies to all the conferencing scenarios described earlier.

The fields, shown in Table 11.3.4, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.4 Party Leave

Observed IMPU

Observed IMPI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Leave Party ID

Supported Bearers (of Leaving Party)

Initiator (of the Party Leave request)

Observed Conference URI

Temporary Conference URI

Party Leave Reason – see Note.

Failed Party Leave Reason

NOTE:	A party could drop off the conference for normal reasons (e.g., just hang up) or could be removed by a conference controller.

#### 11.3.3.3A	Conference Bearer Modification

A Conference Bearer Modification-event is generated for the following cases:

-	When a party to a conference successfully modifies (i.e., add, remove, change) a bearer stream in the conference;

-	When a party to a conference unsuccessfully attempts to modify (i.e., add, remove, change) a bearer stream in the conference.

The fields, shown in Table 11.3.4A, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.4A Conference Bearer Modification

Observed IMPU

Observed IMPI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Observed Conference URI

Temporary Conference URI

Bearer Modify ID

Media Modification

Parties Affected by Bearer Modification

Failed Bearer Modify Reason

#### 11.3.3.4	Start of Intercept on an Active Conference

A Start of Intercept on an Active Conference-event (a conference with at least one party) is generated for the following cases:

-	When interception is activated during an ongoing conference call.

The fields, shown in Table 11.3.5, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.5 Start of Intercept with an Active Conference

Observed IMPU

Observed IMPI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

List of Conferees

Supported Bearers

Observed Conference URI

Temporary Conference URI

#### 11.3.3.5	Conference End

When a conference is terminated, a Conference End-event is generated in the following cases:

-	When a target provisioned or requested conference is terminated.  This occurs when the last party on the conference leaves or the conference is terminated by the conference server;

-	When there is an unsuccessful attempt to terminate a target provisioned or requested conference or a conference that is the target of interception.

The fields, shown in Table 11.3.6, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.6 End of Conference

Observed IMPU

Observed IMPI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Initiator (e.g., target, network, conferee) – see Note

Observed onference URI

Temporary Conference URI

Conference End Reason

Failed Conference End Reason

NOTE:	The initiator can indicate that the decision to end the conference was the target or conferee, if the target or conferee sends an explicit command to end the conference.  It could be the network, if it determines the time length for the conference is ended.

#### 11.3.3.6	Creation of Conference

When a conference is created, a Creation of Conference-event is generated in the following cases:

-	When a target successfully provisions or requests a conference to be created.

This event is applicable provided that at least one of the two identities (IMPU, IMPI) are available at the AS/MRFC. Other scenarios, such as in case the creation is done via a web interface and the IMPU/IMPI cannot be seen are outside the scope of this specification.

The fields, shown in Table 11.3.7, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.7 Creation of Conference

Observed IMPU

Observed IMPI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

List of Potential Conferees (if available)

Observed Conference URI

Temporary Conference URI

Potential Conference Start Date and Time (if available) – See Note 1

Potential Conference End Date and Time (if available) – See Note 1

Recurrence Information – See Note 2.

Identity(ies) of Conference Controller

NOTE 1:	This information is statically provisioned information and is not correlated to the timestamp requirements for LI.

NOTE 2:	Recurrence information indicates the frequency or pattern of recurrence of the created conference.

#### 11.3.3.7	Update of Conference

When a conference is updated, an Update of Conference-event is generated in the following cases:

-	When a target successfully provisions or requests a conference to be updated (e.g., changes to List of Potential Conferees, Start Time, End Time, Recurrence Information, or Cancellation of Conference).

This event is applicable provided that at least one of the two identities (IMPU, IMPI) are available at the AS/MRFC. Other scenarios, such as in case the update is done via a web interface and the IMPU/IMPI cannot be seen are outside the scope of this specification.

The fields, shown in Table 11.3.8, will be delivered to the DF2, if available, by the AS/MRFC.

Table 11.3.8 Update of Conference

Observed IMPU

Observed IMPI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Update Type

List of Potential Conferees (if available)

Observed Conference URI

Temporary Conference URI

Potential Conference Start Date and Time (if available) – See Note 1

Potential Conference End Date and Time (if available) – See Note 1

Recurrence Information – See Note 2.

Identity(ies) of Conference Controller

NOTE 1:	This information is statically provisioned information and is not correlated to the timestamp requirements for LI.

NOTE 2:	Recurrence information indicates the frequency or pattern of recurrence of the created conference.

# 12	Lawful Interception for Evolved Packet System

## 12.1	LI functional architecture for EPS

In addition to the reference configurations applicable to PS interception, the following figures contain the reference configuration applicable for the lawful interception in the EPS nodes ([22], [23]):

LEMF

Mediation

Function

Delivery

Function 2

Mediation uuu

Function

ADMFuuu

X2

X1_2

X1_1

HI1

HI2

MME

Figure 12.1.1:	MME Intercept configuration

Figure 12.1.2:	HSS Intercept configuration

Figure 12.1.3:	S-GW, PDN-GW Intercept configuration

The definition of the LI functional entities (ADMF, DF, MF, LEMF) and interfaces (X, HI) is the same as for 3G as given in chapter 4.

Procedures for LI activation, deactivation and interrogation are the same as for 3G as given in chapter 5, provided that:

- the 3G ICE is replaced by the EPS node;

- the proper target identity applicable to EPS node is used.

When the SGSN is used as node in the Evolved Packet System, to support 2G/3G access and mobility between E-UTRAN and pre-E-UTRAN 3GPP radio access technologies, it is subjected to all the related PS requirements specified throughout this document.

When the ePDG and the AAA server are used as node in the Evolved Packet System, to support untrusted Non-3GPP IP Access, they are subjected to all the requirements specified in this document for the PDG and the AAA server for the case of I-WLAN interworking.

## 12.2	Functional requirements for LI in case of E-UTRAN access and GTP based S5/S8.

The target identities for interception at the MME, HSS, S-GW and PDN-GW are IMSI, MSISDN and ME (Mobile Equipment) Identity.

NOTE 1:	Details about information included in the ME Identity and the relationship with IMEI needs to be considered. The term Mobile Equipment Identity is used in this text according to [22] so as to indicate that the EPC should support multiple equipment identity formats (e.g. those from 3GPP2, WiMAX, etc) as well as the IMEISV.

NOTE 2: 	In case of local breakout the PDN Gateway is in the VPLMN. In this case LI relevant information in the H-PLMN might be available at the H-PCRF.  Interception at the H-PCRF is FFS.

NOTE 3:	In case the ME Identity and/or MSISDN is not available in a node, interception based on the missing identity is not applicable at that node.

NOTE 4:	MSISDN is a possible identity available in the EPC nodes, which may be provided by the HSS to the MME and then forwarded to the S-GW/PDN-GW.

As the MME only handles control plane, interception of Content of Communication is applicable only at the S-GW and PDN-GW. As the HSS only handles signaling, interception of Content of Communication is not applicable at this node.

LI in the PDN-GW is a national option.

For the delivery of the CC and IRI the S-GW and/or, per national option PDN-GW provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one EPS bearer.

The correlation number shall be generated by using existing parameters related to the EPS bearer.

NOTE 5:	If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

Location Dependent Interception for EPC is FFS.

### 12.2.1	Provision of Intercept Related Information

Intercept Related Information (Events) shall be sent at the Mobile Entity Attach, Mobile Entity Detach, Tracking Area Update, Bearer activation (valid for both Default and Dedicated bearer), Start of Intercept with bearer active, Start of Interception with E-UTRAN attached UE, Bearer Modification, Bearer Deactivation, Serving Evolved Packet System (applicable to the HSS), UE requested PDN connectivity, UE requested PDN disconnection, and UE requested bearer resource modification.

Serving Evolved Packet System event reporting is a national option.

#### 12.2.1.1		X2-interface

The following information needs to be transferred from the EPS nodes or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (IMSI, MSISDN, ME identity);

-	events and associated parameters as defined in clause 12.2.1.2 and 12.2.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	correlation number;

-	Quality of Service (QoS) information (if available);

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

#### 12.2.1.2		Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. Details are described in the following clause. The events for interception are configurable (if they are sent to DF2) in the EPC nodes or the HSS and can be suppressed in the DF2. The network procedures for which the events are generated are defined in [22].

The following events are applicable to the MME:

-	Attach;

-	Detach;

- 	Tracking Area Update;

- 		UE requested PDN connectivity;

- 	UE Requested PDN disconnection;

- 	Start of interception with E-UTRAN attached UE.

The following events are applicable to the Serving GW and PDN GW:

-	Bearer activation (valid for both Default and Dedicated bearer);

-	Start of intercept with bearer active;

-	Bearer modification;

-	Bearer deactivation;

-	UE Requested Bearer Resource Modification.

The following events are applicable to the HSS:

- Serving Evolved Packet System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option.

Observed MSISDN MSISDN of the target subscriber (monitored subscriber).

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed ME Id ME Id of the target subscriber (monitored subscriber); when it coincides with the IMEI, it shall be checked for each activation over the radio interface.

Event type Indicates which type of event is delivered: Attach, Detach, Tracking Area Update, UE requested PDN connectivity, UE Requested PDN disconnection, UE Requested Bearer Resource Modification, Bearer activation, Start of intercept with bearer active, Start of interception with E-UTRAN attached UE, Bearer deactivation, Bearer modification, Serving Evolved Packet System.

Event date Date of the event generation in the ICE.

Event time Time of the event generation in the ICE. Timestamp shall be generated relative to ICE internal clock.

PDN Type The parameter is applicable to the MME only and provides the IP version (IPv4, IPv4/IPv6, IPv6) requested by the UE.

PDN Address Allocation The parameter is applicable to the S-GW and PDN-GW; it provides the IP version (IPv4, IPv4/IPv6, IPv6) and IP address(es) allocated for the UE.

Protocol Configuration Options Are used to transfer parameters between the UE and the PDN-GW (e.g. Address Allocation Preference by DHCP).

Attach type Indicates the type of attach (may carry indication of handover in case of mobility with non-3GPP access).

Location Information Location Information is the Tracking Area Identity (TAI), TA List assigned to the UE, E-CGI and/or location area identity that is present at the node at the time of event record production. In case of Tracking Area Update event, the last visited TAI of the UE may be applicable.

PDN address(es) The UE IP address(es) for the PDN connection.

APN When provided by the MME, the parameter carries the Access Point Name provided by the UE. When provided by the S-GW/PDN-GW, it is the Access Point Name used for the connection.

RAT type The Radio Access Type

APN-AMBR The Aggregate Maximum Bit Rate for the APN.

Handover indication Provides information from the GTPv2 protocol that the procedure is triggered as part of a handover.

Procedure Transaction Identifier Identifies a set of messages belonging to the same procedure; the parameter is dynamically allocated by the UE.

EPS bearer identity An EPS bearer identity uniquely identifies an EPS bearer for one UE accessing via E-UTRAN. The EPS Bearer Identity is allocated by the MME.

Bearer activation/deactivation type Indicates the type of bearer being activated/deactivated, i.e. default or dedicated.

Linked EPS bearer identity Indicates, in case of dedicated bearer, the EPS bearer identity of the default bearer.

Initiator The initiator of the procedure, either the network or the UE.

Switch off indicator Indicates whether a detach procedure is due to a switch off situation or not.

Detach type Parameter sent by the network to the UE to indicate the type of detach.

Traffic Flow Template (TFT) The EPS bearer traffic flow template (TFT) is the collection of all packet filters associated with that EPS bearer.

Traffic Aggregate Description (TAD) The TAD consists of the description of the packet filter(s) for the traffic flow aggregate.

Serving MME address The address of the serving MME.

Old Location Information Location Information of the subscriber before Tracking Area Update.

Correlation Number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Failed attach reason Reason for failed attach of the target subscriber.

Failed bearer activation reason Reason for failed bearer activation for the target subscriber.

Failed Bearer Modification reason The reason for failure of Bearer Modification.

IAs The observed Interception Areas.

Bearer Deactivation cause The cause of deactivation of the PDP context.

EPS Bearer QoS This field indicates the Quality of Service associated with the Bearer procedure.

Request type Indicates the type of request in an UE requested PDN connectivity, i.e. initial request or handover.

### 12.2.2	X3-interface

The access method for the delivering of S-GW and/or PDN-GW Intercept Product is based on duplication of packets without modification at the S-GW and/or PDN-GW. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.

S-GW, PDN-GW

intercepted

subscriber

other

party

Delivery

Function 3

LEA

Duplicator

of

packets

Figure 12.2.2.1: Configuration for interception of S-GW/PDN-GW product data

In addition to the intercepted content of communication, the following information needs to be transferred from the S-GW and/or the PDN-GW to the DF3 to perform its functionality:

- target identity;

- correlation number;

- time stamp (optional);

- direction (indicates whether T-PDU is MO or MT) – optional;

- the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

### 12.2.3	EPS related events

#### 12.2.3.1	Attach

When an attach activation is generated from the mobile an attach event is generated by the MME. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME Id

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

Failed attach reason

IAs (if applicable)

PDN Type

APN

Protocol Configuration Options

Attach type

EPS bearer identity

#### 12.2.3.2	Detach

For detach a detach-event is generated. The following elements will be delivered by the MME to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME Id

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

IAs (if applicable)

Detach initiator

Switch off indicator

Detach type

#### 12.2.3.3	Bearer activation

When a bearer activation is generated a bearer activation-event is generated by the S-GW/PDN-GW. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME Id

RAT type (note 1)

PDN address allocation (note 1)

Event Type

Event Time

Event Date

Correlation number

APN (Access Point Name) (note 1)

Bearer activation Type (default, dedicated)

Network Element Identifier

Location Information

Failed bearer activation reason

IAs (if applicable)

EPS bearer QoS (note 2)

APN-AMBR (note 3)

EPS bearer id (NSAPI)

Protocol Configuration Options

Initiator

Procedure Transaction Identifier

Linked EPS bearer identity (note 2)

Traffic Flow Template(s) (TFT) (note 1)

Handover indication

NOTE 1: Only in case of default bearer activation; the parameter includes both PDN type and PDN address(es).

NOTE 2: In case of unsuccessful default bearer activation, the parameter carries the requested EPS bearer QoS, otherwise it carries the EPS bearer QoS associated to the established bearer.

NOTE 3: In case of unsuccessful default bearer activation, the parameter carries the subscribed APN-AMBR, otherwise it carries the APN-AMBR used for the established bearer.

#### 12.2.3.4	Bearer deactivation

When a bearer deactivation is generated a bearer deactivation-event is generated by the S-GW/PDN-GW. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME Id

Event Type

Event Time

Event Date

Correlation number

Bearer deactivation Type (default, dedicated)

Network Element Identifier

Location Information

IAs (if applicable)

EPS bearer id

Initiator

Procedure Transaction Identifier

Bearer deactivation Cause (note )

In case all the bearers belonging to the same PDN connection are released at the same time, one event shall be sent for each bearer.

NOTE:	Cause can be present e.g. in case of inter S-GW TAU, when the new S-GW sends a bearer deactivation request to the old S-GW.

#### 12.2.3.5	Bearer modification

When a bearer modification is detected, a bearer modification event shall be generated. These elements will be delivered by the S-GW/PDN-GW to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME Id

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Location Information

IAs (if applicable)

Initiator

EPS Bearer QoS (Note 1)

EPS bearer id

Procedure Transaction Identifier

RAT type

APN-AMBR (Note 2)

Traffic Flow Template(s) (TFT)

Handover indication

Failed Bearer Modification reason

NOTE 1:	In case of unsuccessful default bearer modification, the parameter carries the requested EPS bearer QoS, otherwise it carries the EPS bearer QoS associated to the modified bearer.

NOTE 2:	In case of unsuccessful default bearer modification, the parameter carries the subscribed APN-AMBR, otherwise it carries the APN-AMBR used for the modified bearer.

#### 12.2.3.6		Start of interception with active bearer

This event will be generated if interception for a target is started and if the target has at least the default bearer active. If more then one bearer is active, for each of them an event record is generated. The parameters which are defined for bearer activation (see related section) will be sent, if available, by the S-GW/PDN-GW to the DF2.

As an option, in case the event is sent due to a change of the involved S-GW, the new S-GW may provide as additional parameter, the "old location information". However, the absence of this information does not imply that interception has not started in the old location S-GW for an active bearer.

#### 12.2.3.7	Tracking Area Update

For each TA update an update-event with the elements about the new location is generated. New MME shall send the event, and the old MME may optionally send the event as well. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME

Event Type

Event Time

Event Date

Network Element Identifier

Location Information (only for the new MME)

Old Location Information (only for the old MME)

IAs (if applicable)

Failure reason

#### 12.2.3.8	Serving Evolved Packet System

The Serving Evolved Packet System report event is generated at the HSS, when the HSS has detected that the intercept subject has roamed. The elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME

Event Type

Event Time

Event Date

Network Element Identifier

Serving MME Address

#### 12.2.3.9	UE requested PDN connectivity

When a PDN connectivity is requested from the mobile to allow multiple PDN connections ([22]), an UE requested PDN connectivity event is generated by the MME. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

APN

Request type

PDN type

Failed reason

IAs (if applicable)

Protocol Configuration Options

EPS bearer identity

#### 12.2.3.10	UE requested PDN disconnection

When a PDN disconnection is requested from the mobile to request for disconnection from one PDN ([22]), an UE requested PDN disconnection event is generated by the MME. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

IAs (if applicable)

Linked EPS bearer identity

#### 12.2.3.11	UE requested Bearer Resource Modification

When UE requested Bearer Resource Modification [22] is detected at the S-GW/PDN-GW, an UE requested Bearer Resource Modification event is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

IAs (if applicable)

Linked EPS bearer identity

EPS bearer identity

Procedure Transaction Identifier

EPS bearer QoS

Traffic Aggregate Description

Failed Bearer Modification reason

Protocol Configuration Options

#### 12.2.3.12	Void

#### 12.2.3.13	Start of interception with E-UTRAN attached UE

This event will be generated if interception for a target is started and if the target is already E-UTRAN attached. If there are multiple PDN connections active for the target then for each them an event report is generated.

These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME id

Event Type

Event Time

Event Date

Network Element Identifier

Location Information

APN

PDN type

IAs (if applicable)

EPS bearer identity of the default bearer

## 12.3	Functional requirements for LI in case of E-UTRAN access and PMIP based S5/S8 interfaces

Functional requirements for LI in the MME, S-GW and HSS do not differ from the ones applicable to the case of GTP based S5-S8 interfaces, as specified in clause 12.2 and subclauses.

LI in the PDN-GW is a national option.

Interception in the PDN-GW shall be based on one or more of NAI, MSISDN, IMEI.

For the delivery of the CC and IRI, the PDN-GW provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one IP-CAN session. However, when different protocols (i.e. GTP and PMIP) are used in the network, different values can be generated by different nodes.

The correlation number shall be generated by using existing parameters related to the IP-CAN session.

NOTE:	If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

### 12.3.1	Provision of Intercept Related Information

Intercept Related Information (Events) shall be sent at attach/tunnel activation, detach/tunnel deactivation, start of interception with active PMIP tunnel, PMIP session modification, PDN-GW initiated PDN-disconnection, UE requested PDN connectivity, Serving Evolved Packet System.

Serving Evolved Packet System reporting is a national option. Requirements on the HSS specified in section 12.2 and subsections apply also to the case in which S5/S8 interfaces are PMIP based.

#### 12.3.1.1	X2 interface

The following information needs to be transferred from the PDN-GW to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clause 12.3.1.2 and 12.3.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception; (FFS)

-	correlation number;

-	Quality of Service (QoS) information, if available;

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

#### 12.3.1.2	Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the PDN-GW and can be suppressed in the DF2. The network procedures for which the events are generated are defined in [23].

The following events are applicable to the PDN-GW:

-	PMIP Attach/tunnel activation;

-	PMIP Detach/tunnel deactivation;

-	PMIP session modification

- 	Start of interception with active PMIP tunnel;

-	PMIP PDN-GW initiated PDN-disconnection.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option.

Observed MN NAI The Network Access Identifier of the Mobile Node (target identity).

Observed MSISDN MSISDN of the target subscriber (monitored subscriber).

Observed IMEI IMEI of the target subscriber (monitored subscriber)

Event type Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, PMIP Session modification, Start of interception with active PMIP tunnel, PMIP PDN-GW initiated PDN disconnection.

Event time Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

Event date Date of the event generation in the ICE.

Correlation number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Lifetime Indicates the lifetime of the tunnel; it is set to a nonzero value in the case of registration; is set to zero in case of deregistration.

Failed attach reason Reason for the failed attach/tunnel deactivation of the target subscriber.

Access technology type Indicates the Radio Access Type.

Handover indicator Provides information on whether the procedure is triggered as part of a handover.

APN The Access Point Name used for the connection.

UE address info Includes one or more IP addresses allocated to the UE.

Additional Parameters Additional information provided by the UE, such as protocol configuration options.

PDN address(es) The UE IP address(es) for the PDN connection.

Revocation trigger Indicates the reason which triggered the PDN-GW initiated PDN-disconnection procedure

Serving Network Identifies the serving network the UE is attached to

DHCP v4 Address Allocation Indication Indicates that DHCPv4 is to be used to allocate the IPv4 address to the UE

Location Information Provides, if received from the PCRF, location information of the target subscriber.

### 12.3.2	X3-interface

The access method for the delivering of PDN-GW Intercept Product is based on duplication of packets without modification at the PDN-GW. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.

PDN-GW

intercepted

subscriber

other

party

Delivery

Function 3

LEA

Duplicator

of

packets

Figure 12.3.2.1: Configuration for interception of PDN-GW product data

In addition to the intercepted content of communication, the following information needs to be transferred from the PDN-GW to the DF3 to perform its functionality:

- target identity;

- correlation number;

- time stamp (optional);

- direction (indicates whether T-PDU is MO or MT) – optional;

- the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

### 12.3.3	LI events for E-UTRAN access with PMIP-based S5 or S8

#### 12.3.3.1	Initial E-UTRAN Attach and UE PDN requested connectivity with PMIP-based S5 or S8

When the E-UTRAN Attach or UE requested PDN connectivity is detected at the PMIP based PDN-GW, a PMIP attach/tunnel activation event shall be generated by the PDN-GW. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Lifetime

Failed attach reason

Access Technology Type

Handover Indicator

APN

UE Address Info

Additional Parameters

Serving Network

DHCPv4 Address Allocation Indication

Location information

#### 12.3.3.2	Detach and PDN disconnection for PMIP-based S5/S8

When the Detach or PDN disconnection is detected at the PMIP based PDN-GW, a PMIP detach/tunnel deactivation event shall be generated by the PDN-GW. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

APN

Additional Parameters

Failed reason

Location information

#### 12.3.3.3	Start of interception with active tunnel for PMIP based S5/S8

This event shall be generated by the PDN-GW if interception for a target is started and if the target has an active PMIP tunnel. If more then one connection is active, for each of them an event record is generated. The parameters which are defined for PMIP attach/tunnel activation (see related section) will be sent, if available, by the PDN-GW to the DF2.

#### 12.3.3.4	Dedicated Bearer Procedures for E-UTRAN Access with PMIP-based S5/S8

All the procedures can be intercepted at the S-GW according to the requirements specified for LI in case of GTP based S5/S8.

PDN-GW is not involved in these procedures, except for the case of PDN-GW initiated PDN-disconnection Procedure.

#### 12.3.3.5	PDN-GW initiated PDN-disconnection Procedure

When a PDN-GW initiated PDN-disconnection procedure is detected, a PMIP PDN-GW initiated PDN-disconnection event shall be generated by the PDN-GW. The following elements will be delivered to the DF2:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

PDN Address(es)

Revocation trigger

Location information

#### 12.3.3.6	PMIP Session modification

When a session modification is detected at the PDN-GW, a PMIP Session modification event shall be generated by the PDN-GW. The following elements will be delivered to the DF2:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Additional Parameters

Failed reason

Serving Network

Handover indicator

DHCPv4 Address Allocation Indication

Location information

## 12.4	Functional requirements for LI in case of trusted non-3GPP IP access

Differently to what happens in E-UTRAN case, in which the user traffic passes through the S-GW and then through the PDN-GW, in case of access to the network through S2a (trusted Non-3GPP access), the PDN-GW is, in case of non roaming and local breakout, located in the VPLMN and is the only possible ICE in 3GPP network.

Interception in the S-GW and PDN-GW shall be based on IMSI or NAI.

NOTE:	The NAI may be a temporary ID, therefore the use of  IMSI is recommended.

For the delivery of the CC and IRI, the S-GW and/or PDN-GW provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

The correlation number is unique in the whole PLMN and is used to correlate CC with IRI and the different IRI's of one IP-CAN session. However, when different protocols (i.e. GTP and PMIP) are used in the network, different values can be generated by different nodes

The correlation number shall be generated by using existing parameters related to the IP-CAN session.

NOTE:	If interception has been activated for both parties of the Packet Data communication both CC and IRI will be delivered for each party as separate intercept activity.

### 12.4.1	Provision of Intercept Related Information

Intercept Related Information (Events) shall be sent at attach/tunnel activation on interfaces s2a and s2c, session modification, detach/tunnel deactivation, start of interception with active tunnel, PDN-GW reallocation upon initial attach on s2c, PDN GW initiated resource allocation Deactivation on s2a, Serving Evolved Packet System.

Serving Evolved Packet System reporting is a national option.

#### 12.4.1.1	X2-interface

The following information needs to be transferred from the S-GW, PDN-GW or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clause 12.4.1.2 and 12.4.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception; (FFS)

-	correlation number;

-	Quality of Service (QoS) information, if available;

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

#### 12.4.1.2		Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the S-GW, PDN-GW or the HSS and can be suppressed in the DF2.

The following events are applicable to the S-GW:

-	PMIP attach/tunnel activation;

-	PMIP detach/tunnel deactivation;

-	PMIP session modification;

-	Start of interception with active PMIP tunnel;

The following events are applicable to the PDN-GW:

-	PMIP attach/tunnel activation;

-	PMIP detach/tunnel deactivation;

-	PMIP session modification;

-	Start of interception with active PMIP tunnel;

-	MIP registration/tunnel activation;

-	DSMIP registration/tunnel activation;

-	DSMIP session modification;

-	MIP deregistration/tunnel deactivation;

-	DSMIP deregistration/tunnel deactivation;

-	Start of interception with active MIP tunnel;

-	Start of interception with active DSMIP tunnel;

-	DSMIP HA Switch;

-	PMIP Resource Allocation Deactivation;

-	MIP Resource Allocation Deactivation.

The following event is applicable to the HSS:

-	Serving Evolved Packet System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option.

Observed MN NAI The Network Access Identifier of the Mobile Node (target identity).

Observed IMSI The IMSI of the target subscriber (monitored subscriber)

Event type Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, PMIP session modification, Start of interception with active PMIP tunnel, MIP registration/tunnel activation, DSMIP registration/tunnel activation, DSMIP session modification, MIP deregistration/tunnel deactivation, DSMIP deregistration/tunnel deactivation, Start of interception with active MIP tunnel, Start of interception with active DSMIP tunnel, DSMIP HA Switch, PMIP resource Allocation Deactivation, MIP Resource Allocation Deactivation, Serving Evolved Packet System.

Event time Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

Event date Date of the event generation in the ICE.

Correlation number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Lifetime Indicates the lifetime of the tunnel; must be set to a nonzero value in the case of registration or lifetime extension; is set to zero in case of deregistration.

Failed attach reason Reason for the failed attach/tunnel deactivation of the target subscriber.

Session modification failure reason Reason for a failure of a session modification attempt for the target subscriber

Access technology type Indicates the Radio Access Type.

Handover indicator Provides information on whether the triggered as part of a handover.

APN The Access Point Name used for the connection.

UE address info Includes one or more IP addresses allocated to the UE.

Additional Parameters Additional information provided by the UE, such as protocol configuration options.

PDN address(es) The UE IP address(es) for the PDN connection.

Home address Contains the UE Home IP address.

Home Agent address Contains the IP address of the Home Agent.

Requested IPv6 Home Prefix The IPv6 Home Prefix requested by the UE.

IPv6 home prefix The IPv6 home prefix assigned by the PDN GW to the UE.

Care of Address The Local IP address assigned to the UE by the Access Network, used as Care of Address for DSMIPv6 over S2c reference point.

HSS/AAA address The address of the HSS/AAA triggering the PDN-GW reallocation.

Target PDN-GW address The address of the PDN-GW which the UE will be reallocated to.

Revocation trigger Contains the cause for the revocation procedure.

Foreign domain address The relevant IP address in the foreign domain.

Visited network identifier An identifier that allows the home network to identify the visited network [24]

Location Information Location information of the target subscriber, e.g. 3GPP2-BSID [26]. Provided if available from the PCRF.

Initiator The initiator of the procedure, either the network or the UE.

### 12.4.2	X3-interface

The access method for the delivering of S-GW and/or PDN-GW Intercept Product is based on duplication of packets without modification at the S-GW and/or PDN-GW. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.

S-GW, PDN-GW

intercepted

subscriber

other

party

Delivery

Function 3

LEA

Duplicator

of

packets

Figure 12.4.2.1: Configuration for interception of S-GW/PDN-GW product data

n addition to the intercepted content of communication, the following information needs to be transferred from the S-GW and/or the PDN-GW to the DF3 to perform its functionality:

- target identity;

- correlation number;

- time stamp (optional);

- direction (indicates whether T-PDU is MO or MT) – optional;

- the target location (if available) or the IAs in case of location dependent interception.

NOTE:	location dependent interception for EPC is FFS.

### 12.4.3	LI events for trusted Non-3GPP IP access

#### 12.4.3.1	Initial Attach and PDN connection activation with PMIPv6 on S2a

When the Attach or PDN connectivity activation is detected over PMIP at the S-GW, PDN-GW, a PMIP attach/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Lifetime

Failed attach reason

Access Technology Type

Handover Indicator

APN

UE Address Info

Additional Parameters

Location Information

#### 12.4.3.2	Initial Attach and PDN connection activation procedures with MIPv4 FACoA on S2a

When the Attach or PDN connectivity activation is detected over MIP at the PDN-GW, a MIP registration/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Lifetime

Failed attach reason

Home Address

Care of Address

Home Agent Address

APN

NOTE: As the S-GW has no Home Agent function, the event is not applicable to the S-GW. The use of MIPv4 in roaming case requires Local Breakout (PDN-GW in VPLMN), so LI in the PDN-GW is mandatory in order to intercept in this scenario.

#### 12.4.3.3	Initial Attach and PDN connection activation procedures with DSMIPv6 over S2c

When the Attach or PDN connectivity activation is detected over DSMIP at the PDN-GW, a DSMIP registration/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Correlation number

Network Element Identifier

Lifetime

Requested IPv6 home prefix

IHome address

APN

Care of Address

Failed attach reason

#### 12.4.3.4	Detach and PDN disconnection with PMIPv6 on S2a

When a Detach or PDN disconnection is detected over PMIP at the S-GW, PDN-GW, a PMIP detach/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Correlation number

Network Element Identifier

APN

Initiator

Location Information

#### 12.4.3.5	Detach and PDN disconnection with MIPv4 FACoA

When a Detach or PDN disconnection is detected over MIP at the PDN-GW, a MIP deregistration/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Correlation number

Network Element Identifier

Home Address

Home Agent Address

Care of address

Initiator

#### 12.4.3.6	Detach and PDN disconnection with DSMIPv6 on S2c

When a Detach or PDN disconnection is detected over DSMIP at the PDN-GW, a DSMIP deregistration/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Correlation number

Network Element Identifier

Home Address

Initiator

#### 12.4.3.7	PDN-GW reallocation upon initial attach on s2c

When a PDN GW reallocation procedure is detected by the PDN-GW, a DSMIP HA Switch event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Network Element Identifier

HSS/AAA address

Target PDN-GW address

#### 12.4.3.8	PDN GW initiated Resource Allocation Deactivation with S2a PMIP

When a PDN GW initiated resource allocation deactivation is detected by the S-GW/PDN-GW, a PMIP Resource Allocation Deactivation event shall be sent. The following elements will be delivered to DF2 if available

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Network Element Identifier

Revocation trigger

UE address info

Correlation number

Location Information

#### 12.4.3.9	PDN GW initiated Resource Allocation Deactivation with S2a MIP v4

When a PDN GW initiated resource allocation deactivation is detected, a MIP Resource Allocation Deactivation event shall be sent. The following elements will be delivered to DF2 if available

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Network Element Identifier

Home Address

Foreign domain address

Correlation number

#### 12.4.3.10	Serving Evolved Packet System

The Serving Evolved Packet System report event is generated at the HSS, when the HSS has detected that the intercept subject has roamed. The elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME Identity

Event Type

Event Time

Event Date

Network Element Identifier

Visited Network Identifier

#### 12.4.3.11 	Start of interception with active tunnel

When interception is started at the S-GW, PDN-GW and the target has an already active tunnel, a start of interception with active tunnel shall be generated. Separate events are defined for the different protocols. The event shall be detected by the same node for which tunnel activation reporting is applicable and reported with the same parameters required for the specific protocol (PMIP, MIP, DSMIP) tunnel activation event, as defined in the related sections. One event shall be sent for each active tunnel.

#### 12.4.3.12	PMIP session modification

When a session modification is detected at the S-GW/PDN-GW, a PMIP session modification event shall be generated by the S-GW/PDN-GW. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed MSISDN

Observed IMEI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Additional Parameters

Session modification failure reason

Serving Network

Handover indicator

DHCPv4 Address Allocation Indication

Location information

#### 12.4.3.13	DSMIP session modification

When the session modification is detected over DS-MIPv6 at the PDN-GW, a DSMIP session modification event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Lifetime

Session modification failure reason

Home address

Care of Address

APN

Requested IPv6 Home Prefix

## 12.5	Functional requirements for LI in case of untrusted non-3GPP IP access

The e-PDG and the AAA server are subjected to all the requirements specified in this document for PDG and AAA server for the case of I-WLAN interworking.

Interception in the PDN-GW is a national option.

Interception in the PDN-GW shall be based on IMSI or NAI.

NOTE:	The NAI may be a temporary ID, therefore the use of  IMSI is recommended.

For the delivery of the CC and IRI, the PDN-GW provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

### 12.5.1	Provision of Intercept Related Information

Intercept Related Information (Events) shall be sent at attach/tunnel activation on interfaces s2b and s2c, detach/tunnel deactivation, session modification, start of interception with active tunnel, Serving Evolved Packet System.

Serving Evolved Packet System reporting is a national option.

#### 12.5.1.1	X2-interface

The following information needs to be transferred from the PDN-GW or the HSS to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clause 12.5.1.2 and 12.5.3 may be provided;

-	the target location (if available) or the IAs in case of location dependent interception; (FFS)

-	correlation number;

-	Quality of Service (QoS) information, if available;

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

The IRI should be sent to DF2 using a reliable transport mechanism.

#### 12.5.1.2	Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. The events for interception are configurable (if they are sent to DF2) in the PDN-GW or the HSS and can be suppressed in the DF2.

The following events are applicable to the PDN-GW:

-	PMIP attach/tunnel activation;

-	PMIP detach/tunnel deactivation;

-	PMIP session modification;

- 	Start of interception with active PMIP tunnel;

-	DSMIP registration/tunnel activation;

-	DSMIP deregistration/tunnel deactivation;

-	DSMIP session modification;

-	Start of interception with active DSMIP tunnel;

-	DSMIP HA Switch;

-	PMIP Resource Allocation Deactivation.

The following events is applicable to the HSS:

-	Serving Evolved Packet System

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from the nodes to DF2. Available IEs from this set of elements as shown below can be extended in the nodes, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option.

Observed MN NAI The Network Access Identifier of the Mobile Node (target identity).

Observed IMSI The IMSI of the target subscriber (monitored subscriber).

Event type Indicates which type of event is delivered: PMIP attach/tunnel activation, PMIP detach/tunnel deactivation, Start of interception with active PMIP tunnel, DSMIP registration/tunnel activation, DSMIP deregistration/tunnel deactivation, Start of interception with active DSMIP tunnel, DSMIP HA Switch, PMIP resource Allocation Deactivation, Serving Evolved Packet System.

Event time Time of the event generation in the ICE. Time stamp shall be generated relative to ICE internal clock.

Event date Date of the event generation in the ICE.

Correlation number The correlation number is used to correlate CC and IRI.

Network Element Identifier Unique identifier for the ICE reporting the event.

Lifetime Indicates the lifetime of the tunnel; must be set to a nonzero value in the case of registration or lifetime extension; is set to zero in case of deregistration.

Failed attach reason Reason for the failed attach/tunnel deactivation of the target subscriber.

Session modification failure reason Reason for a failure of a session modification attempt for the target subscriber

Access technology type Indicates the Radio Access Type.

Handover indicator Provides information on whether the triggered as part of a handover.

APN The Access Point Name used for the connection.

UE address info Includes one or more IP addresses allocated to the UE.

Additional Parameters Additional information provided by the UE, such as protocol configuration options.

Home Agent address Contains the IP address of the Home Agent.

Care of Address The Local IP address assigned to the UE by the Access Network, used as Care of Address for DSMIPv6 over S2c reference point.

HSS/AAA address The address of the HSS/AAA triggering the PDN-GW reallocation.

Target PDN-GW address The address of the PDN-GW which the UE will be reallocated to.

Revocation trigger Contains the cause for the revocation procedure.

Foreign domain address The relevant IP address in the foreign domain.

Visited network identifier An identifier that allows the home network to identify the visited network [24].

Requested IPv6 Home Prefix The IPv6 Home Prefix requested by the UE.

IPv6 home prefix The IPv6 home prefix assigned by the PDN GW to the UE.

Home address Contains the UE Home IP address.

### 12.5.2	X3-interface

The access method for the delivering of PDN-GW Intercept Product is based on duplication of packets without modification at the PDN-GW. The duplicated packets with additional information in a header are sent to DF3 for further delivery to the LEA.

PDN-GW

intercepted

subscriber

other

party

Delivery

Function 3

LEA

Duplicator

of

packets

Figure 12.5.2.1: Configuration for interception of PDN-GW product data

In addition to the intercepted content of communication, the following information needs to be transferred from the PDN-GW to the DF3 to perform its functionality:

- target identity;

- correlation number;

- time stamp (optional);

- direction (indicates whether T-PDU is MO or MT) – optional;

- the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

### 12.5.3	LI events for untrusted Non-3GPP IP access

#### 12.5.3.1	Initial Attach and PDN connection activation with PMIPv6 on S2b

In the VPLMN, LI shall be done at the ePDG according to LI requirements for I-WLAN; no additional requirement applies to the S-GW for this case.

When the attach or PDN connectivity activation is detected over PMIP at the PDN-GW, a PMIP attach/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Lifetime

Failed attach reason

Access Technology Type

Handoff Indicator

APN

UE Address Info

Additional Parameters

#### 12.5.3.2	Initial attach and PDN connection activation for S2c in untrusted non-3GPP IP access

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN.

When the attach or PDN connectivity activation is detected over DS-MIPv6 at the PDN-GW, a DSMIP registration/tunnel activation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Lifetime

Failed attach reason

Home address

Care of Address

APN

Requested IPv6 Home Prefix

#### 12.5.3.3	UE/ePDG-initiated Detach Procedure and UE Requested PDN disconnection with PMIP

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN; no additional requirement applies to the S-GW for this case.

When the detach or UE requested PDN disconnection is detected over PMIP at the PDN-GW, a PMIP detach/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

APN

#### 12.5.3.4	Detach and PDN Disconnection for S2c in Un-trusted Non-3GPP IP access

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN.

When the detach or PDN disconnection is detected over DS-MIPv6 at the PDN-GW, a DSMIP deregistration/tunnel deactivation event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Home address

Initiator

Care of Address

#### 12.5.3.5	Serving Evolved Packet System

The Serving Evolved Packet System report event is generated at the HSS, when the HSS has detected that the intercept subject has roamed. The elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed ME Identity

Event Type

Event Time

Event Date

Network Element Identifier

Visited Network Identifier

#### 12.5.3.6 	Start of interception with active tunnel

When interception is started at the PDN-GW and the target has an already active tunnel, a start of interception with active tunnel shall be generated. Separate events are defined for the specific protocol (PMIP, DSMIP). The parameter applicable to the tunnel activation event,  as defined in the related sections, will be delivered to the DF2 if available. One event shall be sent for each active tunnel.

#### 12.5.3.7	PDN-GW reallocation upon initial attach on s2c

When a PDN GW reallocation procedure is detected by the PDN-GW, a DSMIP HA Switch event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Network Element Identifier

HSS/AAA address

Target PDN-GW address

#### 12.5.3.8	PDN GW initiated Resource Allocation Deactivation with S2b PMIP

When a PDN GW initiated resource allocation deactivation is detected, a PMIP Resource Allocation Deactivation event shall be sent. The following elements will be delivered to DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Date

Event Time

Network Element Identifier

Revocation trigger

UE address info

Correlation number

#### 12.5.3.9	PMIP session modification

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN; no additional requirement applies to the S-GW for this case.

When a session modification is detected at the PDN-GW, a PMIP session modification event shall be generated by the PDN-GW. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Network Element Identifier

Correlation number

Lifetime

UE Address Info

Access Technology Type

Additional Parameters

Session failure modification reason

Handover indicator

#### 12.5.3.10	DSMIP session modification

In the VPLMN, LI shall be done at the ePDG according to LI requirements for PDG for I-WLAN.

When the session modification is detected over DS-MIPv6 at the PDN-GW, a DSMIP session modification event shall be generated. The following elements will be delivered to the DF2 if available:

Observed MN NAI

Observed IMSI

Event Type

Event Time

Event Date

Correlation number

Network Element Identifier

Lifetime

Session failure modification reason

Home address

Care of Address

APN

Requested IPv6 Home Prefix

## 12.6	Functional requirements for LI in case of Handovers between E-UTRAN and CDMA2000 Accesses.

When an handover is performed from CDMA2000 Access to E-UTRAN, the MME shall intercept the attach event received from the HRPD AN based on IMSI.

Interception at S-GW and PDN-GW shall be done according to the requirements given in section 12.2 or 12.3 and related subsections, depending on the protocol used over the S5/S8 interface.

## 12.7	Functional requirements for LI in case of interworking between SGSN and EPS nodes over S4/S12 interfaces

The SGSN and the HSS are subjected to the requirements applicable to these nodes for PS interception, as specified throughout this document.

The S-GW is subjected to the requirements specified in section 12.2 and subsections. The applicable events shall be reported also when received from the SGSN over S4 interface. CC shall be also reported when received over S4/S12 interfaces. The network procedures for which the events applicable to the S-GW, defined in section 12.2 and subsections, are generated when the S-GW is connected over S4/S12 interfaces to a SGSN are defined in [10].

The PDN-GW is subjected to the requirements specified in section 12.2 or 12.3 and related subsections, depending on the protocol used on S5/S8 interfaces, which are applicable also to the case in which the PDN-GW is involved for a target subscriber for which a S4 based SGSN is used.

## 12.8	Functional requirements for LI in case of interworking between SGSN and PDN-GW over Gn/Gp interfaces

According to [10] and [22] a PDN-GW may provide a Gn/Gp interface for interworking with the SGSN. When this interface is provided, from LI perspective the PDN-GW acts as a GGSN towards the involved SGSN. In this case, in addition to the requirements specified in this clause, all the requirements specified by this document for the GGSN are applicable to the PDN-GW.

The PDN-GW shall use the same correlation number in records when the PDP context/EPS bearer modification signalling is detected due to the handover between different accesses involving a Gn/Gp interface (i.e. from E-UTRAN to 2G/3G and vice versa).  After the handover, the PDN-GW shall report the events applicable to the new access and continue to use the same correlation number inside the same PDP context/EPS bearer.

The SGSN is subjected to the requirements applicable to this node for PS interception, as specified throughout this document.

Annex A (informative):
Information flows for Lawful Interception invocation of circuit switched services

The following figures show the information flows for the invocation of Lawful Interception for various types of calls. The figures show some of the basic signalling messages of the target calls and the events on the X2 and X3-interfaces. The call control messages to and from the network are shown for informational purposes only; some of them may not be sent or may be combined in certain networks. The handling of the bearers for the basic calls is not shown. The bearer points are established in a manner to minimise content loss without delaying the call to the target subscriber. The bearer establishment to agency will be in parallel or immediately following the bearer establishment to the target subscriber. The flows portray both forward and backward bearer establishment and release to the agency.

# A.1	Mobile originated circuit switched calls

Figure A.1 shows the interception of a basic mobile originated circuit switched speech or data call where the originating mobile (A) is the target for interception. B is not necessarily also a mobile subscriber and resides on a different exchange.

Figure A.1: Interception of mobile originated circuit switched calls

In figure A.1 the result (answer) of the set-up of the stublines is not shown. This assumes no special action is taken in case of failure.

# A.2	Mobile terminated circuit switched calls

Figure A.2 shows the interception of a basic mobile terminated circuit switched speech or data call where the terminating mobile (B) is the target for interception. A is not necessarily also a mobile subscriber and resides on a different exchange.

Figure A.2: Interception of mobile terminated circuit switched calls

# A.3	Call hold / call waiting

Figures A.3 and A.4 show the interception of calls involving call hold / call waiting. Figure A.3 covers the case where one pair of stublines is used per target, figure A.4 covers the case where a separate pair of stublines is used for each target call. The mobile that receives the waiting call (A) is the target for interception.

Figure A.3: Interception of call hold / call waiting - stublines per target

Figure A.4: Interception of call hold / call waiting - stublines per target call

# A.4	Multiparty calls

Figures A.5 and A.6 show the interception of multiparty calls. Figure A.5 covers the case where one pair of stublines is used per target, figure A.6 covers the case where a separate pair of stublines is used for each target call. The mobile setting up the multiparty call (A) is the target for interception.

Figure A.5: Interception of multiparty calls - stublines per target

Figure A.6: Interception of multiparty calls - stublines per target call

# A.5	Call forwarding / call deflection

The following pictures show the information flows for the interception of forwarded calls. Information flows will be given for three typical cases of call forwarding. All other types of call forwarding / call deflection are intercepted similar to one of these.

## A.5.1	Unconditional call forwarding

Figure A.7 shows the interception of unconditionally forwarded calls. The mobile that activated unconditional call forwarding (B) is the target for interception. In this case interception will be performed at the 3G GMSC, where the Service Request Indicator (SRI) request for B is issued and subsequently the SRI response indicating that the call shall be forwarded is received.

Figure A.7: Interception of unconditional call forwarding

## A.5.2	Call forwarding on not reachable (IMSI detached)

Call forwarding on not reachable because the IMSI is detached is also handled on the 3G GMSC. Interception of this type of call forwarding is similar to interception of unconditional call forwarding.

## A.5.3	Call forwarding on busy (network determined)

Figure A.8 shows the interception of call forwarding on busy (network determined). The mobile that activated call forwarding on busy (B) is the target for interception. In this case interception will be performed at the 3G MSC where B resides, where the busy condition is detected and the call is forwarded.

Figure A.8: Interception of call forwarding on busy (network determined)

## A.5.4	Call forwarding on not reachable (no response to paging/radio channel failure)

Call forwarding on not reachable because of no response to paging or radio channel failure is also handled on the 3G MSC similar to call forwarding on busy (network determined). Interception of this type of call forwarding is therefore done in the same way (see clause A.5.3).

## A.5.5	Call forwarding on no reply

Figure A.9 shows the interception of call forwarding on no reply. The mobile that activated call forwarding on no reply (B) is the target for interception. In this case interception will be performed at the 3G MSC where B resides, where the no reply condition is detected and the call is forwarded. Initially, the interception is similar to the interception of a basic mobile terminated circuit switched speech of data call. On no reply time-out, the interception will continue on the forwarded call to C.

Figure A.9: Interception of call forwarding on no reply

In figure A.9 the release of the stublines is done after the forwarded call is released by A or C. It is a national option not to support interception of forwarded calls. In that case, the release of the stublines is done after the call is forwarded and B is no longer involved.

## A.5.6	Call forwarding on busy (user determined)/call deflection

Call forwarding on busy (user determined) and call deflection are also handled on the 3G MSC similar to call forwarding on no reply. Interception of this type of call forwarding is therefore done in the same way (see A5.5).

## A.5.7	Call waiting / call forwarding on no reply

Figures A.10 and A.11 show the interception of a call involving both call waiting and call forwarding on no reply. Figure A.10 covers the case where one pair of stublines is used per target, figure A.11 covers the case where a separate pair of stublines is used for each target call. The mobile that activated call forwarding on no reply and receives the waiting call (B) is the target for interception. In figure A.10 a new pair of stublines needs to be set up when the call is forwarded since the first pair of stublines is still used for the initial call.

Figure A.10: Interception of call waiting / call forwarding on no reply - stublines per target

Figure A.11: Interception of call waiting / call forwarding on no reply - stublines per target call

# A.6	Explicit call transfer

Figures A.12 and A.13 show the interception of explicit call transfer. Figure A.12 covers the case where one pair of stublines is used per target, figure A.13 covers the case where a separate pair of stublines is used for each target call. The mobile transferring the call (B) is the target for interception.

Figure A.12: Interception of explicit call transfer - stublines per target

Figure A.13: Interception of explicit call transfer - stublines per target call

In figures A.12 and A.13 the release of the stublines is done after the transferred call is released by A or C. It is a national option not to support interception of transferred calls. In that case, the release of the stublines is done after the call is transferred and B is no longer involved.

Annex B (informative):
Information flows for Lawful Interception invocation of GSN Packet Data services

The following figures show the information flows for the invocation of Lawful Interception for Packet Data and typical scenarios. The figures show some of the basic signalling messages of the target Packet Data communication and the events on the X2 and X3 interfaces. The dotted lines indicate signalling depending on whether CC and/or IRI information has been requested. The Gateway 3G GGSN may setup/release packet tunnels and send IRI information depending on national requirements.

The use of the Gateway 3G GGSN for interception is a national option.

# B.1	Mobile Station Attach

Figure B.1 shows the interception of a basic Mobile Station Attach where the mobile (A) is the target for interception.

Figure B.1: Interception of mobile originated Mobile Station Attachment

# B.2	Mobile Initiated Mobile Station Detach

Figure B.2 shows the interception of a Mobile Initiated Mobile Station Detach where the originating mobile (A) is the target for interception.

Figure B.2: Interception of mobile originated Mobile Station Detachment

# B.3	Network initiated Mobile Station Detach

Figure B.3 shows the interception of a network initiated (by 3G SGSN or HLR) Mobile Station Detach where the mobile (A) is the target for interception.

NOTE:	* Additional signals in case of HLR initiated.

Figure B.3: Interception of network initiated Mobile Station Detach

# B.4	Intra 3G GSN Routing Area Update

Figure B.4 shows the interception of an Intra Routing Area Update where the mobile (A) is the target for interception. The sequence is the same for the combined RA / LA Update procedure but additional signalling is performed between the current 3G SGSN and the prior 3G SGSN before the Routing Area Update Accept message is sent to the MS.

Figure B.4: Interception of an Intra Routing Area Update

# B.5	Inter 3G GSN Routing Area Update

Figure B.5 shows the interception of an Inter Routing Area Update where the mobile (A) is the target for interception. The sequence is the same for the combined RA / LA Update procedure but additional signalling is performed between the 3G GSN, HLR and the old 3G GSN before the Routing Area Update Accept message is sent to the MS. In case of PDP context not being active less signalling is required.

Figure B.5: Interception of an Inter Routing Area Update

# B.6	PDP Context Activation

Figure B.6 shows the interception of a PDP Context activation where the mobile (A) is the target for interception. The sequence for a network initiated PDP Context activation is analogous but is preceded by the 3G GSN sending a Request PDP Context Activation to the MS.

Figure B.6: Interception of a PDP Context Activation

# B.7	Start of interception with PDP context active

A tunnel is established to DF3 and an event is sent to DF2.

# B.8	MS initiated PDP Context Deactivation

Figure B.7 shows the interception of a MS initiated PDP Context deactivation where the mobile (A) is the target for interception.

Figure B.7: Interception of a PDP Context Deactivation

# B.9	Network initiated PDP Context Deactivation

Figure B.8 shows the interception of a Network initiated PDP Context deactivation where the mobile (A) is the target for interception. The 3G GGSN may send, (depending on national requirements) the PDP Context deactivation and release the Packet Data tunnel after the Delete PDP Context Response has been sent or received, (signalling between the 3G SGSN and the 3G GGSN is not shown here).

Figure B.8: Interception of a Network initiated PDP Context Deactivation

# B.10	SMS

Figures B.9a and B.9b show the interception of a Mobile-terminated SMS. Figures B.10a and B.10b show the interception of a Mobile-originated SMS. In all the scenarios, the mobile subscriber (A) is the target for interception.

Figure B.9a: MT-SMS interception after 3G SGSN receives notification of SMS delivery to MS(A)

Figure B.9b: MT-SMS interception after 3G SGSN receives SMS from SMSC

Figure B.10a: MO-SMS interception after 3G SGSN receives notification of SMS delivery from SMSC

Figure B.10b: MO-SMS interception after 3G SGSN receives SMS from MS(A)

Annex C (informative):
Information flows for the invocation of Lawful Interception for Packet Data with multimedia

The following figures show the information flows for the invocation of Lawful Interception for Packet Data with multimedia. The figures show some of the basic signalling messages of the target Packet Data communication and the events on the X2 interfaces. The dotted lines indicate signalling depending on whether IRI information has been requested. The figures illustrate interception in the visited network.

The figures in this annex only apply to scenarios where the P-CSCF is located in the visited network. In some operator deployment scenarios, the P-CSCF will be in the Home Network. Where the P-CSCF is located in the Home Network and UE to P-CSCF signalling encryption is applied, all SIP messages between the P-CSCF and the UE will be encrypted within the visited network and therefore plain text interception in the visited network may not be possible.

# C.1	Multimedia registration

Figures C.1.1 and C.1.2 show the intercept of the Multimedia registration for the case of visited network interception (refer to TS 23.228 clauses 5.3.2.4 and 5.3.2.5).

Figures C.1.1 and C.1.2 show the intercept of the Multimedia registration for the case of visited network interception, where the P-CSCF is located in the Visited Network (refer to TS 23.228 clauses 5.3.2.4 and 5.3.2.5).

Figure C.1.1: Intercept of Start of Multimedia Registration

Figure C.1.2: Intercept of Continuation of Multimedia Registration

NOTE:	The same SIP Registration command is used for the initial registration and any registration updates. Registration deletion request is accomplished with a Registration command that indicates a '*' contact or zero expiration time.

# C.2	Multimedia Session Establishment and Answer

Figure C2 shows the intercept of the Multimedia Establishment and Answer in the visited network, where the P-CSCF is located in the Visited Network (refer to 3G TS 23.228, clause 5.7.1).

Figure C.2 Intercept of Multimedia Establishment and Answer at Visiting Network

# C.3	Multimedia Release

Figure C.3 shows the intercept of the Multimedia Release in the visited network, where the P-CSCF is located in the Visited Network (3G TS 23.228, clause C.2.1 reference available).

Figure C.3 Intercept of Multimedia Release at Visiting Network

# C.4	Multimedia with Supplementary Service – Call Forwarding

Not defined in this release.

# C.5	Multimedia with Supplementary Service – Explicit Call Transfer

Not defined in this release.

# C.6	Multimedia with Supplementary Service – Subscriber Controlled input

Not defined in this release.

Annex D (informative):
Information flows for Lawful Interception invocation at the MGW using H.248

The following figures show the use of H.248 in setting up a bearer intercept point at the MGW.

# D.1	Mobile to Mobile call, originating side is target

Figure D.1 shows the network model for interception of a mobile-to-mobile call, where the originating mobile subscriber is the target for interception.

Figure D.2 message sequence only shows the H.248 elements related to the necessary topology, which could be used in this example.

Normal call establishment using other H.248 elements shall be in accordance with TS 23.205. 
It should be noted that other means exist with H.248 to achieve similar interception.

Figure D.1: Mobile to Mobile call originating side is target (network model)

Figure D.2: Mobile to Mobile call originating side is target

Annex G (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Cat | Subject/Comment | Old | New | WI |
|  | SA_03 |  |  | - |  | Approved at SA#6 and placed under TSG SA Change Control | 1.0.0 | 3.0.0 |  |
|  | SA_10 | SP-000625 | 0001 | - |  | Addition of parameters to the X3-Interface | 3.0.0 | 3.1.0 | Security |
| 2000-03 | SP-11 | SP-010137 | 0002 | - |  | Correction of Location information parameters in interception event records | 3.1.0 | 3.2.0 | Security |
| 2000-03 | SP-11 | SP-010146 | 0003 | - |  | Update of TS 33.107 for Release 4 - Inclusion of PS LI requirements | 3.1.0 | 4.0.0 | Security |
| 2000-06 | SP-12 | SP-010374 | 0004 | 1 | B | Update of TS 33.107 for Release 5 | 4.0.0 | 5.0.0 | SEC1-LI |
| 2001-12 | SP-14 | SP-010612 | 0010 | - | A | Start of secondary interception of an active PDP context | 4.1.0 | 5.1.0 | SEC1-LI |
| 2001-12 | SP-14 | SP-010613 | 0011 | - | C | Alignment of TS 33.107 for Release 5 Network Architecture | 4.1.0 | 5.1.0 | SEC1-LI |
| 2001-12 | SP-14 | SP-010614 | 0014 | - | A | Correct the MO-SMS and MT-SMS events | 4.1.0 | 5.1.0 | SEC1-LI |
| 2001-12 | SP-14 | SP-010615 | 0016 | - | A | Source of PDP context initiation | 4.1.0 | 5.1.0 | SEC1-LI |
| 2002-03 | SP-15 | SP-020109 | 0017 | - | B | PDP context Deactivation cause | 5.1.0 | 5.2.0 | SEC1-LI |
| 2002-03 | SP-15 | SP-020110 | 0018 | - | B | The use of H.248 in setting up a bearer intercept point at the MGW | 5.1.0 | 5.2.0 | SEC1-LI |
| 2002-03 | SP-15 | SP-020111 | 0021 | - | B | Inter-SGSN RA update with active PDP context | 5.1.0 | 5.2.0 | SEC1-LI |
| 2002-03 | SP-15 | SP-020112 | 0022 | - | B | Addition of PDP context modification Event and Transferring the QoS information element across the X2 interface | 5.1.0 | 5.2.0 | SEC1-LI |
|  | - | - | - | - | - | Change History new version corrected for SP-15 CRs | 5.2.0 | 5.2.1 | SEC1-LI |
| 2002-06 | SP-16 | SP-020345 | 0023 | - | B | Changes to 33.107 to support interception at a GGSN | 5.2.1 | 5.3.0 | SEC1-LI |
| 2002-06 | SP-16 | SP-020345 | 0024 | - | B | Addition of SMS type information | 5.2.1 | 5.3.0 | SEC1-LI |
| 2002-06 | SP-16 | SP-020345 | 0025 | - | C | Inclusion of Serving System IRI in TS 33.107 | 5.2.1 | 5.3.0 | SEC1-LI |
| 2002-09 | SP-17 | SP-020511 | 0026 | - | F | Essential clarification to the Timestamp IE | 5.3.0 | 5.4.0 | SEC1-LI |
| 2002-09 | SP-17 | SP-020511 | 0027 | - | F | Additional X3-interface parameters | 5.3.0 | 5.4.0 | SEC1-LI |
| 2002-12 | SP-18 | SP-020702 | 0028 | - | F | Event Time | 5.4.0 | 5.5.0 | SEC1-LI |
| 2002-12 | SP-18 | SP-020704 | 0029 | - | F | Essential correction to the LI events generated during inter-SGSN RAU, when PDP context is active | 5.4.0 | 5.5.0 | SEC1-LI |
| 2002-12 | SP-18 | SP-020703 | 0030 | - | F | Essential correction to the LI events generated during inter-SGSN RAU, when PDP context is active | 5.4.0 | 5.5.0 | SEC1-LI |
| 2002-12 | SP-18 | SP-030478 | 0031 | - | F | Missing QoS Parameter in IRI | 5.5.0 | 5.6.0 | SEC1-LI |
| 2003-09 | SP-21 | SP-030479 | 0032 | - | B | TEL URL for IMS interception identity (Release 6) | 5.5.0 | 6.0.0 | SEC1-LI |
| 2003-09 | SP-21 | SP-030479 | 0032 | - | D | Stereo delivery to LEMF | 5.5.0 | 6.0.0 | SEC1-LI |
| 2003-12 | SP-22 | SP-030590 | 0034 | - | F | MSISDN/IMEI clarification for GPRS interception | 6.0.0 | 6.1.0 | SEC1-LI |
| 2003-12 | SP-22 | SP-030591 | 0035 | - | F | Reporting TEL URL | 6.0.0 | 6.1.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040397 | 0036 | - | F | Correction on Network initiated Mobile Station Detach signalling flow | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040398 | 0037 | - | F | TEL-URL missing in activation of LI in the CSCFs | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040399 | 0038 | - | F | Correction on the use of session initiator parameter | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040400 | 0039 | - | F | Correction to HLR interception event name | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040401 | 0040 | - | B | Clarification for Push to talk over Cellular | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040402 | 0041 | - | F | Adding an encryption parameter to IRI across X2 interface | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040403 | 0042 | - | F | References | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-06 | SP-24 | SP-040404 | 0043 | - | F | Enhancements for the Functional Architecture chapter | 6.1.0 | 6.2.0 | SEC1-LI |
| 2004-09 | SP-25 | SP-040693 | 0044 | - | F | Correction on the use of session initiator parameter | 6.2.0 | 6.3.0 | SEC1-LI |
| 2004-09 | SP-25 | SP-040693 | 0045 | - | F | ICE (Intercepting Control Elements), INE (Intercepting Network Elements) definition | 6.2.0 | 6.3.0 | SEC1-LI |
| 2004-09 | SP-25 | SP-040693 | 0046 | - | F | Clarification to SMS interception | 6.2.0 | 6.3.0 | SEC1-LI |
| 2004-09 | SP-25 | SP-040693 | 0047 | - | F | Replace SIP URL with SIP URI | 6.2.0 | 6.3.0 | SEC1-LI |
| 2004-12 | SP-26 | SP-040850 | 0048 | - | B | Lawful Interception for WLAN Interworking | 6.3.0 | 6.4.0 | SEC1-LI |
| 2004-12 | SP-26 | SP-040850 | 0049 | - | F | 33.107 Cleanup | 6.3.0 | 6.4.0 | SEC1-LI |
| 2004-12 | SP-26 | SP-040850 | 0050 | - | B | Clarification on MMS interception | 6.3.0 | 6.4.0 | SEC1-LI |
| 2005-06 | SP-28 | SP-050256 | 0052 | - | F | Correction on the use of identities for I-WLAN lawful interception | 6.4.0 | 6.5.0 | SEC1-LI |
| 2005-06 | SP-28 | SP-050257 | 0051 | 1 | F | Clarifications for the usage of the notion of a service in distributed IP networks | 6.4.0 | 7.0.0 | SEC-LI |
| 2005-06 | SP-28 | SP-050257 | 0053 | - | C | Correlation for IMS intercption | 6.4.0 | 7.0.0 | SEC1-LI |
| 2005-09 | SP-29 | SP-050570 | 0054 | - | F | Clarifications to the RAU event | 7.0.0 | 7.1.0 | SEC1-LI |
| 2005-09 | SP-29 | SP-050570 | 0055 | - | C | Simplifications to LDI handling | 7.0.0 | 7.1.0 | SEC1-LI |
| 2005-12 | SP-30 | SP-050779 | 0054 | - | B | Start of interception for already attached UE | 7.1.0 | 7.2.0 | LI-7A |
| 2005-12 | SP-30 | SP-050763 | 0056 | - | A | Availability of IMSI at PDG | 7.1.0 | 7.2.0 | IMS2 (SEC1-LI) |
| 2006-03 | SP-31 | SP-060064 | 0057 | - | F | WLAN Interworking - Additional Details for TS 33.107 | 7.2.0 | 7.3.0 | LI-7A |
| 2006-09 | SP-33 | SP-060659 | 0058 | 1 | F | Editorial Update by Rapporteur | 7.3.0 | 7.4.0 | LI-7A |
| 2007-03 | SP-35 |  | 0060 | - | B | Stage 2 MBMS Interception | 7.4.0 | 7.5.0 | LI-7A |
| 2007-03 | SP-35 | SP-070156 | 0061 | 1 | F | SMS IRI Reporting for WLAN Interworking | 7.4.0 | 7.5.0 | LI-7A |
| 2007-06 | SP-36 | SP-070331 | 0063 | - | B | Direct Tunnel LI | 7.5.0 | 7.6.0 | LI-7A |
| 2007-06 | SP-36 | SP-070332 | 0062 | - | B | NSAPI (Network layer Service Access Point Identifier) optional in IRI | 7.6.0 | 8.0.0 | LI8 |
| 2007-09 | SP-37 | SP-070601 | 0065 | - | B | WLAN IRI at AAA for re-authentication | 8.0.0 | 8.1.0 | LI8 |
| 2007-09 | SP-37 | SP-070599 | 0064 | - | A | Stage 2 MBMS Interception | 8.0.0 | 8.1.0 | LI-7A |
| 2007-12 | SP-38 | SP-070788 | 0066 | - | C | P-CSCF IMS LI Optional | 8.1.0 | 8.2.0 | LI8 |
| 2007-12 | SP-38 | SP-070788 | 0067 | - | C | MBMS IRI Registration | 8.1.0 | 8.2.0 | LI8 |
| 2008-03 | SP-39 | SP-080172 | 0068 | 1 | D | CR on P-CSCF IMS LI Optional | 8.2.0 | 8.3.0 | LI8 |
| 2008-03 | SP-39 | SP-080172 | 0069 | 1 | D | Removing "P" suffix from references | 8.2.0 | 8.3.0 | LI8 |
| 2008-03 | SP-39 | SP-080172 | 0070 | 1 | B | Changes for Interception of IRI and CC at a WAG | 8.2.0 | 8.3.0 | LI8 |
| 2008-06 | SP-40 | SP-080262 | 0071 | - | F | CSCF SIP Event reporting | 8.3.0 | 8.4.0 | LI8 |
| 2008-06 | SP-40 | SP-080262 | 0072 | - | B | Conference Event Reporting | 8.3.0 | 8.4.0 | LI8 |
| 2008-06 | SP-40 | SP-080262 | 0073 | - | D | Editorial corrections | 8.3.0 | 8.4.0 | LI8 |
| 2008-09 | SP-41 | SP-080514 | 0074 | - | B | Updates to TS 33.107 to support LI for EPSs | 8.4.0 | 8.5.0 | LI8 |
| 2008-12 | SP-42 | SP-080762 | 0077 | - | F | Editorial corrections to 33.107 | 8.5.0 | 8.6.0 | LI8 |
| 2008-12 | SP-42 | SP-080762 | 0075 | - | F | Corrections and clarifications of LI for EPS and alignment with latest version of SAE stage 2 specs. | 8.5.0 | 8.6.0 | LI8 |
| 2008-12 | SP-42 | SP-080762 | 0076 | - | F | Clarification on 3G DT with the GGSN | 8.5.0 | 8.6.0 | LI8 |
| 2009-03 | SP-43 | SP-090133 | 0078 | - | F | Alignment with SAE stage 2 specifications approved by TSG SA#42 | 8.6.0 | 8.7.0 | LI8 |
| 2009-04 |  |  |  |  |  | Editorial correction to cover page | 8.7.0 | 8.7.1 |  |
| 2009-06 | SP-44 | SP-090272 | 0079 | - | F | Correction on UE requested bearer resource modification - Alignment with SAE stage 2 specification | 8.7.1 | 8.8.0 | LI8 |
| 2009-06 | SP-44 | SP-090272 | 0080 | - | F | Clarification on parameter APN | 8.7.1 | 8.8.0 | LI8 |
| 2009-06 | SP-44 | SP-090272 | 0081 | - | F | Clarification on the handover between 2G/3G access and E-UTRAN with Gn/Gp | 8.7.1 | 8.8.0 | LI8 |
| 2009-06 | SP-44 | SP-090272 | 0082 | - | F | Clarification on parameter PDN type | 8.7.1 | 8.8.0 | LI8 |
| 2009-09 | SP-45 | SP-090522 | 0083 | - | F | Correction on identities and parameters for LI in case of E-UTRAN access and PMIP based S5/S8 | 8.8.0 | 8.9.0 | LI8 |
| 2009-09 | SP-45 | SP-090522 | 0084 | - | F | Correction on Serving Evolved Packet System event | 8.8.0 | 8.9.0 | LI8 |
| 2009-10 | -- | -- | -- | -- | -- | Correction of misimplementation of CR0084 | 8.9.0 | 8.9.1 |  |
| 2009-12 | SP-46 | SP-090817 | 86 | - | F | Correction on events names | 8.9.1 | 8.10.0 | LI8 |
| 2009-12 | SP-46 | SP-090817 | 87 | - | F | Restoring section header 9.4.5 | 8.9.1 | 8.10.0 | LI8 |
| 2009-12 | SP-46 | SP-090817 | 88 | - | F | Correction on PDP context modification event | 8.9.1 | 8.10.0 | LI8 |
| 2009-12 | SP-46 | SP-090817 | 85 | - | F | Correction on LI correlation for S4-SGSN | 8.9.1 | 8.10.0 | LI8 |
| 2009-12 | - | - | - | - | - | Update to Rel-9 version (MCC) | 8.10.0 | 9.0.0 | - |
| 2010-06 | SP-48 | SP-100364 | 89 | - | F | Correction in IMS Conference text | 9.0.0 | 9.1.0 | LI9 |
| 2010-06 | SP-48 | SP-100253 | 90 | - | F | Reporting of Dual Stack PDP address from the SGSN | 9.1.0 | 10.0.0 | LI10 |
| 2010-10 | SP-49 | SP-100570 | 92 | - | A | Correction in IMS Conference Service X2 interface | 10.0.0 | 10.1.0 | TEI9 |
| 2010-10 | SP-49 | SP-100570 | 93 | - | A | IMS Conference Service configuration for CC interception | 10.0.0 | 10.1.0 | TEI9 |
| 2010-10 | SP-49 | SP-100481 | 91 | - | F | Unsuccessful bearer modificatio | 10.0.0 | 10.1.0 | LI10 |
| 2010-10 | SP-49 | SP-100481 | 94 | - | B | LI architecture and functions for KMS based IMS Media Security | 10.0.0 | 10.1.0 | LI10 |
| 2010-10 | -- | -- | -- | -- | -- | ToC update | 10.1.0 | 10.1.1 | -- |
| 2010-12 | SP-50 | SP-100845 | 97 | 1 | A | IMSI based activation | 10.1.1 | 10.2.0 | LI10 |
| 2010-12 | SP-50 | SP-100865 | 98 | 1 | B | MME start of interception with bearer active | 10.1.1 | 10.2.0 | LI8 |
| 2010-12 | SP-50 | SP-100865 | 103 | 1 | C | Corrections and Alignment for IMS Conferencing | 10.1.1 | 10.2.0 | LI8 |
| 2011-03 | SP-51 | SP-110023 | 104 | - | B | Location information from trusted non-3GPP access | 10.2.0 | 10.3.0 | LI10 |
| 2011-03 | SP-51 | SP-110023 | 106 | - | C | Security requirements for LI in KMS based IMS media security | 10.2.0 | 10.3.0 | LI10 |
| 2011-03 | SP-51 | SP-110023 | 111 | - | F | Initiator parameter definition | 10.2.0 | 10.3.0 | LI10 |
| 2011-03 | SP-51 | SP-110021 | 109 | - | A | IMS Conf LI 33.107 | 10.2.0 | 10.3.0 | LI8 |
| 2011-06 | SP-52 | SP-110260 | 112 | - | C | TLS and IPsec profiling for Xk interface | 10.3.0 | 10.4.0 | LI10 |
| 2011-09 | SP-53 | SP-110511 | 113 | - | B | Reporting of PMIP and DSMIP session modification | 10.4.0 | 11.0.0 | LI11 |
| 2012-03 | SP-55 | SP-12-0034 | 114 |  | F | Correction on MIP specific parameters provided over the X2 interface. | 11.0.0 | 11.1.0 | LI11 |
| 2012-06 | SP-56 | SP-120336 | 114a | 1 | C | IMSI in untrusted non-3GPP access | 11.1.0 | 11.2.0 | LI11 |
| 2012-06 | SP-56 | SP-120336 | 115 | 1 | C | SGs received location transfer over the X2 interface | 11.1.0 | 11.2.0 | LI11 |
| 2012-06 | SP-56 | SP-120336 | 116 | 2 | F | UE IP Address at X2 interface | 11.1.0 | 11.2.0 | LI11 |
| 2012-06 | SP-56 | SP-120336 | 117 | 1 | F | Handover indication at X2 interface | 11.1.0 | 11.2.0 | LI11 |
| 2012-06 | SP-56 | SP-120336 | 118 | 1 | F | IMS Conference Services | 11.1.0 | 11.2.0 | LI11 |
| 2012-06 | SP-56 | SP-120336 | 119 | 1 | F | IMS Provison of CC | 11.1.0 | 11.2.0 | LI11 |
| 2012-09 | SP-57 | SP-120627 | 120 | 1 | F | Reference list correction to align with the corrected TS 29.212 title | 11.2.0 | 11.3.0 | TEI11 |
| 2014-06 | SP-64 | SP-140319 | 146 | - | F | IMS IMEI Interception | 11.3.0 | 11.4.0 | LI11 |
| 2014-09 | SP-65 | SP-140585 | 156 | - | F | IMEI-based LI stage 2 description | 11.4.0 | 11.5.0 | LI11 |