3GPP TSG SA WG3 (Security) Meeting #78	S3-150023

26-30 January 2015, Sorrento (Italy)

Source:	Deutsche Telekom AG

Title:	pCR to TR 33.806 on merging hardening requirements on Operating Systems, section B.4.3

Document for:	Discussion and Decision

Agenda Item:	7.11

Work Item / Release:	Rel-13

Abstract of the contribution:

The present contribution proposes text for clause B.4.3 “Security Requirements related to Hardening - Operating Systems” of the new Annex B on requirements, based on existing requirements in TR 33.806 and in DT’s catalogue.

It is the intention to eventually add test cases to the new requirements text in Annex B. For now, however, ‘TBA’ has been noted in place of test case details as none of the requiremens from DT’s catalogue has test cases yet and some of the existing test case descriptions in the main body of the TR need improvement.

## 1 Introduction

This contribution proposes new requirements text for Annex B.4.3, based on existing requirements in TR 33.806 and in DT’s catalogue.

Section 2 discusses the  Editor's Note in Annex B.4.3. Section 3 lists the existing requirements and proposes in Word comments how they should be addressed. Section 4 contains the new requirements text for Annex B.

# 2 Discussion

Annex B.4.3 contains the following Editor's Note which needs to be resolved:

Editor’s note: relates to clause 6.7/RX-2, 7.3, 7.4, 7.5 and Req.s 3.37-3,4, 6,7 from DT’s catalogue.  6.7/RX-2, 7.3, 7.4, 7.5 are present in both B.4.2 and B.4.3. This overlap needs to be addressed.

##### 2.1 Resolution of Editor's Note on overlap with clause B.4.2

The overlap between B.4.2 and B.4.3 can be addressed by the following assignment:

| Requirement | Description | Assigned to | Reason |
| --- | --- | --- | --- |
| 6.7/RX-2 | Removal of physical test/debug ports | (B.4.2) | No OS relation: baseline requirement. (Only for physically exposed products) |
| 7.3 | No unnecessary or insecure ports/ services | B.4.2 | Not all services are part of the OS: baseline requirement. |
| 7.4 | Services-Interfaces Binding | B.4.2 | Not only OS-related: could be solved by OS means (e.g. packet filter) or by service-specific configuration. |
| 7.5 | IP-Source address spoofing mitigation | B.4.3 | OS function |

##### 2.2 Resolution of Editor's Note on overlap between TR and DT requirements

TR section 7.4, and DT requirements 3.37-3 and 3.01-2 handle the same topic: binding of services to specific interfaces only. The companion contribution S3-150024 proposes to insert this topic into clause B.4.2.

TR section 7.5, and DT requirement 3.37-10 handle the same topic: spoofing mitigation (rp_filter). The topic is merged in the pCR below.

# 3 Relevant existing requirements for clause B.4.3

The following requirements are related to clause B.4.3, and addressed by the pCR below:

DT Req 3.37-4: only secure OAM protocols

DT Req 3.37-6: minimised kernel network functions

DT Req 3.37-7: no autostart for removable media

TR clause 7.5 and DT Req 3.37-10: spoofing mitigation

The full text of the affected clauses is copied here even if only part of the clause is relevant.

+++START OF CHANGES to main body of TR 33.806+++

## 7.5	IP-Source address spoofing mitigation

-	Requirement Name: IP-source address spoofing mitigation

-	Requirement Reference: TBD

-	Requirement Description:

Rx-1: protection mechanism against IP-source address spoofing attacks shall be enabled. For instance if LINUX is the running O.S. then enable the spoofing prevention built-in function by setting rp_filter to 1.

Editor's Note: it shall be clarified whether all the interfaces should comply with this requirement or whether only particular interface should be impacted.

-	Threat References: the unique identifiers assigned to the threats the requirement intends to meet.

-	Test Case:

Test case for Rx-1:

If LINUX is not the running O.S. then an alternative test case shall be written by the tester.

Test overview:

This test verifies if the spoofing prevention function is enabled, and performs its enabling if disabled

Precondition(s):

- User has sufficient administrative rights to activate spoofing prevention function

Postcondition(s):

- The spoofing prevention mechanism is enabled

Procedure and expected result

| Step | Direction | Action | Expected result |
| --- | --- | --- | --- |
| 1 | User → MME | open command line terminal (clt) | clt open |
| 2 | User → clt | cat /proc/sys/net/ipv4/conf/call/rp_filter | 1 |

+++END OF CHANGES to main body of TR 33.806+++

## +++START OF CHANGES to Annex C (DT’s catalogue) +++

Req 3.37-4	Network based access used for operating system administration must have integrity, be encrypted and securely authenticated.

Access is only permitted by using secure protocols (e.g., SSHv2, HTTPS, SNMPv3). The administrator must ensure that any network connection between his workstation or a management system and the operating system to be admin­ istrated is securely authenticated, encrypted and protected against tampering.

Motivation: If the administrator transmits changes to the configuration settings via unencrypted or unsecure connec­ tions, there is a risk that unauthorized parties exploit vulnerabilities. Information could be gained (configuration set­ tings, access IDs, etc.) to exploit additional security vulnerabilities.

For this requirement the following threats are relevant:

- Unauthorized access to the system

- Denial of executed activities

Req 3.37-6	Kernel based network functions not needed for the operation as a server must be deactivated. Routing functions are not needed on a server; consequently the routing function must be disabled. Additionally the answering routine for broadcast ICMP packages must be disabled. Usually this and other network features are already configured correctly out-of-the-box.

Motivation: The routing functions enable misuse scenarios, meaning that an attack can route malicious packets through the server to connected networks.

For this requirement the following threats are relevant:

- Disruption of availability

- Attacks can go unnoticed

Req 3.37-7            The automatic launch of applications on removable media must be deactivated.

Removable media such as CD-, DVD-, USB-Sticks or USB-Storage drives shall not automatically start any applications they contain.

Motivation: Automatic application launch could inadvertently launch malware.

For this requirement the following threats are relevant:

- Attacks can go unnoticed

## +++NEXT CHANGE+++

Req 3.37-10          Systems must not process IP packets which source address is not reachable via the incoming interface.

It is necessary to ensure there are no unneeded default routes which is typically the case for internal systems.

Motivation: In such a case the IP packet comes from an untrusted source (spoofed address) or a routing error exists in the network. In both cases the packet has to be dropped.

Implementation example: Use of "Reverse Path Filter" (RPF) which provides this feature.

For this requirement the following threats are relevant:

- Disruption of availability

## +++END OF CHANGES to Annex C (DT’s catalogue) +++

## 4  New text for Annex B.4.3 “Hardening - Operating Systems”

## +++START OF CHANGES to new Annex B+++

### B.4.3 	Operating Systems

#### B.4.3.1 IP-Source address spoofing mitigation

Requirement Name: IP-Source address spoofing mitigation

Requirement Reference: TBA

Requirement Description:

Systems shall not process IP packets if their source address is not reachable via the incoming interface. Implementation example: Use of "Reverse Path Filter" (RPF) provides this function.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.3.2 Minimised kernel network functions

Requirement Name: Kernel based network functions not needed for the operation of the network element shall be deactivated.

Requirement Reference: TBA

Requirement Description:

Kernel based network functions not needed for the operation of the network element shall be deactivated. Examples: IP routing functions are not needed on an MME; consequently the routing function shall be disabled. Additionally the answering routine for broadcast ICMP packets shall be disabled.

Note: Full documentation of required kernel network functions of the TOE and their purpose needs to be provided by the vendor as prerequisite for the test case.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

#### B.4.3.2 Automatic launch of removable media

Requirement Name: Automatic launch of removable media

Requirement Reference: TBA

Requirement Description:

The operating system shall not automatically launch applications on removable media such as CD-, DVD-, USB-Sticks or USB-Storage drives. If the operating system supports an automatic launch, it shall be deactivated.

Threat References: TBA

Security Objective References: TBA

Test Case: TBA

## +++END OF CHANGES to new Annex B +++