3GPP TSG SA WG3 (Security) Meeting #79	S3-151420

Nanjing, China, 20 – 24 April 2015                         	                                     revision of S3-1514327

Source:	China Mobile

Title:	SCAS: Revision of critical assets

Document for:	Discussion/approval

Agenda Item:	7.11

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution revised section 5.2 critical assets.

Introduction

This contribution adds a critical asset to section 5.2.

Discussion

The following asset has been mentioned in Annex A:

- MME files (See section A.4.2)

We propose to add this mentioned asset to section 5.2. In addition, the format error for MME software will also be revised.

Proposals

We propose to add above asset to section 5.2.

pCR

***	BEGIN CHANGES	***

5.2	Critical assets

Editor's note: As specified by TR 33.805 [8], clause 5.2.2.4.2.2, this subclause lists all critical assets. Each asset shall be given a unique identifier for later reference from the threats.

The critical assets of MME to be protected are:

-	User account data and credentials (e.g. passwords);

-	Log data;

Editor note: it is FFS if all of the log data is critical asset.

-	Configuration data, e.g. MME's IP address, ports, VPN ID, Management Objects (e.g. user group, command group)  etc.

Editor note: it is FFS whether only security relevant part of the configuration data is critical asset

-	Operating System (OS), I.e. the files that make up the OS and its processes (code and data);

-	MME Application;

Editor note: it is FFS what other applications are critical asset.

-	Mobility Management data: e.g. subscriber's identities (e.g. IMSI), subscriber keys (I.e. KNASenc, KNASint, NH), authentication parameters, address of serving eNB, APN name, data related to mobility management like UE status, UE's IP address, etc., session management like PDN type, QoS and so on, or node selection and routing selection, e.g. IP address of UE related S/P-GW, selected routing connection based on UE's identity, etc.

-	Sufficient processing capacity: that processing powers are not consumed close to limits;

-	Hardware, e.g. mainframe, board, power supply unit etc.

-	The interfaces of MME to be protected and which are within SECAM scope: for example

-	Console interface, for local access: local interface on MME

-	OAM interface, for remote access: interface between MME and OAM system

NOTE: 	The detailed interfaces of the MME class is described in clause 4, Network Product Class Description of the present document.

-    MME Software: binary code or executable code

Note: MME files may be any file owned by a  user (root user as well as non root uses), including User account data and credentials, Log data, configuration data, OS files, MME application, Mobility Management data or MME Software..

***	END OF CHANGES	***