3GPP TSG SA WG3 (Security) Meeting #79	S3-151505

20-24 April 2015, Nanjing (China)		revision of S3-151384, 256, 356

Source:	Nokia Networks, Ericsson, Huawei, HiSilicon

Title:	Proposed EASE_CIoT Key Issue ‘‘Unauthorized modification of signalling data’’

Document for:	Discussion and Decision

Agenda Item:	8.7.1

Work Item / Release:	Rel-13

Abstract of the contribution:

The present contribution proposes text for a new Key Issue #x: Unauthorized modification of signalling data.

## 1 Introduction

The pseudo CR proposed in this section is meant to be included in the draft TR whose skeleton has been proposed in S3-151233.

# 2 Pseudo CR

## +++START OF CHANGES +++

## 5.x	Key Issue #x: Unauthorized modification of signalling data

### 5.x.1	Key issue details

In current GPRS, an attacker may enforce the use of no protection of signalling data or protection by a weak encryption algorithm.

Furthermore, even if a strong encryption algorithm is used a man-in-the-middle could modify individual bits in a message while leaving encryption intact. The fact that a stream cipher is used for encryption and the error detecting code is linear makes the task for the attacker easier. If the attacker knows the plaintext then he can modify it to turn it into a plaintext of his choice. The technical prerequisite for the attacker is that he can toggle individual bits in an LLC frame while being able to forward the otherwise unchanged frame transparently between UE and SGSN and that the attacker knows the frame structure, including the division into headers, plaintext and error detecting code.

In general, roaming is required for all services. However, there will be some UEs for which their HPLMN operator does not expect the UE to roam between countries allowing encryption and countries not allowing encryption (as stated by SA1 in the LS in S3-151445). Support of GEA0 will be needed only for UEs possibly roaming into countries not allowing encryption or where the home operator is located in a country not allowing encryption.

### 5.x.2	Security threats

The most obvious threat is that the attacker can modify the Ciphering Mode procedure, which results in a bidding down attack and consequently in the loss of data confidentiality or data integrity. This is described in another key issue.

Furthermore, some mobility management messages can be sent unprotected in current GPRS. This could result in a Denial-of-Service attack.

Finally, the attacker could modify mobility management messages as described in the key issue details. This could also result in a Denial-of-Service attack. E.g., a successful sending of a Routing Area Update message or a De-registration message could result in a temporary unreachability of the UE until the UE contacts the network the next time.

Editor’s Note: A risk analysis of the effect of modifications of mobility management messages, other than the Ciphering Mode procedure is ffs. The extent of damage that an attacker can do may depend on additional assumptions about UE characteristics e.g. about the initiator of communications (always UE-initiated or not) and the UE mobility pattern (stationary vs. global roaming).

### 5.x.3	Security requirements

Modification of mobility management and session management messages shall be prevented.

Editor’s Note: It is ffs under what assumptions strong encryption of signalling messages alone could provide sufficient security for certain classes of CIoT UEs.

## +++END OF CHANGES +++