3GPP TSG SA WG3 (Security) Meeting #80	S3-151757

24-28 August 2015, Tallinn (Estonia)

Source:	NEC

Title:	Adding a test case related to the requirement 5.3.2.3 “No unused software”

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to add a test case related to the requirement 5.3.2.3 of TS 33.117.

# Introduction

This contribution proposes to add the test case related to the requirement 5.3.2.3 of TS 33.117

Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here since in TS 33.117 the test case immediately follows the requirement.

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

#### 5.3.2.3	No unused software

Requirement Name: Unused software shall not be installed or shall be uninstalled.

Requirement Reference: TBA

Requirement Description:

Unused software components or parts of software which are not needed for operation or functionality of the system shall not be installed or shall be deleted after installation. This includes also parts of software, which will be installed as examples but typically not be used (e.g. default web pages, example databases, test data).

Threat References: TBA

Security Objective References: TBA

Test Case:

Test Name: TC_NO_UNUSED_SOFTWARE

## Purpose:

To ensure that there is no unused software or associated components that might be installed in the network product which are not required for its operation or functionality.

## Procedure and execution steps:

## Pre-Conditions:

A list of all available software and associated components containing at least the following information shall be included in the documentation accompanying the Network Product:

•	name of the software;

•	version of the software installed;

•	list of dependencies and versions;

•	any add-ons and functions;

•	any special hardware/debugging ports;

•	software support type;

•	licensing information;

•	requirement during functioning of system;

•	brief description of their purpose;

## Execution Steps

The accredited evaluator’s test lab is required to execute the following steps:

- Verification of the compliance to the prerequisites

  - Verification that the list of software and components is available in the documentation of the Network Product.

  - Validation that all entries in the list of software and components are meaningful and reasonably necessary for the operation of the Network Product class.

- Identification of the software or components which are installed in the system using any suitable command line tools or any other suitable means of determination.

- Validation that there are no entries in the list of software installed  in the system apart from the ones that have been mentioned and deemed necessary for the operation of the network product in the attached documentation.

## Expected Results:

## The report will contain the names and version of the tool(s) used for finding out what software is installed in the system. The detailed report will contain the name and version information of all the software installed in the system generated by the tool.

## The list of all available software which has been deemed necessary for the operation of the network product by the vendor shall also be included as the test result. Any software not in the list of allowed software will be highlighted and brought out as a part of the report.

## There should be no unnecessary software installed in the network product except for the ones which are deemed necessary for its operation.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- The used tool(s) name and version  information,

- Settings and configurations used,

- the output pertaining to the test case performed and ,

- the test results i.e. list of allowed and disallowed software

***	END OF FIRST CHANGE	***