3GPP TSG SA WG3 (Security) Meeting #80	S3-151758

24-28 August 2015, Tallinn (Estonia)

Source:	NEC

Title:	Adding a test case related to the requirement 5.3.2.4 “No unused functions”

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to add a test case related to the requirement 5.3.2.4 of TS 33.117.

# Introduction

This contribution proposes to add the test case related to the requirement 5.3.2.4 of TS 33.117

Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here since in TS 33.117 the test case immediately follows the requirement.

# Proposed pCR

***	BEGIN OF FIRST CHANGE ***

#### 5.3.2.4	No unused functions

Requirement Name: Unused functions of the TOE's software and hardware shall be deactivated.

Requirement Reference: TBA

Requirement Description:

During installation of software and hardware often functions will be activated that are not required for operation or function of the system. If unused functions of software cannot be deleted or deinstalled individually as required in clause B.4.2.3 of the present document, such functions shall be deactivated in the configuration of the system permanently.

Also hardware functions which are not required for operation or function of the system (e.g. unused interfaces) shall be permanently deactivated. Permanently means that they shall not be reactivated again after system reboot.

Example: a debugging function in software which can be used for troubleshooting shall not be activated during normal operation of the TOE.

Threat References: TBA

Security Objective References: TBA

Test Case:

Test Name: TC_NO_UNUSED_FUNCTIONS

## Purpose:

To ensure that there is no unused hardware or software functions that might be installed in the network product which are not required for its operation or functionality.

## Procedure and execution steps:

## Pre-Conditions:

A list of all available software and associated components containing at least the following information shall be included in the documentation accompanying the Network Product:

•	name of the software;

•	version of the software installed;

•	list of dependencies and versions;

•	any add-ons and functions;

•	any special hardware/debugging ports;

•	software support type;

•	licensing information;

•	requirement during functioning of system;

•	brief description of their purpose;

## Execution Steps:

The accredited evaluator’s test lab is required to execute the following steps:

- Identification of the hardware and software functions which are installed in the system or might have been disabled using any suitable command line tools or any other suitable means of determination.

- Validate that there are no entries in the list of hardware and software functions installed in the system apart from the ones that have been mentioned and deemed necessary for the operation of the network product in the attached documentation.

## Expected Results:

## The report will contain the names and version of the tool(s) used for finding out what software and associated function is installed in the system. The detailed report will contain the name and version information of all the software and components installed in the system generated by the test tool.

## The list of all available software which has been deemed necessary for the operation of the MME by the vendor shall also be included as the test result. Any software not in the list of allowed software will be highlighted and brought out as a part of the report.

## There should be no unused function installed in the network product except for the ones which are deemed necessary for its operation.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- The used tool(s) name and version  information,

- Settings and configurations used,

- The list of software and associated functions

- the test results i.e. allowed list of functions

***	END OF FIRST CHANGE	***