3GPP TSG SA WG3 (Security) Meeting #80	S3-151774

24-28 August 2015, Tallinn (Estonia)

Source:	Alcatel-Lucent

Title:	Addressing duplication of events in requirement 5.2.6.3.1 “Network Device Security event logging”

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to remove duplication of security events in requirement 5.2.6.3.1 of TS 33.117.

# Introduction

At the SA79bis Ad-Hoc meeting in June 2015 two requirements from Annex B were identified as covering similar areas namely Annex B. 3.3.6.1 – Security Event Logging and Annex B.3.7.3.1 – Network Device Security Event Logging. In particular both sub-clauses contain some overlap of  events,; (a) failed login attempts/incorrect login attempts) and (b) Critical rise in system values such as disk space, CPU load over a longer period/resource usage.

Both Annex B sub-clauses (B.3.3.6.1 and B.3.7.3.1) have been mapped to TS33.117 5.2.3.6.1 and 5.2.6.3.1 respectively.

Sub-clause 5.2.3.6.1 is also the subject of pCR from Telecom Italia (S3-151708) which proposes changes to the security event table structure.

This contribution proposes to delete the duplicate security events from sub-clause 5.2.6.3.1 and also to add an additional column to the security event table to ensure it is aligned with the table proposed in sub-clause 5.2.3.6.1 in contribution S3-151708. Some editorial changes have been included in the new table structure

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

#### 5.2.6.3 	Logging

##### 5.2.6.3.1	Network device security event logging

Requirement Name: Network device security event logging

Requirement Reference: TBA

Requirement Description: Security-relevant events shall be logged with a precise time stamp and a unique system reference. Network devices shall log the occurrence of security-relevant events. So that these events can be evaluated and classified, they shall be logged together with a unique system reference (e.g. host name, IP or MAC address) and the exact time the event occurred.

The following security-relevant events shall be logged by a network device:

| Event |  |  | Event data to be logged |  |
| --- | --- | --- | --- | --- |
| Event Types |  | Description |  | Event data to be logged |  |
| Configuration change |  | Changes to configuration of the network device |  | • Change made, • User |  |
| Reboot/shutdown/crash |  | This event records any action on the network device that forces a reboot or shutdown OR where the network device has crashed. |  | • Action performed (reboot, shutdown, etc.), • User (for intentional actions) |  |
| Interface status change |  | Change to the status of interfaces on the network device (e.g. shutdown) |  | • Interface name and type, • Status (shutdown, missing link, etc.) |  |

Note: Sub-clause 5.2.3.6.1 contains additional event types that may also apply to network device security event logging.

Threat References: TBA

Security Objective references: tba.

Test case: TBA

***	END OF CHANGES	***