3GPP TSG SA WG3 (Security) Meeting #80                                                         S3-151859

24-28 August 2015, Tallinn (Estonia)

Source:	China Mobile,CATR

Title:	Adding test case to the requirement 5.2.3.6.2 on Log transfer to centralized storage

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution adds the test case related to requirement 5.2.3.6.2 related to log transfer to centralized storage.

Introduction

This contribution adds the test case related to requirement 5.2.3.6.2 related to log transfer to centralized storage.

Proposed pCR

***	BEGIN OF FIRST CHANGE	***

5.2.3.6.2	Log transfer to centralized storage

Requirement Name: Log transfer to centralized storage

Requirement Reference: TBA

Requirement Description:

a)	The Network Product shall support forward of security event logging data to an external system. Standard protocols like Syslog shall be preferred.

b)	Log functions should support secure uploading of log files to a central location or to a system external for the Network Product that is logging.

Threat References: TBA

Security Objective references: tba.

Test Name: TC_LOG TRANS_TO_CENTR STORAGE

## Purpose:

To ensure log shall be transferred to centralized storage.

## Procedure and execution steps:

## Pre-Conditions:

- The manufacture shall list the standard protocols which transfer security event logging data.

- The session between network product and central location or system external for network product log functions has been set up.

- The tester has privilege to operate network product and related logs can be outputted.

## Execution Steps

The accredited evaluator’s test lab is required to execute the following steps:

- The tester operates network product and related logs are output.

- The tester uses tools (e.g. wireshark) to capture some security event logging data in the switch which forwards security event logging data between network product and the external system.

- The tester checks whether the used transport protocol is standard protocol.

- The tester checks whether the captured data contain plain text log.

- The tester checks whether the central location or system external for network product log functions has stored the related logs.

## Expected Results:

- The listed protocols are standard protocols.

- The tester analyzes the captured data and finds that used protocol is standard protocol which is declared by the manufacturer

- The tester analyzes the captured data and finds that there is no plain text log.

- The tester finds that the central location or system external for network product log functions has stored the related logs.

## Expected format of evidence:

A testing report provided by the testing agency which will consist of the following information:

- Settings, protocols and configurations used,

- Screenshot

- Test result (Passed or not)

***	END OF FIRST CHANGE	***